This FILE mx12p_ui.doc

is part of the MX12P control panel program.  It is copyright (C) 2000 by

Gregory L. Kreider.  All rights reserved.

This text contains the comments to the mx12p_ui code.

==============================================================================

==============================================================================

MODULE:   mx12p_ui

Description:

This program contains a UI control panel program used to control an

Adimec MX12P camera (via a PSU120).  The UI is written in Tk.  There

is also C glue code that sits between the UI program and the MX12P and

PSU120 libraries.  This file will be combined with a wish interpreter

to be able to run the Tk program.  SWIG is used to create this modified

interpreter, generates several links between C and Tk for several

functions and variables.

The data organization of the program is so:

PSU120 lib  <-->  MX12P lib     <-->  mx12p_ui.c  <-->  mx12p_ui.tk

TCmd             TUserContext                          local vars

none                TCamParameters    mx12p_globals

mx12p_fname

Although data is duplicated, at least on the UI side this is necessary.

Tk can interact with Tk variables, but not through SWIG-created links,

so we need the mx12p_globals structure to communicate between the two.

The UI values are also kept strictly separate from the library

parameters (TCamParameters), since we only expose a subset to Tk.

The functions and variables in the header file mx12p_ui.h that lie

between the /* ++swig+ */ and /* ++swig- */ markers are sent to a

specially-generated SWIG template (by Make_mx12p_ui_swig).  Not every

function and variable is visible to Tk.

Functions meant to be called by Tk take no arguments and return an

integer.  A return value less than 0 indicates an error has occured, 0

no error, and positive values (if any) are variable values.

The control panel is organized as a number of panels: one per functional

grouping of camera parameters (following the Adimec control panel),

a title bar, a set of buttons controlling when to send changes to the

parameters to the PSU120, a set of buttons to save and restore camera

configurations, and a program control line.  For each panel we define

three functions: a panel_XXX, which sets up the Tk elements,

disable_XXX, which makes all controls in the panel inactive and erases

any key bindings, and enable_XXX, which makes all controls active and

sets up key bindings.

==============================================================================

==============================================================================

FUNCTION: setup_mx12p

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions)

Side Effects:

mx12p_vars and the user context are changed

Description:

This function sets up default values for the UI variable, reads (if

possible) $HOME/.mx12prc for the last used comm port, and sets up the

mx12p library.

==============================================================================

FUNCTION: closedown_mx12p

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions)

Side Effects:

tries to write the current comm port to $HOME/.mx12prc

Description:

closedown_mx12p writes the current comm port to $HOME/.mx12prc after

closing it.

==============================================================================

FUNCTION: link_mx12p_vars

File:     mx12p_ui.c

Args:     (void)

Output:   (mx12p_globals*)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions; ErrMX_RC if can't open file)

Side Effects:

none

Description:

Returns a pointer to the UI data structure, for linking via SWIG.

[SWIG encodes this pointer and stores it in a Tcl variable; you

then use the reader and writer functions that SWIG derives to

communicate to mx12p_vars via this pointer.]

==============================================================================

FUNCTION: closeMX12P

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions; ErrMX_NoConnect if no open connection to PSU)

Side Effects:

serial port is closed and camera parameters are so marked

Description:

This function tries to close an open comm port and, if it succeeds,

clears the global variables used to track this.

==============================================================================

FUNCTION: openMX12P

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions)

Side Effects:

serial port is opened and camera paramters are so marked

Description:

openMX12P makes a connection to the PSU120.  It first shuts an already

open port (aborting if this fails), then calls CamOpen and updates

the global variables that track the port status.  Finally, it copies

the camera parameters to the UI variable so that the UI reflects the

current state of the PSU120.

==============================================================================

FUNCTION: transferToMX12P

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions; ErrMX_NoConnect if no open connection to PSU)

Side Effects:

sends current UI parameters to PSU120, updates clock and sensor

Description:

transferToMX12P copies the UI camera values to the PSU.  Because

CamTransfer can change the sensor type and pixel clock values, we

restore these after the write.

==============================================================================

FUNCTION: attentionMX12P

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions; ErrMX_NoConnect if no open connection to PSU)

Side Effects:

confirms communication channel with PSU120

Description:

CamOpen/PSUOPEN simply opens the serial port, supposedly one connected

to a PSU120.  This function, which should be called after openMX12P,

confirms that there is indeed a PSU listening.

==============================================================================

FUNCTION: readFromPSU

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions; ErrMX_NoConnect if no open connection to PSU)

Side Effects:

reads default values from PSU and copies to UI side

Description:

readFromPSU gets the default camera values from the PSU120.  If

there is a problem with the read, the port is closed and we abort from

this function.  Otherwise, we copy the new values back to the UI.

==============================================================================

FUNCTION: writeToPSU

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (see ErrX_ variable for explanation for called

functions; ErrMX_NoConnect if no open connection to PSU)

Side Effects:

send current UI values to PSU as default

Description:

writeToPSU copies the current UI parameters to the library data

structure and sends them to the PSU to be stored as defaults.  If

the write fails, the comm port is closed.

==============================================================================

FUNCTION: readFromFile

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (ErrMX_FileName if name too long (>= 256 characters;

ErrMX_SaveFile if can't open file for reading)

Side Effects:

reads UI parameters from file and sends to PSU

Description:

readFromFile reads a text-based configuration file with a camera set-up.

Each camera parameter in the mx12p_vars data structure, excluding

transient and derived values such as the current comm port and MX_Qxxx

variables, can be read from the file.  The file should consist of any

number of lines, each less than 256 characters.  A '#' in the first

column indicates a comment line and is ignored.  Otherwise, the first

characters should be the name of a structure member in mx12p_globals,

followed by whitespace, followed by an integer value for that

parameter.  This function parses each line so, reading the value into

mx12p_vars.  No check is made as to the value read.  Lines that do

not contain a member name or have no value are ignored.  Finally, when

the entire file has been parsed, the camera values are sent to the PSU120.

==============================================================================

FUNCTION: writeToFile

File:     mx12p_ui.c

Args:     (void)

Output:   (int)

0 if no problems during execution

<0 if error (ErrMX_FileName if name too long (>= 256 characters;

ErrMX_SaveFile if can't open file for writing)

Side Effects:

UI parameters are written to the file (existing file is overwritten)

Description:

writeToFile saves the current camera set-up to a text file.  It first

sends a comment line identifying the file contents and the date that it

was created.  Then, each of the camera parameters (ignoring transient

and derived values) is written to the file.  Everything is lined up

neatly in columns.

==============================================================================

FUNCTION: parseRCFile

File:     mx12p_ui.c

Args:     (FILE*)

#1 fin: file (already opened) from which to read

Output:   (int)

0 if no problems during execution

<0 if error (ErrMX_RCParse if data line not recognized)

Side Effects:

UI parameters are taken from file

Description:

parseRCFile parses the given file.  It ignores lines beginning with a

'#', and looks for one starting with "LastCommPort".  If found, it

skips white space after the name and then reads an integer into the

comm port variable.  Any format error or other line raises an error.

==============================================================================

FUNCTION: writeRCFile

File:     mx12p_ui.c

Args:     (FILE*)

#1 fout: file (already opened) to which to write

Output:   (int)

0; no other values are possible

Side Effects:

the comm port is stored in the file

Description:

This function writes the currently open port (must check before calling

this that a port is open) to the file following the format in parseRCFile.

==============================================================================

FUNCTION: copyVarToPar

File:     mx12p_ui.c

Args:     (void)

Output:   (void)

Side Effects:

UI camera parameter values are copied to library data structure

Description:

UI camera parameter values are copied to library data structure.

Transient values (comm port) and derived values (MX_Qxxx) are not

stored on the UI side.  The Invalidate flag is cleared; we only update

what has changed.

==============================================================================

FUNCTION: copyParToVar

File:     mx12p_ui.c

Args:     (void)

Output:   (void)

Side Effects:

library camera parameters are copied to UI/C structure

Description:

Library camera parameters are copied to the UI/C structure (and, within

the Tk code, to the actual UI variables).  Only actual parameters, as

well as the pixel clock and sensor type, are copied back.

==============================================================================

==============================================================================

FUNCTION: controlpanel

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

creates main window

Description:

The main function, the control panel sets up the panels and windows

and de-activates all controls (they will be activated once a port is

open).  This function returns immediately; pressing the 'Quit' button

will call a function (finshCP) that exits wish.

==============================================================================

FUNCTION: disable_controlpanel

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

disable_controlpanel calls disable_XXX for each panel (the comm port

panel is never deactivated, nor the bottom row of buttons).

==============================================================================

FUNCTION: enable_controlpanel

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

enable_controlpanel calls enable_XXX for each panel and re-establishes

global key bindings.

==============================================================================

FUNCTION: panel_Status

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Status panel contains two text lines, displayed at the top center

of the control panel.  The first gives the title of the program (and

also the camera that it is driving), the second the state of the

communication channel with the PSU120.  Help texts are available for

each of the lines; no other interaction is possible.

==============================================================================

FUNCTION: panel_Gain

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Gain panel contains a slider to set the video gain (1X - 32X in

steps of 2X).  The text for the slider is assigned separately (for

the exponential scaling).  A help text is available; the key to

activate this panel is 'G'.

==============================================================================

FUNCTION: disable_Gain

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Gain

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings.

==============================================================================

FUNCTION: panel_Tint

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Integration Time (Tint) panel contains a slider to set the

integration time (digital value 0 to 19).  The time, which depends

on several other camera paramters (sensor type, interlacing, etc),

is displayed as a text next to the slider; it is generated in a

separate function.  A help text is available; the key to activate

this panel is 'I'.

==============================================================================

FUNCTION: disable_Tint

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Tint

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings.

==============================================================================

FUNCTION: panel_Black

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Black Level panel contains the controls for adjusting the zero

level output signal.  There is a slider which sets the level (an

8-bit value); the text depends on whether the fine black control is

enabled and is generated in a separate function.  There are two

buttons, one which enables the level shift, and one that switches

between coarse (0-100% Vmax) and fine (0-20% Vmax).  All controls

are deactivated until the level shift has been enabled.  A help

text is available; the key to activate this panel is 'B'.

==============================================================================

FUNCTION: disable_Black

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Black

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings, depending

on whether the black level shift is enabled.

==============================================================================

FUNCTION: panel_Readout

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Readout panel contains controls for parameters dealing with the

style of read-out.  There are buttons for turning on/off Gamma

Correction and mirroring.  A help text is available; the key to

activate this panel is 'R'.

==============================================================================

FUNCTION: disable_Readout

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Readout

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings, depending

on whether the black level shift is enabled.

==============================================================================

FUNCTION: panel_Video

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Video Standard panel contains controls for the format of the

output signal.  There are buttons to choose between 50 and 60 Hz

(one of the two must be set), and to interlace.  There is a help

text available; the key to activate this panel is 'V'.

==============================================================================

FUNCTION: disable_Video

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Video

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings, depending

on whether the black level shift is enabled.

==============================================================================

FUNCTION: panel_Mode

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Camera Mode panel contains controls setting the trigger conditions

for a frame read-out.  The buttons allow a selection between the four

modes (described in the MX12P manual).  There is a help text available;

the key to activate this panel is 'M'.

==============================================================================

FUNCTION: disable_Mode

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Mode

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings, depending

on whether the black level shift is enabled.

==============================================================================

FUNCTION: panel_CommPort

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Comm Port panel selects the serial port to use to communicate with

the PSU120.  It is always active.  Choosing one of the port numbers

(the PSU120 library supports 8) calls openCamera to open the serial

port and check that a PSU is attached (any existing connection is

broken).  A help text is available; the key to activate this panel

is 'P'.

==============================================================================

FUNCTION: panel_Advanced

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

The Advanced Options panel contains controls for miscellaneous camera

functions.  There are buttons to turn on/off composite sync, to

choose TTL or differential outputs, and to turn on/off the two option

flags.  A help text is available; the key to activate this panel is 'O'.

==============================================================================

FUNCTION: disable_Advanced

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Advanced

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings, depending

on whether the black level shift is enabled.

==============================================================================

FUNCTION: panel_Update

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

This panel contains buttons that control when changes to the camera

parameters are sent to the PSU120.  There are buttons to choose

between an automatic update, or on command.  There is also a button

to force an update.  A help text is available; the key to activate

this panel is 'U'.

==============================================================================

FUNCTION: disable_Update

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Update

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings, depending

on whether the black level shift is enabled.

==============================================================================

FUNCTION: panel_Config

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

This panel allows the user to read and write the current camera

parameters.  There are two sets of buttons, one for sending/getting

the data from the PSU120's memory (in which case the configuration

becomes the default start-up value), or from a file.  A help text

is available; the key to activate this panel is 'C'.

==============================================================================

FUNCTION: disable_Config

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

disable controls in panel

Description:

This function makes the Gain panel insensitive to user interactions.

==============================================================================

FUNCTION: enable_Config

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame panel is drawn in

Output:   n/a

Side Effects:

enable controls in panel

Description:

This function re-activates the controls and key/mouse bindings, depending

on whether the black level shift is enabled.

==============================================================================

FUNCTION: panel_General

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: parent frame to draw in

Output:   n/a

Side Effects:

none

Description:

This panel contains three buttons for the operation of the control panel

program.  The 'About' command will display a copyright notice and

version information.  The 'Quit' command (also bound to key 'Q') closes

any open connections and exits the program.  The 'Help' command (also

bound to key 'H') displays a general help text (which refers to the

texts for each panel).  These buttons are always active.

==============================================================================

FUNCTION: help_Window

File:     mx12p_ui.tk

Args:     {lab txt}

#1 lab: title/window title

#1 txt: text to display in window

Output:   n/a

Side Effects:

none

Description:

The help_Window creates a separate text window with one button to

dismiss the window.  This window is given the focus and grab.

==============================================================================

FUNCTION: disable_global_shortcuts

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

global key bindings are erased

Description:

This function, called when there is no open connection to the PSU120,

removes all key bindings.

==============================================================================

FUNCTION: enable_global_shortcuts

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

sets up global key bindings

Description:

This function re-creates the key bindings to each panel, as well as

the help and quit keys.

==============================================================================

FUNCTION: help_Status

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Status lines.

==============================================================================

FUNCTION: help_Gain

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Video Gain panel.

==============================================================================

FUNCTION: help_Tint

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Integration Time panel.

==============================================================================

FUNCTION: help_Black

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Black Level panel.

==============================================================================

FUNCTION: help_Readout

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Readout panel.

==============================================================================

FUNCTION: help_Standard

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Video Standard panel._

==============================================================================

FUNCTION: help_Mode

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Camera Mode panel.

==============================================================================

FUNCTION: help_CommPort

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for opening a connection to the PSU120.

==============================================================================

FUNCTION: help_Advanced

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Advanced Options panel.

==============================================================================

FUNCTION: help_Update

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for the Update panel.

==============================================================================

FUNCTION: help_Config

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Help text for Configuration panel.

==============================================================================

FUNCTION: help_ControlPanel

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

General help text for the program.  The text depends on whether a comm

port is open.

==============================================================================

FUNCTION: about_ControlPanel

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Version and copyright information for the program.

==============================================================================

FUNCTION: do_error_processing

File:     mx12p_ui.tk

Args:     {errval errfn closecam}

#1 errval: error code

#2 errfn: function being executed when command failed

#3 closecam: 1 if comm port has been closed

Output:   n/a

Side Effects:

none

Description:

do_error_processing creates a warning when a command fails (returns an

error value --- less than 0).  It adds a single line that explains the

error code, and informs the user if the connection has been closed

because of the fault.

==============================================================================

FUNCTION: get_error_text

File:     mx12p_ui.tk

Args:     {errval}

#1 errval: error code

Output:

text describing error code

Side Effects:

none

Description:

This function returns a single line describing the error code.

==============================================================================

FUNCTION: adjustFrameWidths

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

fixes the size of the panels

Description:

adjustFrameWidths hard-wires the sizes of sizes of the various panels

so they form a neat grid.  At least, they do on my system ...  Ideally

we would want to display the window, then read back the actual width

and height of each panel and use that to determine the sizes of all

others.  I can't figure out, though, how to read this information:

because the frames are created without a fixed size, cget'ing -width

and -height returns 0.  So, for now, go with this solution.

==============================================================================

FUNCTION: spacer

File:     mx12p_ui.tk

Args:     {fr size}

#1 fr: frame that becomes spacer

#1 size: height of frame (big, med, or small)

Output:   n/a

Side Effects:

none

Description:

A spacer is an empty frame, used for vertical whitespace in a window.

Size can be "big", "med", or "small".

==============================================================================

FUNCTION: darkenFrame

File:     mx12p_ui.tk

Args:     {fr dark}

#1 fr: frame to darken

#1 dark: color to make frame

Output:   n/a

Side Effects:

background color of object is changed

Description:

darkenFrame is used to change the color of a frame and all children.

The color is supplied to the function.

==============================================================================

FUNCTION: lightenFrame

File:     mx12p_ui.tk

Args:     {fr}

#1 fr: frame to darken

Output:   n/a

Side Effects:

background color of object is changed

Description:

lightenFrame restores the background color of a frame and all children

to a value stored in a global variable winCP_bg (set when creating the

main window).

==============================================================================

FUNCTION: darkenColor

File:     mx12p_ui.tk

Args:     {c}

#1 c: color to darken

Output:

darkened color string

Side Effects:

none

Description:

darkenColor reduces the rgb values by 5% and returns a new color string.

==============================================================================

FUNCTION: update_controlPanel

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

update_controlPanel calls the update function for each text string

(camera type, connection status, gain, tint, and black level).

==============================================================================

FUNCTION: update_Status

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

sets connection text

Description:

This function assigns a text in the second status line corresponding to

the state of the comm connection to the PSU120.

==============================================================================

FUNCTION: update_Gain_text

File:     mx12p_ui.tk

Args:     {val}

#1 val: ignored

Output:   n/a

Side Effects:

sets gain text

Description:

Sets the text next to the video gain slider to the text.  The argument

is passed by the slider command, but is ignored (we use the global

value).

==============================================================================

FUNCTION: gain_text

File:     mx12p_ui.tk

Args:     n/a

Output:

string with gain value

Side Effects:

none

Description:

The gain text is the string "gain X", where gain is 2 ^ gainstep and where

gainstep is the value used by the PSU120.

==============================================================================

FUNCTION: update_Tint_text

File:     mx12p_ui.tk

Args:     {val}

#1 val: ignored

Output:   n/a

Side Effects:

sets Tint text

Description:

Sets the text next to the integration time slider.  The argument is passed

by the slider, but is actually ignored (we use the global value).  The

integration time calculation depends on the camera mode, and this function

routs to the appropriate function.

==============================================================================

FUNCTION: init_Tint_text

File:     mx12p_ui.tk

Args:     n/a

Output:

string with Tint value

Side Effects:

none

Description:

Similar to update_Tint_text, but returns the Tint string instead of

setting the label text in the UI.  This function is called when creating

the label, so update_Tint_text would fail.

==============================================================================

FUNCTION: Tint_text

File:     mx12p_ui.tk

Args:     t

#1 t: time (in ms)

Output:

formatted string "time ms"

Side Effects:

none

Description:

Tint_text converts the integration time (calculated for the different

camera modes) to a string, appending "ms".

==============================================================================

FUNCTION: findTint_cont

File:     mx12p_ui.tk

Args:     n/a

Output:

Tint (in nms) for continuous read-out

Side Effects:

none

Description:

findTint_cont calculates the integration time for continuous read-out.

The time depends on the camera type, read-out and frame speed, and

interlacing; the actual calculation can be found in the mx12p library.

==============================================================================

FUNCTION: findTint_ctrl

File:     mx12p_ui.tk

Args:     n/a

Output:

Tint (in nms) for continuous read-out

Side Effects:

none

Description:

This function returns the integration time for a camera in "Control"led

mode (actually, there is no integration time, so the function returns -1).

==============================================================================

FUNCTION: findTint_other

File:     mx12p_ui.tk

Args:     n/a

Output:

Tint (in nms) for continuous read-out

Side Effects:

none

Description:

This function calculates the integration time for a camera in the two

"Trigger" modes.  The time depends on the camera type and read-out speed.

==============================================================================

FUNCTION: update_MXType_text

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

sets status text

Description:

Sets the text in the title line with the camera type.

==============================================================================

FUNCTION: MXType_text

File:     mx12p_ui.tk

Args:     n/a

Output:

string with camera name

Side Effects:

none

Description:

MXType_text returns the string "MX12P/XxYx", where X indicates the sensor

type and Y the horizontal clock frequency.

==============================================================================

FUNCTION: update_Black_text

File:     mx12p_ui.tk

Args:     val

#1 val: ignored

Output:   n/a

Side Effects:

sets black level text

Description:

Sets the text next to the black level slider.  The argument is passed

when the slider moves, but is ignored here (we use the global variables).

==============================================================================

FUNCTION: black_text

File:     mx12p_ui.tk

Args:     n/a

Output:

string with black level

Side Effects:

none

Description:

The black level depends not only on the slider value but also the coarse

and fine setting (0--100% or 0--20%).  black_text returns "level%".

==============================================================================

FUNCTION: set_Gain

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Handles changes to the gain value (sends to camera and updates label text).

==============================================================================

FUNCTION: set_Tint

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Handles changes to the Tint value (sends to camera and updates label text).

==============================================================================

FUNCTION: set_Black

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Handles changes to the black level parameters (sends to camera, updates

label text, and enables/disables panel depending on whether black shift

is enabled).

==============================================================================

FUNCTION: set_Gamma

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy).

==============================================================================

FUNCTION: set_Mirror

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy).

==============================================================================

FUNCTION: set_Standard

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Handles changes to the frame rate (sends to camera if allowed and updates

the integration time).

==============================================================================

FUNCTION: set_Interlaced

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy).

==============================================================================

FUNCTION: set_Mode

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy) and changes the

maximum allowed Tint value (19 for continous read-out, 31 otherwise).

==============================================================================

FUNCTION: set_Sync

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy).

==============================================================================

FUNCTION: set_Interrface

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy).

==============================================================================

FUNCTION: set_Option1

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy).

==============================================================================

FUNCTION: set_Option2

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sends value to camera (depending on update policy).

==============================================================================

FUNCTION: openCamera

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Tries to open the serial port and looks for a response from the PSU120

(note that the comm port ID is offset from the serial port by one ---

/dev/ttyS0 is comm port 1).  If there is none, the function displays

the error text and clears the comm port.  If the open succeeds, then

openCamera reads the current camera parameters.  It updates the control

panel with the new information (fail or succeed).

==============================================================================

FUNCTION: closeCamera

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Tries to close a connection to the camera and pops up an error text if

this fails.  Updates the control panel when done.

==============================================================================

FUNCTION: transferToCamera

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

This function transfers the current parameters to the camera and, if it

fails, pops up an error text and closes the connection.  It updates the

control panel when done.  Note that a check whether the update policy

allows a transfer should be made before calling this function.

==============================================================================

FUNCTION: PSU120_ReadFrom

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Gets the default camera parameters from the PSU.  If the call succeeds,

the values are transferred to the UI variables, if not, an error text

appears.  PSU120_ReadFrom then writes the variables back to the PSU to

ensure consistancy between the UI and library, and updates the main

window accordingly.

==============================================================================

FUNCTION: PSU120_WriteTo

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Saves the current camera parameters as defaults.  If the write fails,

the function displays the error text.

==============================================================================

FUNCTION: File_ReadFrom

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

File_ReadFrom gets a file name and tries to read camera valus from it.

If the read succeeds (and the read automatically transfers the new

values to the camera), the values are copied back Tk-side.  Otherwise,

the error message is displayed.  As with PSU120_ReadFrom, the newly

read parameters are immediately written back to the PSU120 to ensure

consistancy.

==============================================================================

FUNCTION: File_WriteTo

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

File_WriteTo gets a file name and writes the current parameters out.

If the write fails, it displays the error text.

==============================================================================

FUNCTION: finishCP

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

main window destroyed

Description:

finishCP is the clean-up function when exiting the program.  It closes

any open connections, destroys the main window, and exits.

==============================================================================

FUNCTION: update_Camera

File:     mx12p_ui.tk

Args:     v

#1 v: camera parameter to change

Output:   n/a

Side Effects:

parameter is changed C-side

Description:

update_Camera takes a single argument, th name of the parameter that has

changed.  It copies the UI value C-side (for black level, all three

parameters are sent) and sends the value to the PSU120 (if allowed by

the update mode).

==============================================================================

FUNCTION: init_MX_vars

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

Sets up the UI variables.  It initializes the SWIG link with the C-side

global mx12p_vars, and copies the default values back to Tk.

==============================================================================

FUNCTION: copyback_MXVars

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

Tk MX_ globals are set to their C-side values

Description:

copyback_MXVars gets the values of all camera parameters from C.

==============================================================================

FUNCTION: init_CCA_vars

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

gets Tk globals that aren't linked C-side

Description:

Tk globals needed for the control panel (data arrays for the integration

time calculation, default camera type, flags, and file name) are

initialized.

==============================================================================

FUNCTION: ---   (top-level Tk commands)

File:     mx12p_ui.tk

Args:     n/a

Output:   n/a

Side Effects:

none

Description:

When starting the script the following actions take place:

- global window variables are created

- wish window is removed from screen

- C-side is initialized

- control panel globals are initialized

- Tk-side camera parameters are copied from C-side

- the main window is created

- if a comm port has been read during C initialization, then the

connection is opened

Note that there is no call into a main-like procedure: the main window

will react to any changes, and this drives the interactions with the

camera.

==============================================================================

==============================================================================

GLOBAL VARIABLE: mx12p_vars

File:            mx12p_ui.c

Type:            mx12p_globals

Description:

Stores the values used on both the Tk and C sides, including all

camera parameters.

==============================================================================

GLOBAL VARIABLE: mx12p_fname

File:            mx12p_ui.c

Type:            char*

Description:

Char array allocated by Tk when the user specifies a file name to

read/write camera parameters.

==============================================================================

GLOBAL VARIABLE: mx12p_cam

File:            mx12p_ui.c

Type:            TCamParameters

Description:

Stores the camera parameters for communication with the PSU120.  Kept

separate from the UI copy mx12p_globals.

==============================================================================

==============================================================================

ERROR VARIABLE: ErrMX_RC

File:           mx12p_ui.c

Value:          -10

Message:        Can't open ~/.mx12prc to save commport

==============================================================================

ERROR VARIABLE: ErrMX_RCParse

File:           mx12p_ui.c

Value:          -11

Message:        Error parsing ~/.mx12prc

==============================================================================

ERROR VARIABLE: ErrMX_NoConnect

File:           mx12p_ui.c

Value:          -12

Message:        No connection to PSU120 is now open.

==============================================================================

ERROR VARIABLE: ErrMX_SaveFile

File:           mx12p_ui.c

Value:          -13

Message:        Error handling save file (creating, writing, closing, etc).

==============================================================================

ERROR VARIABLE: ErrMX_FileName

File:           mx12p_ui.c

Value:          -14

Message:        File name too long (total path length: 256 characters).

==============================================================================

==============================================================================

STRUCTURE:  mx12p_globals

File:       mx12p_ui.c

Scope:      file

Definition:

typedef struct {

/* camera setup */

int MX_Used;                      /* track whether device is in use */

int MX_CommPort;                  /* requesed comm port */

int MX_OpenPort;                  /* actual comm port being used */

/* camera interface to world */

int MX_BlackEnable;               /* enable black level shifting */

int MX_BlackFine;                 /* set fine black level control */

int MX_BlackValue;                /* black level offset */

int MX_Gain;                      /* gain of on-board amp */

int MX_Tint;                      /* integration time */

int MX_Gamma;                     /* gamma correction enabled */

int MX_Mirror;                    /* left/right read-out enabled */

int MX_Standard;                  /* 50 or 60 Hz */

int MX_Interlaced;                /* interlace enabled */

int MX_Mode;                      /* camera triggering */

int MX_Sync;                      /* composite sync in output enabled */

int MX_Interface;                 /* TTL or differential */

int MX_Option1;                   /* option 1 enabled */

int MX_Option2;                   /* option 2 enabled */

int MX_QPixelClock;               /* read back from PSU */

int MX_QSensorType;               /* read back from PSU */

} mx12p_globals;

==============================================================================

==============================================================================