#

DECUS C LANGUAGE SYSTEM

RSX-11M V4.0 Executive and

F11ACP C Extensions Library

by

Robert B. Denny

DECUS Structured Languages SIG

Release V2.0

#18-Oct-82

#

.note Copyright (C) 1980,1982 DECUS

General   permission  to  copy  or

modify, but  not  for  profit,  is

hereby  granted, provided that the

above copyright notice is included

and reference  made  to  the  fact

that  reproduction privileges were

granted by DECUS.

The information in  this  document

is   subject   to  change  without

notice and should not be construed

as   a   commitment   by   Digital

Equipment Corporation or by DECUS.

Neither      Digital     Equipment

Corporation,   DECUS,   nor    the

authors  assume any responsibility

for the use or reliability of this

document    or    the    described

software.

This  software  is  made available

without any support whatsoever. If

you   are   responsible   for   an

implementation of this system, you

should    expect    to   have   to

understand and modify  the  source

code if you encounter any problems

in implementing or maintaining the

compiler  or its run-time library.

The DECUS Structured Languages SIG

is   the   primary    focus    for

communication  among users of this

software.

UNIX is   a   trademark   of    Bell    Telephone

Laboratories.   RSX,  RSTS/E,  RT11  and  VMS  are trademarks of

Digital Equipment Corporation.

This document describes the RSX-11M V4.0 Executive  Extensions  Library

for  the  DECUS C programming system. It assumes that the reader

is familiar with the C language and with the  RSX-11M  executive

services.  This  manual  should  be used in conjunction with the

RSX-11M V4.0 Executive Reference Manual AA-L675A-TC.

For the most part, the routines in this library  provide  a  convenient

run-time  interface  to the RSX-11M Executive's services from C.

In keeping with the efforts  to  baseline  and  standardize  the

common  C  runtime  library  functions,  the  RSX-11M extensions

library is distributed separately.

All but 3 of the RSX-11M V4.0 executive services are available  through

the extensions library. This includes:

* Indirect command file support

* Obtaining system and task information

* Timer services

* Full AST support in C

* Communication between tasks

* Program Logical Address Space (PLAS) services

* Suspend and resume execution of tasks

* Parent-offspring tasking

* Alternate CLI support

* Direct queueing of I/O requests (QIO's)

* Exit services

The  Connect  To  Interrupt  Vector and the 2 Specify SST Vector

Table services are not implemented.

With this comprehensive set  of  C-callable  functions,  including  AST

service,   it   should   be  possible  to  avoid  most  MACRO-11

programming in system applications.

In  addition,  a  number  of  ".H"  files  containing  structure

definitions,  offset  definitions  and  I/O  function and return

value  definitions  have  been  provided  to  facilitate  system

programming.  These files are a collection of things that I have

made   as  needed  and  are  by  no  means  a  complete  set  of

definitions.

Finally, three new routines have been added which make  it  very

convenient  to  access  files  via direct communication with the

Files-11 ACP, given only a filespec string, an FNB somewhere and

a free LUN.

Most of the routines in this  library  will  function  correctly

with  RSX-11M  V3.2.   Those  directives  that are not supported

under V3.2 will not be included in the library when  built  with

CXMAK.CMD if you specify V3.2.

.note WARNING

Some   of  the  routines  in  this

library have  never  been  tested.

Be  particularly suspicious of the

new additions for RSX-11M V4.0.

In general, the function calls take a form  similar  to  that  for  the

corresponding  system  macro  call.  In  a  few  cases, optional

parameters are not accessible from C.

All (but 1) of the C-callable functions are implemented with '$S'  form

system  macro  call.  In addition, care has been taken to ensure

that all (but 1) of the functions  are  re-entrant.  This  means

that  the  extensions  library  may  be  implemented as a shared

library, with one copy of the code in  memory  being  shared  by

multiple  programs.  The  exception  to  this  is  the  ^&gmcr\&

function. DEC provided no way to make this  re-entrant,  as  the

command line buffer is in the DPB.

.hl 1 ^&Access to External Labels and $DSW\&

DECUS C  allows  identifiers to contain the "$" character. In addition,

the "__" character eventually becomes "." in the object file. If

an identifier is declared external, it is  made  global  in  the

object file. This makes it possible to access externally labeled

locations.

The file  CX.H  contains  an "extern int $dsw;" declaration. This makes

the RSX-11M Directive Status Word ($DSW) accessible from C. This

file must be _#include'ed at the beginning of any program  using

the Extensions Library.

.note

Be   careful  about  using  global

"equates" (such as IS.SUC)  in  C.

These  are NOT address labels. For

example, if  you  declare  "extern

int   is__suc",  where  IS.SUC  is

externally equated to "1" and then

use is__suc in an expression,  you

will   get   the   ^&contents   of

location 1\& (and probably an  odd

address  trap!).  It  is  possible

(but very  tacky)  to  get  around

this  by  prefixing the use of the

equated symbol  in  an  expression

with  the  "_&" operator, since it

means "take  this  literally,  not

what  it  points  to". Please read

the next section.

.hl 1 ^&System Defined Symbols and Structures\&

When an executive service is requested  in  MACRO-11,  the  request  is

formed  by  expanding  a pre-defined macro. In many cases, there

are also pre-defined symbolic  codes,  symbolic  data  structure

offsets,  etc.  Since  these  values are not address labels, but

literal values, there is no semantically  clean  way  to  access

them globally in C.

The preferred  technique for using system defined global literals is to

_#define them in a header file or the  program  itself.  Several

header  files  are  supplied with the Extensions Library for use

with the functions requiring global literals. The  literals  are

defined  per  their MACRO-11 definitions (except "." in MACRO is

"__" in C).

There is one required header file, CX.H, which contains _#define's  for

all  of  the  DSW  return  codes  and the typdef definitions for

"word", "byte" and  "rad50".  This  file  must  be  _#include'ed

before  any  of the other extensions library header files, since

those definitions are used in the other files. See  the  example

program in appendix A.

Notice  that  most 16 bit quantities are defined as "word" which

is typdef'ed as  "unsigned".   This  reflects  their  nature  as

unsigned  quantities.  One  of the most common pitfalls in doing

system work from C is unintended sign extension when  converting

8-bit to 16-bit format or from 16-bit to 32-bit form.  Note that

there is no unsigned 32 bit type in C.

All of  the  Extensions  Library  header  file  _#define's were made by

extracting the appropriate  system  macro  from  RSXMAC.SML  and

munging  it  with  FASTEXT  or  TECO  into the required _#define

syntax and numeric radix. Hopefully, this means that the  values

are correct.

.note

By  convention,  the symbols which

are _#define'ed are capitalized.

.hl 1 ^&Optional Parameters\&

Many of  the  system  macros  have  optional   parameters.   When   the

corresponding  function is called from C, all parameters must be

present. However, a parameter value of  zero  is  equivalent  to

omitting the parameter in the macro call. For example:

cmkt(0,t2srv);	/* Cancel mark time served by t2srv() */

cmkt(7,0);		/* Cancel mark time using flag 7 */

cmkt(7,t2srv);	/* Combination of above 2 */

cmkt(0,0);		/* Cancel ALL mark time requests */

.hl 1 ^&Function return value\&

Most extensions  library  functions  return  the  value  of the RSX-11M

directive status word ($DSW). A few abort the task if they fail.

The DSW may be referred to as "$dsw" within the C  program  also

if  "extern  int  $dsw;"  is  stated,  or  if  the  file CX.H is

_#include'ed. The RSX-11M V4.0 Executive Reference Manual,  sec.

1.2,  explains  the  role  of  the DSW. The C header file, CX.H,

contains _#define's for all of the codes that may be returned in

the DSW. The meanings of these codes are given in the  Executive

Reference   Manual,  appendix  B.  In  addition,  the  directive

descriptions give the codes associated with that  directive.  To

include  the  definitions  in  your program, use the following C

compiler directive:

_#include <cx.h>

.hl 1 ^&Virtual Addresses in C\&

When interacting with the RSX-11M executive at the  level  provided  by

the  extensions library, it is inevitable that virtual addresses

will be manipulated and used as pointers. This  is  particularly

true  when  using  the  memory management functions. Also, the C

storage allocator functions  return  virtual  addresses,  not  C

pointers. It is important to make this distinction, owing to C's

powerful   address   arithmetic   capabilities.   See   ^&The  C

Programming Language\& by Kernighan and  Ritchie,  sections  5.4

and 5.6.

It is a kludge to define a virtual address as an integer. Since virtual

addresses  on the PDP-11 are "pointers to bytes (or characters)"

it is wise to adopt the convention of defining them as character

pointers. To make things crystal clear, the  file  CX.H  does  a

"typedef  char  *address;",  making  "address"  synonymous  with

"character pointer". See the file WDBDF.H for  an  example.  You

might  also define the standard allocator functions as returning

a value of type address.

.hl 1 ^&AST Service Routines\&

The RSX-11M C Extensions Library provides full AST service  capability.

Any  C function may serve as an AST service routine. The address

of the AST service  routine  is  passed  as  a  function-pointer

parameter  to  the  function which supports AST's. Normally, the

AST routine address is an optional parameter and  a  zero  value

has the effect of omitting the AST.

It is  necessary  to  understand the details of AST service startup and

exit. When an AST occurs, the executive ALWAYS pushes the task's

Event Flag mask word, its PSW, its  PC  and  its  DSW  (in  that

order,  DSW  on  the  top)  onto  the  task's stack. Consult the

RSX-11M V4.0 Executive Reference Manual, section 2.3.4, for  the

details of this.

In addition to the 4 task state words just mentioned, the executive MAY

push additional parameters, depending on the reason for the AST.

These  trap-dependent  parameters  are  often  needed by the AST

service routine. The extensions  library  contains  a  function,

^&gtdp\&,  which  allows the C user to access them. The ^&gtdp\&

function takes as its parameter the word offset from the top  of

the  stack  as it is shown in the Exec manual section 2.3.4, and

returns the integer value of the word at that  location  on  the

stack.

.note

The   normal  C  environment  save

routine (csv_$)  destroys  R0.   A

slightly  larger  version of these

routines are provided  within  the

CX    library,   in   the   module

ACSV.MAC.  The  csv_$  routine  in

ACSV.MAC    preserves   R0.   This

version of csv_$ and  cret_$  will

automatically be included in all C

programs   which  utilize  the  CX

library, whether they actually use

ASTs or not.

Normally, C routines do not save R0 and R1.  This has dire consequences

if an AST service routine modifies  R0  or  R1,  destroying  the

interrupted routine's context.  As just noted, the csv_$ routine

in  the  CX  library  preserves R0 and R1.  In addition, the AST

service routine MUST call the  ^&astset\&  function  immediately

upon  entry.   This  function  saves  R0  and R1. An AST service

routine (obviously) cannot pass a function return,  since  there

was  no  real  "caller".  The  ^&astx\&  routine mentioned below

restores R0 and R1.

Finally, but MOST IMPORTANTLY, the trap-dependent  parameters  must  be

removed  from  the  stack  before the executive AST service exit

routine is called. A C function which is used as an AST  service

routine  MUST  use  the  extensions library function ^&astx\& to

exit  the  function.  The  ^&astx\&  routine  takes  an  integer

parameter which is the number of words to remove from the top of

stack  prior to entering the executive AST service exit routine.

This  parameter  must  be  equal  to  the  number  of  words  of

trap-dependent  parameters  that the exec pushed when it started

the AST.  As  mentioned  in  the  previous  paragraph,  it  also

restores all of the registers.

.note

It  is very important that care be

taken in the use  of  ^&astx\&.  A

directive    error    causes    an

immediate task abort. An incorrect

value  for  the   stack   cut-back

parameter   will   lead   to  task

failure.

.hl 1 ^&Changes from the V3.2 Extensions Library\&

This  library  supersedes  the  V1.1  release  of  the   RSX-11M

Executive C Extensions Library.  The most notable changes are in

the  global  definitions  of the types "address", "word", "byte"

and "rad50". These changes have been incorporated into the  ".H"

files.   In  addition, several directives new to RSX-11M Version

4.0 have been  added.   Also,  the  P-section  names  have  been

changed to those generated by the current C compiler.

Three  new  functions, setfnb(), xparse() and dirset() have been

added to allow file access via  direct  communication  with  the

Files-11   ACP.   While  not  actually  executive  calls,  these

routines make it possible to do useful file-oriented  processing

at  the system level.  They also could serve as the basis for an

RSX-specific "standard I/O library"  which  is  streamlined  and

does  not  require  FCS  etc. (like Dave Conroy's original one).

Any volunteers?

The library is still usable under  V3.2,  and  if  you  use  the

"CXMAK.CMD"  file  to  build the library, the routines which are

new to V4.0 will not be built into the library.  The reason  for

this  is  that  the  macros will not expand when the V3.2 system

macro library is used.

Each function description consists of the function  name,  the  English

name,  parameter  type  definitions  in C, and the function call

format in C. The corresponding description in Chapter 5  of  the

RSX-11M  V4.0  Executive  Reference  Manual  should be used as a

primary reference. If system symbols are involved, there will be

a header file for use with  the  C  function,  and  it  will  be

specified in the function description.

The following  is  a  list  of the RSX-11M executive services and their

function calls provided by  the  extensions  library.  Functions

marked with "(V4)" are available only with RSX-11M V4.0:

Abort Task				abrt

Alter task priority			altp

Assign LUN				alun

AST routine setup			astset**

AST service exit			astx

Attach region				atrg

Clear event flag			clef

Cancel mark time requests		cmkt

Connect					cnct

Create address window			craw

Create group global event flags		crgf

Create region				crrg

Cancel scheduled requests		csrq

Declare significant event		decl

Disable AST recognition			dsar

Disable checkpointing			dscp

Detach region				dtrg

Eliminate address window		elaw

Eliminate group global event flags	elgf

Emit status                             emst (V4)

Enable AST recognition			enar

Enable checkpointing			encp

Exit iff event flag not set		exif

Exit					exitt <--

Exit with status			exst

Extend task				extk

Get Command for CLI                     gcci (V4)

Get CLI information			gcii (V4)

Get LUN information			glun

Get MCR command line			gmcr

Get mapping context			gmcx

Get partition parameters		gprt

Get region parameters			greg

Get trap-dependent parameters**		gtdp**

----------------

** Not an RSX-11M executive service ! Get time parameters gtim

Get task parameters gtsk Map address window map Mark time mrkt

Queue  I/O request qio Queue I/O request and wait qiow Receive

data or stop rcst Receive data rcvd Receive data or exit  rcvx

Read all event flags rdaf Read extended event flags rdxf

Request and  Pass  Offspring Information  rpoi (V4) Request task

rqst Receive by reference rref Resume task rsum Run task run

Specify Command Arrival AST             scaa (V4)

Set Command Line Interpreter            scli (V4)

Set up  FNB                             setfnb** Send data sdat

Send, Request and Connect               sdrc (V4)

Send Data, Request and Pass OCB         sdrp (V4) Set event flag

setf Specify FPP exception AST sfpa

Send Message                            smsg (V4)  Suspend  task

spnd Spawn task spwn Specify receive data AST srda

Specify Requested   Exit  AST               srex  (V4)  Send  by

reference sref Specify receive by reference AST srra

Set System  Time                          stim  (V4)  Stop   for

logical  OR of eflags 1-16 stlo0 Stop for logical OR of eflags

17-32 stlo1 Stop for logical OR of eflags 33-48 stlo2 Stop for

logical OR of eflags 49-64 stlo3 Stop for logical OR of eflags

65-80 stlo4 Stop for logical OR of  eflags  81-96  stlo5  Stop

task stop Stop for single event flag stse

Unlock Group  Global Event Flags         ulgf (V4) Unmap address

window umap Unstop task ustp Wait for significant  event  wsig

Wait  for  logical OR of eflags 1-16 wtlo0 Wait for logical OR

of eflags 17-32 wtlo1 Wait for  logical  OR  of  eflags  33-48

wtlo2  Wait  for  logical  OR  of  eflags 49-64 wtlo3 Wait for

logical OR of eflags 65-80 wtlo4 Wait for logical OR of eflags

81-96 wtlo5 Wait for single event flag wtse

.hl 1 ^&Abort Task\&

********

* ABRT *

********

File name:	abrt.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word status;		/* Returned $DSW */

status = abrt(tsknam);

[end]

.hl 1 ^&C register save and restore with AST support\&

*********

* ACSV$ *

*********

File name:	acsv.mac

Usage

jsr	r5,csv$

_...

jmp	cret$

Description

C program Run-time Environment with RSX AST's

Each C subroutine starts with a call to CSV$  and  exits

by  jumping  to CRET$.  Upon exit, the stack need not be

equal to its value on entrance. This  version  preserves

R0, so that AST's will work.

During  the  execution of all C subroutines, register R5

points  to  the   current   "environment."    Within   a

subroutine, it appears as follows:

______________________________

|		|

SP ->	| 1st loc. var. | -10(R5)	C$AUTO-2(R5)

|______________________________|

|		|

| Saved R2	|  -6(R5)

|______________________________|

|		|

| Saved R3	|  -4(R5)

|______________________________|

|		|

| Saved R4	|  -2(R5)

|______________________________|

|		|

R5 ->	| Saved R5	|

|______________________________|

|		|

| Return add.	|  +2(R5)

|______________________________|

|		|

| First arg.	|  +4(R5)	C$PMTR+0(R5)

|______________________________|

|		|

| Second arg.	|  +6(R5)	C$PMTR+2(R5)

|______________________________|

Within  a subroutine, Registers R0-R4 and the top of the

stack, (sp) are available for use. Registers R0  and  R1

are not preserved by subroutines and may be used to pass

a return value.

R5  must  not be modified by a subroutine.  All variable

addressing must be done by offsets  to  R5.   Subroutine

arguments  must  be  accessed  by  reference  to C$PMTR.

Subroutine local variables must be accessed by reference

to  C$AUTO.   This  permits  modification   of   calling

sequences without rewriting all subroutines.

CSV$ refers to global symbol $$main to call the run-time

startup program from the (RSX) library.

An  RSX  AST routine must call astset() immediately upon

entry to save R0 and R1. These registers are restored by

astx(), which must be used to exit from an  AST  service

routine.  See  the  documentation  of  the  CX  library,

CX.DOC.

[end]

.hl 1 ^&Alter Priority\&

********

* ALTP *

********

File name:	altp.mac

Usage

_#include <cx.h>

word nprio;		/* New priority 1-250 decimal */

rad50 tsknam[2];	/* 2 word RAD50 task name */

word status;		/* Returned DSW */

status = altp(tsknam, nprio)	/* Returns DSW */

[end]

.hl 1 ^&Assign LUN\&

********

* ALUN *

********

File name:	alun.mac

Usage

_#include <cx.h>

word lun;	/* Logical Unit Number */

word dnam;	/* Dev. name: 2 ASCII char's */

/* packed into an "int" */

word unit;	/* Device unit number */

word status;	/* Returned DSW */

status = alun(lun, dnam, unit);

Description

This  version  of  alun  is   retained   for   backwards

compatibility  with  previous  versions  of  CX.   It is

recommended that the alunx() function  be  used  in  new

programs  due  to the ease with which the device name is

passed.

Notes

A union may be used to pack the device name into an int:

union	{

int dnam;

char dev[2];

} device;

etc...

[end]

.hl 1 ^&Assign LUN (Extended)\&

*********

* ALUNX *

*********

File name:	alunx.mac

Usage

_#include <cx.h>

word lun;	/* Logical Unit Number */

char *dnam;	/* Device name string */

word unit;	/* Device unit number */

word status;	/* Returned DSW */

status = alunx(lun, dnam, unit);

Description

This version of the assign LUN directive has been  added

to  make  it  easier to use from C.  The alun() function

requires that the 2-character device name be packed into

a word and passed by value.  That was  in  keeping  with

the  general  philosophy  of keeping the mapping between

the C call and the MACRO usage  as  close  as  possible.

However, packing the device name requires a hack such as

the use of a union, and useless run-time code to fill in

the union since DECUS C cannot initialize a union.

alunx()  takes  the device name as a string, the address

of which is passed as the parameter.  Only the first two

letters of the string are used, so the string may be  an

entire  file specification which begins with an explicit

device name, or may be the device name field of an FNB.

The most common usage, however, will be:

status = alunx(lun, "XX", unit);

Notes

Only the 1st two characters of the string are used.

No null is needed at the  end  of  the  string,  so  the

address  passed  may be that of the device name field of

an FNB.

No checks are made for the validity of  the  characters,

whatever is given will be passed to the directive.

The string need not be word-aligned.

[end]

.hl 1 ^&Set AST service environment\&

**********

* ASTSET *

**********

File name:	astset.mac

Usage

_#include <cx.h>

astset();

Description

This  function  must be called immediately upon entrance

to any function which is used as an AST service routine.

Since  AST  service  is  (obviously)  asynchronous,  ALL

registers  must  be preserved across the execution of an

AST service routine.  astset() handles this  requirement

by  saving  R0 and R1 in a transparent manner by tucking

them  under  the  normal  C  stack  frame.   The  astx()

function automatically restores the original environment

prior to issuing the AST service exit directive.

This  means  that  a  function  which  is used as an AST

service routine may NOT be used as  a  normal  function.

It must always exit with a call to astx()!

For more information on usage of C AST service routines,

see the section on AST service in the CX manual.

[end]

.hl 1 ^&AST Service Exit\&

********

* ASTX *

********

File name:	astx.mac

Usage

_#include <cx.h>

word n;	/* _# words of trap-dep. param's for AST */

astx(n);	/* AST exit, cut n WORDS off stack */

Description

This  "function"  effects  exit  from  a  C  AST service

routine. It MUST be called to exit from such a  routine;

a   "return"  will  fail  owing  to  the  unusual  stack

structure which has R0 and R1 saved in a magic  location

and  may  have  trap-dependent parameters there as well.

See the documentation on C AST service routine usage  in

the CX manual for more info.  Also see the documentation

on astset().

If the particular type of AST starts with trap-dependent

parameters on the stack, astx MUST be directed to remove

them prior to doing the actual AST service exit. See the

RSX-11M  V4.0 Executive Reference Manual, sections 2.3.3

and 2.3.4. Call with a 0 if there are none.

Notes

Failure of AST exit causes task abort.

Bugs

Does  unnecessary  thrashing  if  no  TDPs  are  to   be

stripped.

.hl 1 ^&Attach Region\&

********

* ATRG *

********

File name:	atrg.mac

Usage

_#include <cx.h>

_#include <rdbdf.h>	/* RDB struct definition */

struct rdb regblk;	/* Region Definition Block */

word status;		/* Returned DSW */

status = atrg(_&regblk);

[end]

.hl 1 ^&Clear Event Flag\&

********

* CLEF *

********

File name:	clef.mac

Usage

_#include <cx.h>

word efn;		/* Event flag number */

word status;		/* Returned DSW */

status = clef(efn);

[end]

.hl 1 ^&Cancel Mark Time Requests\&

********

* CMKT *

********

File name:	cmkt.mac

Usage

_#include <cx.h>

word efn;		/* Event flag number */

{typ} astsrv();		/* AST service function */

word status;		/* Returned DSW */

status = cmkt(efn, astsrv);

Description

Cancel mark time requests specified by efn, AST routine,

both,  or cancel all mark time requests. See the RSX-11M

V4.0 Exec Ref Manual, section 5.3.8.

[end]

.hl 1 ^&Connect to Task\&

********

* CNCT *

********

File name:	cnct.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* Offspring task name */

word eflag;		/* Event flag number */

{typ} astsrv();		/* AST service routine */

word esb[8];		/* 8 word status block */

word status;		/* Returned DSW */

status = cnct(tsknam, eflag, astsrv, esb)

[end]

.hl 1 ^&Create Address Window\&

********

* CRAW *

********

File name:	craw.mac

Usage

_#include <cx.h>

_#include <wdbdf.h> 	/* WDB definitions */

struct wdb winblk;	/* Actual WDB */

word status;		/* Returned DSW */

status = craw(_&winblk);

[end]

.hl 1 ^&Create Group Global Event Flags\&

********

* CRGF *

********

File name:	crgf.mac

Usage

_#include <cx.h>

word group;		/* UIC group number or 0 */

word status;		/* Returned DSW */

status = crgf(group);

[end]

.hl 1 ^&Create Region\&

********

* CRRG *

********

File name:	crrg.mac

Usage

_#include <cx.h>

_#include <rdbdf.h> 	/* RDB definitions */

struct rdb regblk;	/* Region definition block */

word status;		/* Returned DSW */

status = crrg(_&regblk);

[end]

.hl 1 ^&Cancel Scheduled Requests\&

********

* CSRQ *

********

File name:	csrq.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word status;		/* Returned DSW */

status = csrq(tsknam);

[end]

.hl 1 ^&Declare Significant Event\&

********

* DECL *

********

File name:	decl.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = decl();

[end]

.hl 1 ^&Disable AST Recognition\&

********

* DSAR *

********

File name:	dsar.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = dsar();

[end]

.hl 1 ^&Disable Checkpointing\&

********

* DSCP *

********

File name:	dscp.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = dscp();

[end]

.hl 1 ^&Detach Region\&

********

* DTRG *

********

File name:	dtrg.mac

Usage

_#include <cx.h>

_#include <rdbdf.h> 	/* RDB Definitions */

struct rdb regblk;	/* Region Definition Block */

word status;		/* Returned DSW */

status = dtrg(_&regblk);

[end]

.hl 1 ^&Eliminate Address Window\&

********

* ELAW *

********

File name:	elaw.mac

Usage

_#include <cx.h>

_#include <wdbdf.h> 	/* WDB definitions */

struct wdb winblk;	/* Window definition block */

word status;		/* Returned DSW */

status = elaw(_&winblk);

[end]

.hl 1 ^&Eliminate Group Global Event Flags\&

********

* ELGF *

********

File name:	elgf.mac

Usage

_#include <cx.h>

word group;		/* Group number */

word status;		/* Returned DSW */

status = elgf(group);

[end]

.hl 1 ^&Emit Status\&

********

* EMST *

********

File name:	emst.mac

Usage

_#include <cx.h>

rad50 tname[2];		/* Spec'd connected task */

word estat;		/* Emitted status value */

word dstat;		/* Returned DSW */

dstat = emst(tname, estat);

[end]

.hl 1 ^&Enable AST recognition\&

********

* ENAR *

********

File name:	enar.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = enar();

[end]

.hl 1 ^&Enable Checkpointing\&

********

* ENCP *

********

File name:	encp.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = encp();

[end]

.hl 1 ^&Exit If\&

********

* EXIF *

********

File name:	exif.mac

Usage

_#include <cx.h>

word efn;		/* Event flag number */

word status;		/* Returned DSW */

status = exif(efn);

[end]

.hl 1 ^&Exit Without Status\&

*********

* EXITT *

*********

File name:	exitt.mac

Usage

_#include <cx.h>

exitt();

Notes

The function name is wierd to avoid  clash  with  normal

exit() routine.

Failure results in task abort.

[end]

.hl 1 ^&Exit With Status\&

********

* EXST *

********

File name:	exst.mac

Usage

_#include <cx.h>

word statwd;		/* Exit status code */

exst(ststwd);

Notes

Exit  status codes (EX$xxx) in CX.H. Failure causes task

abort.

[end]

.hl 1 ^&Extend Task\&

********

* EXTK *

********

File name:	extk.mac

Usage

_#include <cx.h>

word delta;		/* + or	task size change */

word status;		/* Returned DSW */

status = extk(delta);

[end]

.hl 1 ^&Get Command Line for CLI\&

********

* GCCI *

********

File name:	gcci.mac

Usage

_#include <cx.h>

_#include <clidf.h>	/* CLIPB _& message codes */

_#include <ocbdf.h>	/* OCB definitions */

_#include <gccidf.h>	/* GCCI-specific structures */

struct cmdbuf cmd;	/* Command buffer */

struct cmibuf info;	/* Information buffer */

address caddr;		/* Spec'd cmd address in pool */

word ncp;		/* Action if no command */

word status;		/* Returned DSW */

status = gcci(_&cmd, _&info, caddr, ncp);

Notes

Only CLI tasks may issue this directive.

The file CLIDF.H contains the structure definitions  for

the  command  and  information  buffers,  and  the "ncp"

symbols.

You  should   thoroughly   familiarize   yourself   with

Alternate CLI operations before using this directive.

The  sizes  of  the  command and information buffers are

fixed  by  the   structure   definitions   in   CLIDF.H,

therefore,  they  are  not explicitly passed as function

parameters.

[end]

.hl 1 ^&Get CLI Information\&

********

* GCII *

********

File name:	gcii.mac

Usage

_#include <cx.h>

_#include <gcibuf.h>

struct gcibuf clidat;	/* CLI info buffer */

rad50 clinam[2];	/* RAD50 CLI name (optional) */

char *tiname;		/* TI: Device name string */

word tiunit;		/* TI: Unit number */

word status;		/* Returned DSW */

status = gcii(_&clidat, clinam, tiname, tiunit);

Notes:

Terminal and CLI names are  optional.   Do  not  specify

both. See the notes in the Executive Manual.

This  directive  takes  the TI: device name as a string,

rather than 2 ASCII characters packed into a word.

[end]

.hl 1 ^&Get LUN Information\&

********

* GLUN *

********

File name:	glun.mac

Usage

_#include <cx.h>

_#include <lunbuf.h> 	/* Defines struct lunbuf */

struct lunbuf lundat;	/* LUN info buffer */

word lun;		/* LUN */

word status;		/* Returned DSW */

status = glun(lun, _&lundat);

[end]

.hl 1 ^&Get MCR Command Line\&

********

* GMCR *

********

File name:	gmcr.mac

Usage

***********************

***  NON RE-ENTRANT ***

***********************

_#include <cx.h>

char *cmdlin;		/* Must be a pointer */

cmdlin = gmcr();

Description

This  function  departs  from  the  usual   CX   library

convention  of  keeping  the  functions re-entrant.  The

GMCR$  directive  itself  is  non-reentrant;   the   DPB

receives  the  actual command line, the address of which

is passed to the caller unless there was an error.

Notes

Returns address of first byte of command line or NULL if

failed.

$DSW contains character count  of  gotten  cmd  line  if

success, else

##### the  DSW  error  code.  See the Exec Reference Manual sec.

5.3.33.

[end]

.hl 1 ^&Get Mapping Context\&

********

* GMCX *

********

File name:	gmcx.mac

Usage

_#include <cx.h>

_#include <wdbdf.h>

struct wdb wdbvec[n];	/* Array of WDB's */

word status;		/* Returned DSW */

status = gmcx(_&wdbvec)	/* Note the '_&' to pass the address */

Notes

You must have enough WDB's for the _# of windows.

[end]

.hl 1 ^&Get Partition Parameters\&

********

* GPRT *

********

File name:	gprt.mac

Usage

_#include <cx.h>

_#include <pparbf.h>

rad50 prtnam[];		/* RAD50 partition name */

struct pparbf pardat;	/* Partition param's buffer */

word status;		/* Returned DSW */

status = gprt(prtnam, _&pardat);

Notes

If a NULL pointer to the partition name is  passed,  the

parameters  returned  are  those  of  the issuing task's

unnamed subpartition. If the first word of the partition

name is zero, this function returns a  code  of  IE.PNS,

rather  than  letting the exec try to interpret the call

as a GREG$S.

[end]

.hl 1 ^&Get Region Parameters\&

********

* GREG *

********

File name:	greg.mac

Usage

_#include <cx.h>

_#include <rparbf.h>

rad50 regnam[2];	/* RAD50 region name */

struct rparbf regdat;	/* Region parameters buffer */

word status;		/* Returned DSW */

status = greg(regnam, _&regdat);

Notes

If a NULL pointer(!) to the region name is  passed,  the

parameters  returned are those of the task region of the

issuing task.

[end]

.hl 1 ^&Get Trap-Dependent Parameter\&

********

* GTDP *

********

File name:	gtdp.mac

Usage

_#include <cx.h>

word woff;	/* WORD offset to TDP */

word tdp;	/* Ret. Trap-Dep. Parameter */

tdp = gtdp(woff);

Description

This routine is not an executive services  call.  It  is

the mechanism by which an AST service routine may access

the   interrupted   task's   state   or  trap  dependent

parameters pushed on to the stack when the AST  occured.

The  RSX-11M  V3.2  Executive Reference Manual, sections

2.3.3 and 2.3.4 contain critical information  concerning

AST's and their service.

The parameter supplied with the call to gtdp is the WORD

offset  from  the  top  of the stack AS SPECIFIED IN THE

EXEC MANUAL  SECTION  2.3.4  to  the  AST  parameter  of

interest.   There  are  7 (11 for 'M-plus) variations of

the task's stack at AST, as explained in  that  section.

For  example,  if the AST was due to the expiration of a

mark time, variation _#5, the gtdp function would return

the following:

gtdp(0)	Event flag     [trap-dependent]

gtdp(1)	Task's DSW     [task state]

gtdp(2)	Task's old PC  [task state]

gtdp(3)	Task's old PSW [task state]

gtdp(4)	Task's EF Mask [task state]

Notes

The parameter is a WORD offset. This is for  safety,  to

avoid  odd  address  traps,  since  the  access  in this

routine is by word instructions.

No checks are made for offset legality.

.hl 1 ^&Get Time Parameters\&

********

* GTIM *

********

File name:	gtim.mac

Usage

_#include <cx.h>

_#include <timbuf.h>

struct timbuf timdat;	/* Time parameters buffer */

word status;		/* Returned DSW */

status = gtim(_&timdat);

[end]

.hl 1 ^&Get Task Parameters\&

********

* GTSK *

********

File name:	gtsk.mac

Usage

_#include <cx.h>

_#include <tparbf.h>

struct tparbf tskdat;	/* task param's structure */

word status;		/* Returned DSW */

status = gtsk(_&tskdat);

[end]

.hl 1 ^&Map Address Window\&

*******

* MAP *

*******

File name:	map.mac

Usage

_#include <cx.h>

_#include <wdbdf.h>

struct wdb winblk;	/* WDB */

word status;		/* Returned DSW */

status = map(_&winblk);

[end]

.hl 1 ^&Mark Time\&

********

* MRKT *

********

File name:	mrkt.mac

Usage

_#include <cx.h>

word eflag;	/* Event flag number */

word tmg;	/* Time interval magnitude */

word tnt;	/* Time interval units */

{typ} astsrv();	/* AST service routine */

word status;	/* Returned DSW */

status = mrkt(eflag, tmg, tnt);

[end]

.hl 1 ^&Queue I/O Request\&

*******

* QIO *

*******

File name:	qio.mac

Usage

_#include <cx.h>

_#include <qiofun.h>  /* I/O Function codes */

_#Include <qioret.h>  /* I/O Status return codes */

word func;	/* I/O function code */

word lun;	/* Logical unit number */

word eflag;	/* Event flag number */

word iosb[2];	/* I/O status block */

{typ} astsrv();	/* AST service routine */

int devpar[6];	/* Array of 6 parameters */

word status;	/* Returned DSW */

status = qio(func,lun,eflag,iosb,astsrv,devpar);

Notes

_#include  <qiottd.h>  for   FDX/HDX   terminal   driver

function, subfunction and return status codes.

[end]

.hl 1 ^&Queue I/O Request and Wait\&

********

* QIOW *

********

File name:	qiow.mac

Usage

_#include <cx.h>

_#include <qiofun.h>  /* I/O Function codes */

_#Include <qioret.h>  /* I/O Status return codes */

word func;	/* I/O function code */

word lun;	/* Logical unit number */

word eflag;	/* Event flag number */

word iosb[2];	/* I/O status block */

{typ} astsrv();	/* AST service routine */

int devpar[6];	/* Array of 6 parameters */

word status;	/* Returned DSW */

status = qiow(func,lun,eflag,iosb,astsrv,devpar);

Notes

_#include   <qiottd.h>   for   FDX/HDX  terminal  driver

function, subfunction and return status codes.

A Non-zero event flag number must  be  supplied  or  the

function  will  return  prior  to  completion of the I/O

request (i.e., the wait will fail).

.hl 1 ^&Receive Data or Stop\&

********

* RCST *

********

File name:	rcst.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word buffer[15];	/* Received data packet */

word status;		/* Returned DSW */

status = rcst(tsknam, buffer);

Notes

The first 2 words  of  the  'buffer'  will  contain  the

sender task name.

To  receive  data from any task, 'tsknam' must be a NULL

pointer.

[end]

.hl 1 ^&Receive Data\&

********

* RCVD *

********

File name:	rcvd.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word buffer[15];	/* Received data packet */

word status;		/* Returned DSW */

status = rcvd(tsknam, buffer);

Notes

The first 2 words  of  the  'buffer'  will  contain  the

sender task name.

To  receive  data from any task, 'tsknam' must be a NULL

pointer.

[end]

.hl 1 ^&Receive Data or Exit\&

********

* RCVX *

********

File name:	rcvx.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word buffer[15];	/* Received data packet */

word status;		/* Returned DSW */

status = rcvx(tsknam, buffer);

Notes

The first 2 words  of  the  'buffer'  will  contain  the

sender task name.

To  receive  data from any task, 'tsknam' must be a NULL

pointer.

[end]

.hl 1 ^&Read All Event Flags\&

********

* RDAF *

********

File name:	rdaf.mac

Usage

_#include <cx.h>

word buffer[4];		/* Event flags */

word status;		/* Returned DSW */

status = rdaf(buffer);

[end]

.hl 1 ^&Read Extended Event Flags\&

********

* RDXF *

********

File name:	rdxf.mac

Usage

_#include <cx.h>

word buffer[6];		/* 6 words of flags */

word status;		/* Returned DSW */

status = rdxf(buffer);

[end]

.hl 1 ^&Request and Pass Offspring Information\&

********

* RPOI *

********

File name:	rpoi.mac

Usage

_#include <cx.h>

_#include <ocbdf.h>	/* OCB def. (CLI tasks only) */

/*

* Codes for 'sc' parameter:

*/

_#define  RP__OEX  0200	/* Force exit */

_#define  RP__OAL  0001	/* Pass all OCB's */

_#define  RP__ONX  0002	/* Pass 1st OCB */

rad50 tname[2];		/* Offspring task name */

word uic;		/* Offspring task UIC */

rad50 parent[2];	/* Parent task name */

char cmdlin[];		/* Command line buffer */

word cmdlgt;		/* Length of command line */

word sc;		/* Mode flag(s) */

char *tiname;		/* Name of TI: device */

word tiunit;		/* Unit _# of TI: device */

rad50 rtask[2];		/* Run task name (CLIs only) */

struct ocb *ocbpt;	/* Address of OCB (CLIs only) */

word status;		/* Returned DSW */

status = rpoi(tname,uic,parent,cmdlin,cmdlgt,

sc,tiname,tiunit,rtask,ocbpt);

Notes

Be sure to read and understand the description  of  this

directive in the Exec Reference manual section 5.3.53.

The  device name of the TI: device is passed as a string

address.  Only the 1st two characters of the string  are

used.

The 'rtask' and 'ocb' parameters must be zero unless the

issuing task is a CLI.

This  directive  requires  substantial  stack  space  to

construct the DPB.

.hl 1 ^&Request Task\&

********

* RQST *

********

File name:	rqst.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word status;		/* Returned DSW */

status = rqst(tsknam);

Notes

Task is run under the UIC in its task header.

[end]

.hl 1 ^&Receive by Reference\&

********

* RREF *

********

File name:	rref.mac

Usage

_#include <cx.h>

_#include <wdbdf.h>

struct wdb winblk;	/* Window definition block */

word status;		/* Returned DSW */

status = rref(_&winblk);

[end]

.hl 1 ^&Resume Task\&

********

* RSUM *

********

File name:	rsum.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word status;		/* Returned DSW */

status = rsum(tsknam);

[end]

.hl 1 ^&Run Task\&

*******

* RUN *

*******

File name:	run.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word smg;		/* Schedule delta magnitude */

word snt;		/* Schedule delta units */

word rmg;		/* Reschedule interval mag. */

word rnt;		/* Reschedule interval units */

word status;		/* Returned DSW */

status = run(tsknam, smg, snt, rmg, rnt);

Notes

Uses  requested  task's  default  UIC.  See   the   Exec

reference manual sec. 5.3.57

[end]

.hl 1 ^&Specify Command Arrival AST\&

********

* SCAA *

********

File name:	scaa.mac

Usage

_#include <cx.h>

{typ} astsrv();		/* AST service routine */

word status;		/* Returned DSW */

status = scaa(astsrv);

Notes

See  the  documentation  on  C AST service. 'astsrv' = 0

disables the AST. This function is legal only for CLI's.

[end]

.hl 1 ^&Set Command Line Interpreter\&

********

* SCLI *

********

File name:	scli.mac

Usage

_#include <cx.h>

rad50 clinam[2];	/* RAD50 CLI name */

char *tiname;		/* TI: device name */

word tiunit;		/* TI: unit number */

word status;		/* Returned DSW */

status = scli(clinam, tiname, tiunit);

Notes

Issuing task must be privileged or a CLI.

[end]

.hl 1 ^&Send Data\&

********

* SDAT *

********

File name:	sdat.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* 2 word RAD50 task name */

word buffer msg[13];	/* 13 word buffer */

word eflag;		/* Local event flag number */

word status;		/* Returned DSW */

status = sdat(tsknam, msg, eflag);

[end]

.hl 1 ^&Send, Request and Connect\&

********

* SDRC *

********

File name:	sdrc.mac

Usage

_#include <cx.h>

rad50 tname[2];		/* Offspring task name */

word msgbuf[13];	/* 13-word buffer to send */

word efn;		/* Event flag */

{typ}astsrv();		/* AST service routine */

word esb[8];		/* Exit status block */

word status;		/* Returned DSW */

status = sdrc(tname, msgbuf, efn, astsrv, esb);

Notes

Only the 1st word of the ESB is used.

[end]

.hl 1 ^&Send, Request and Pass OCB\&

********

* SDRP *

********

File name:	sdrp.mac

Usage

_#include <cx.h>

_#include <ocbdf.h>	/* OCB Definitions */

/*

* 'flag' parameter symbolics

*/

_#define  SD__REX  128	/* Force exit */

_#define  SD__RAL    1	/* Pass all OCB's */

_#define  SD__RNX    2	/* Pass 1st OCB */

rad50 tname[2];		/* Offspring task name */

word msgbuf[13];	/* 13-word buffer */

word efn;		/* Event flag */

word flag;		/* Exec. flags (see above) */

rad50 parent[2];	/* Parent task name */

struct ocb *ocbpt;	/* Address of OCB (CLIs only) */

word status;		/* Returned DSW */

status = sdrp(tname,msgbuf,efn,flag,parent,ocbpt);

[end]

.hl 1 ^&Set Event Flag\&

********

* SETF *

********

File name:	setf.mac

Usage

_#include <cx.h>

word eflag;		/* Event flag number */

word status;		/* Returned DSW */

status = setf(eflag);

[end]

.hl 1 ^&Fill in File Name Block\&

**********

* SETFNB *

**********

File name:	setfnb.mac

Usage

_#include <cx.h>

_#include <fnb.h> 	/* Filename Block def. */

struct fnb xfnb;	/* Filename block */

char *fsp;		/* File spec string */

int lun;		/* LUN to use for lookups */

char *dext;		/* Default extension */

int status;		/* Status returned */

status = setfnb(_&xfnb, fsp, lun, dext);

Description

This function is not strictly a mapping  of  an  RSX-11M

executive  directive into a C call.  It is provided with

the CX library to  allow  block-oriented  file  I/O  and

control  operations  with  the Files-11 ACP, rather than

using the FCS or RMS I/O library functions.

setfnb() performs the messy part of getting a file  open

on  a  specified  LUN by performing all filespec parsing

and directory lookup operations required to  fill  in  a

Filename  Block (FNB).  All that is needed as inputs are

a filespec string and a free LUN.

Default directory and device  processing  are  performed

transparently,  i.e.,  the  filespec  need not be "fully

qualified".

If the file already exists, all that remains to be  done

to get the file open is to issue one of the Files-11 ACP

"file  access"  QIO's.  You may then read and write blox

with the IO.RVB and IO.WVB functions.

If the file does not exist, setfnb()  will  return  with

IE.NSF  (No such file).  In this case, you must manually

issue the  Files-11  "create  file"  (IO.CRE)  function,

followed  by  a  directory enter (IO.ENA) function, thus

creating the file. All of the fields in the  FNB  except

N.FID  are  valid upon return from setfnb() if the error

is IE.NSF.

Notes

Returns Directive or I/O status. If hi byte < 0,  status

is directive status, else is I/O status.

Errors  from  filespec  parsing  or directory operations

return IE.BAD (for lack of anything better).

If setfnb() returns with a status  of  IE.NSF  (no  such

file)  all fields except N.FID are silll usable, and the

FNB is all set for use in creating a new file.

[end]

.hl 1 ^&Specify FPP Exception AST\&

********

* SFPA *

********

File name:	sfpa.mac

Usage

_#include <cx.h>

{typ} astsrv();		/* AST service routine */

word status;		/* Returned DSW */

status = sfpa(astsrv);

[end]

.hl 1 ^&Send Message to ERRLOG\&

********

* SMSG *

********

File name:	smsg.mac

Usage

_#include <cx.h>

_#include <exedf.h>	/* To get ERRLOG symbols */

char *dspkt;		/* Data Subpacket Buffer */

word dsplen;		/* Length of packet, bytes */

word erlprm[4];		/* ERRLOG packet parameters */

word status;		/* Returned DSW */

status = smsg(dspkt, dsplen, erlprm);

Description

This directive is quite poorly documented in the RSX-11M

V4.0 Executive Reference Manual.  What it actually  does

is  to  generate  an  ERRLOG  packet and queue it to the

error logger task.  If a  data  subpacket  is  specified

('dspkt'  is  non-null),  it  is included in the packet.

You must have formatted the data subpacket in some  form

that can be recognized by the RPT report generator task,

however.

Study  the error logger packet format in the appendix of

the  Guide  to  Writing  I/O  drivers   manual,   EPKDF$

definition.  Also read the code in the Executive listing

for  the  ERROR   module.   It   is   possible,   though

non-trivial,  to add another packet type code to the RPT

report generator  and  use  SMSG  to  put  custom  error

logging  messages  into your error log.  See the code in

DISPATCH.CNF.

Notes:

Contrary to the documentation in the  Exec  manual,  the

issuing task need be privileged only if it is issuing an

SMSG directive which zeroes the I/O and error counts for

a specified device.

.hl 1 ^&Suspend Current Task\&

********

* SPND *

********

File name:	spnd.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = spnd();

[end]

.hl 1 ^&Specify Power Recovery AST\&

********

* SPRA *

********

File name:	spra.mac

Usage

_#include <cx.h>

{typ} astsrv();		/* AST service function */

word status;		/* Returned DSW */

status = spra(astsrv);

[end]

.hl 1 ^&Spawn Task\&

********

* SPWN *

********

File name:	spwn.mac

Usage

_#include <cx.h>

rad50 tsknam[2]; 	/* 2 word RAD50 task name */

word uic;		/* UIC of offspring task */

word eflag;		/* Event flag number */

{typ} astsrv();		/* AST service routine */

word esb[8];		/* 8 word status block */

char *cmdlin;		/* --> command line */

word cmdlgt;		/* Length (bytes) of cmd line */

word status;		/* Returned DSW */

status = spwn(tsknam,uic,eflag,astsrv,esb,

cmdlin,cmdlgt);

Notes

Does not support specified TI:.

[end]

.hl 1 ^&Specify Receive Data AST\&

********

* SRDA *

********

File name:	srda.mac

Usage

_#include <cx.h>

{typ} astsrv();		/* AST service function */

word status;		/* Returned DSW */

status = srda(astsrv);

Notes

Call with NULL pointer to turn off receive data AST's.

[end]

.hl 1 ^&Send by Reference\&

********

* SREF *

********

File name:	sref.mac

Usage

_#include <cx.h>

_#include <wdbdf.h>	/* WDB definition */

rad50 tsknam[2];	/* Receiver task name */

word eflag;		/* Event flag number */

struct wdb winblk;	/* Window definition block */

word status;		/* Returned DSW */

status = sref(tsknam, _&winblk, eflag);

[end]

.hl 1 ^&Specify Requested Exit AST\&

********

* SREX *

********

File name:	srex.mac

Usage

_#include <cx.h>

{typ} astsrv();		/* AST service routine */

word status;		/* Returned DSW */

status = srex(astsrv);

[end]

.hl 1 ^&Specify Receive-by-Reference AST\&

********

* SRRA *

********

File name:	srra.mac

Usage

_#include <cx.h>

{typ} astsrv();		/* AST service function */

word status;		/* Returned DSW */

status = srra(astsrv);

Notes

Call  with NULL pointer to turn off receive by reference

AST's.

[end]

.hl 1 ^&Set System Time\&

********

* STIM *

********

File name:	stim.mac

Usage

_#include <cx.h>

_#include <timbuf.h>	/* Time param's struct */

struct timbuf newtim;	/* New time */

struct timbuf oldtim;	/* Old time */

word status;		/* Returned DSW */

status = stim(_&newtim, _&oldtim);

Notes

Second parameter may be zero if no old time needed.

[end]

.hl 1 ^&Stop on Logical-OR of Event FLags\&

********

* STLO *

********

File name:	stlo.mac

Usage

_#include <cx.h>

word msk;		/* 16 bit mask word */

word status;		/* Returned DSW */

status = stlo0(msk);	/* Stop on flags 1-16 */

status = stlo1(msk);	/* Stop on flags 17-32 */

status = stlo2(msk);	/* Stop on flags 33-48 */

status = stlo3(msk);	/* Stop on flags 49-64 */

status = stlo4(msk);	/* Stop on flags 65-80 */

status = stlo5(msk);	/* Stop on flags 81-96 */

Notes

DEC chose not to make the group parameter a variable. It

is fixed at assembly time regardless of the form of  the

macro!!  Soooo,  there  are 6 entry points for stlo, one

for each group of event flags.

[end]

.hl 1 ^&Stop Current Task\&

********

* STOP *

********

File name:	stop.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = stop();

[end]

.hl 1 ^&Stop on Single Event Flag\&

********

* STSE *

********

File name:	stse.mac

Usage

_#include <cx.h>

word eflag;		/* Event flag number */

word status;		/* Returned DSW */

status = stse(eflag);

[end]

.hl 1 ^&Unmap Address Window\&

********

* UMAP *

********

File name:	umap.mac

Usage

_#include <cx.h>

_#include <wdbdf.h>

struct wdb winblk;	/* WDB */

word status;		/* Returned DSW */

status = umap(_&winblk);

[end]

.hl 1 ^&Unstop Task\&

********

* USTP *

********

File name:	ustp.mac

Usage

_#include <cx.h>

rad50 tsknam[2];	/* RAD50 task name */

word status;		/* Returned DSW */

status = ustp(tsknam);

[end]

.hl 1 ^&Wait for Significant Event\&

********

* WSIG *

********

File name:	wsig.mac

Usage

_#include <cx.h>

word status;		/* Returned DSW */

status = wsig();

[end]

.hl 1 ^&Wait for Logical-OR of Event Flags\&

********

* WTLO *

********

File name:	wtlo.mac

Usage

_#include <cx.h>

word msk;		/* 16 bit mask word */

word status;		/* Returned DSW */

status = wtlo0(grp, msk); /* Wait on flags  1-16 */

status = wtlo1(grp, msk); /* Wait on flags 17-32 */

status = wtlo2(grp, msk); /* Wait on flags 33-48 */

status = wtlo3(grp, msk); /* Wait on flags 49-64 */

status = wtlo4(grp, msk); /* Wait on flags 65-80 */

status = wtlo5(grp, msk); /* Wait on flags 81-96 */

Notes

DEC chose not to make the grp parameter a  variable.  It

is  set  at  assembly time regardless of the form of the

macro used. Therefore, there are 6 separate entries  for

wtlo, one per group.

[end]

.hl 1 ^&Wait for Single Event Flag\&

********

* WTSE *

********

File name:	wtse.mac

Usage

_#include <cx.h>

word eflag;		/* Event flag number */

word status;		/* Returned DSW */

status = wtse(eflag);

[end]

.hl 1 ^&Profile support entry module (with trace) for ast's\&

*********

* trace *

*********

File name:	apcsv.mac

Usage

_#include <stdio.h>

extern	FILE	*$$flow;

$$flow = fopen("trace.out", "w");

/* or $$flow = stderr; to trace to the console	*/

_...

$$flow = NULL;		/* Turn off flow trace	*/

##Internal

jsr	r5,pcsv$

_.word	name		; Pointer to name

Where name has the structure:

name:	.word	0	; Counter

.asciz	/name/	; function name

Description

This  module  is  called  whenever  a  function that was

compiled with the profile option is executed.  It checks

that the stack will remain  above  600  octal  when  the

function  executes  and  optionally prints a flow-trace.

It is a modified version of the pcsv() function  in  the

DECUS C library which saves R0 so that C routines may be

used as RSX-11 AST service routines.

For  more information on C AST service routines, see the

astset() documentation and the AST usage information  in

the CX library manual.

If  $$flow  is  set  to  a file descriptor, the function

name, location the function  is  called  from,  and  the

calling  environment,  are  printed  on  each  call to a

function compiled with the profile option.

The output format is:

function__name<TAB>caller<TAB>environment

Where function__name is the name of the  function  being

called,  caller  is  the  address of the calling program

(actually, the return address), and environment  is  the

R5  argument  pointer  (for  local addressing within the

routine being called). The information will  be  written

to $$flow with a newline before and after.

If  profiling has been enabled and the program exits via

error() or via an  operating-system  trap  (such  as  an

illegal   memory   reference),   a   register  dump  and

subroutine trace will be written to stderr.

See the description of calltrace() for details.

Internal

See acsv$ for details of the calling environment.

When this routine is entered for the first time, it sets

up to intercept synchronous system traps, and loads  the

error exit address and profile pointer. The trap service

routines  are in the "traps" module. See the description

there for more information.

Bugs

[end]