Computer Corporation of America

Terminal Driver Modifications for RSX-11M V3.2A

01-MAR-80

PAGE 2

1.0  INTRODUCTION

1.1  General Description

The Computer Corporation of America RSX-11M V3.2A Full  Duplex

Terminal Driver modifications allow increased flexability and func-

tionality in the use of a RSX system.

The driver modifications provide the following functions:

1.  RUBOUTS and CONTROL-R are echoed when echo is  suppressed.

This is needed to support the TYMNET network.

2.  BACKSPACE (CONTROL-H) and CONTROL-A are treated  like  RU-

BOUT.

3.  The last character typed by the user is  returned  to  the

unsolicited  input ast routine.  The unmodified driver re-

turns the first character in the typeahead  buffer.   This

is  needed  for  the  execution abort function of COMET to

work properly.  If ESCAPE is  not  the  first  unsolicited

character  typed  the  unmodified driver will not pass the

ESCAPE character to the COMET unsolicited input  AST  rou-

tine.

4.  A new function IO.DIS (function code 3000) has been imple-

mented.   This  function will hang up DZ-11 lines that are

set as REMOTE.

5.  REMOTE lines can be answered at speeds selected  with  the

SET /SPEED command.

6.  An AST routine can be specified for ANSWER  or  HANGUP  of

REMOTE lines.

PAGE 3

2.0  Use of the Modified Terminal Driver

2.1  Hangup/Answer AST support

The RSX-11M V3.2A Terminal Driver was modified to provide sup-

port  of  hangup and answer ASTs.  The selection of this option may

be made at attach time along with specification  of  typeahead  and

optionaly control-C ASTs.  The form of the attach QIO is:

QIO$    IO.ATA[!TF.NOT],....,<ast,[parameter2][,ast2][,ast3]>

Where:

1.  ast is the unsolicited input AST routine address.

2.  parameter2 is a line identification parameter  which  will

be returned to the AST Routine when an AST occurs.

3.  ast2 is the control-C AST routine Address.

4.  ast3 is the answer/hangup AST routine address.

The optional answer/hangup ast routine address is the  addition  to

the attach directive syntax.  This word must be zero in tasks which

are attaching a terminal with typeahead but  without  answer/hangup

ASTs.

When the AST routine is called the  top  word  of  the  task's

stack  contains a parameter.  The high order byte is the line iden-

tification byte.  The low byte contains  a  zero  if  the  AST  was

caused by a hangup and a one if the AST was caused by answering the

line.

PAGE 4

2.2  Disconnect function

The Terminal driver was modified to provide a disconnect func-

tion.   This  function  allows  a task attached to a remote line to

hang up the line.  The form of this function is:

IO.DIS=3000

.

.

.

QIO$       IO.DIS,....

This routine will currently function with  the  DZ-11  line  multi-

plexer.  For other devices this function will have no effect.

Note that the function code must be assigned  the  value  3000

explicitly since 3000 is not the default value of IO.DIS.

PAGE 5

3.0  Internal Driver Description

3.1  Hangup/Answer AST support

The hangup of a line is detected in the RSX-11M  Version  3.2A

Terminal  Driver  module  TTMOD routine MHUP.  This routine handles

the processing of a disconnect of all REMOTE lines, independent  of

their  physical  type.   The routine FPAST1 is then called to issue

the AST to the task attatched to the terminal.  The answer  AST  is

detected similarly in the routine MANS (located in TTMOD).

A new AST queueing routine FPAST1, in the Terminal Driver  Mo-

dule  TTFP, allocates a block of pool for the AST block.  The block

is filled with the required parameters for  automatic  deletion  on

dequeueing.   The  upper  byte  of the parameter word is set to the

parameter specified on attach and the low order byte is set to zero

for hangup or one for answer.  The AST Block is then queued for the

task.

The AST routine address specified in the attach  statement  is

stored  in  the Terminal Driver AST Block.  The Terminal Driver AST

Block is a multipurpose data structure created when a  terminal  is

attached  with typeahead AST support.  The first section is the AST

control block which is queued to  the  task  for  each  unsolicited

character.   The  second section of the block is a storage area for

AST information.  This information includes the parameter byte, the

unsolicited  input  AST routine address, and the control-C AST rou-

tine address.  One word was added to the end of this block to  con-

tain the answer/hangup AST routine address.

PAGE 6

3.2  Disconnect

The Disconnect function is implimented by calling the  correct

device dependent hangup routine.  The only device dependent routine

currently implimented is the DZ-11 routine (YZMOFF).

3.3  Unsolicited Input Modifications

The Unsolicited Input AST routine was  modified  so  that  the

last character entered into the typeahead buffer is returned to the

AST routine.  This change is needed when the notification  subfunc-

tion  bit  is  specified  on  the attach.  The original unsolicited

character AST routine will always return the first character in the

typeahead buffer.  To support the execution abort function in COMET

it is necessary to pass the most recent character to COMET.

A new routine GLCTAB was added to module TTICH to get the last

character  from  the  buffer.  The AST queueing routine in TTFP was

modified to call GLCTAB inplace of GCTAB the routine which will re-

turn the first character in the typeahead buffer.

PAGE 7

4.0  Terminal Driver Modification procedure.

The driver changes require assembling the new driver  modules,

taskbuilding the driver, and using VMR to unload the old driver and

load the new driver.

In order for the IO.DIS function code to work the driver data-

base  must be modidfied as well.  The driver database is located in

the RSX-11M executive module SYSTB.  SYSTB contains a word for each

device specifying the legal function codes.  The function code used

for the IO.DIS function must be added to the legal Terminal  Driver

functions.   The procedure for making these changes requires apply-

ing the correction file to the SYSTB module with SLP,  reassembling

the  module,  taskbuilding the executive and configuring the system

with VMR.  If the patches are applyed to the terminal driver  with-

out  modifying the driver database the IO.DIS function will have no

effect but the other modifications will function properly.

4.1  Building the modified RSX system.

The UIC [11,40] should contain the  distribution  sources  for

the terminal driver the correction files, and command files used to

modify the Terminal Driver.

The indirect command file [11,40]SYSTTMOD.CMD will  apply  the

modifications  to the Terminal Driver and optionaly to the RSX Exe-

cutive.  See TTDRVMOD.DOC for details of the patching process.