RFILSYS(4)          Xinu Programmer's Manual           RFILSYS(4)

NAME

rfilsys - remote file system pseudo-device driver (types

rfm, rf)

SYNOPSIS

#include <rfile.h>

#include <fserver.h>

read(device buffer, length)

write(device, buffer, length)

open(device, name, mode)

close(device)

control(device, function, arg1, arg2)

init(device)

seek(device,offset)

DESCRIPTION

The _#r_#f_#m and _#r_#f pseudo-device drivers work as a pair to pro-

vide access to remote files using a Xinu remote file server

across an Internet.  There is one master remote file

pseudo-device (type _#r_#f_#m) for a given remote server.  When

users open the master remote file pseudo-device, they pass

it the name of a specific file and the access mode for that

file.  The call to OPEN(2) returns the device descriptor of

a remote file pseudo-device (type _#r_#f) connected to the named

file.

Once opened, a user calls READ(2), WRITE(2), GETC(2), or

PUTC(2), to transfer data between the user program and the

remote file; they call SEEK(2) to position the file pointer.

The user calls CLOSE(2) to disconnect from the file.

Finally, users call CONTROL(2) on the master pseudo-device

to test file access protections, remove files, or change

file names.

The standard Xinu device name for the remote file master

pseudo-device is _#R_#F_#I_#L_#S_#Y_#S; the standard Xinu device name for

individual remote file pseudo-devices is _#R_#F_#I_#L_#E.  The opera-

tions on these devices are defined by:

open(device, name, mode)

Opens a connection to a remote file given the file name

and access mode, and returns the device descriptor used

to access the file.  See ACCESS(2) for valid file

modes.

read(device buffer, length)

Transfers up to  _#l_#e_#n_#g_#t_#h bytes of data from a file to

the user's _#b_#u_#f_#f_#e_#r, and returns the number of bytes

found or _#E_#O_#F if no more data remains in the file.

write(device, buffer, length)

Version 6b               Printed 1/12/87                        1

RFILSYS(4)          Xinu Programmer's Manual           RFILSYS(4)

Writes _#l_#e_#n_#g_#t_#h bytes of data to a file from the user's

_#b_#u_#f_#f_#e_#r.  File length extends automatically if needed.

close(device)

Disconnect from a file, leaving it on secondary

storage.

control(device, function, arg1, arg2)

Handles file manipulation other than data transfer.

The possible functions are:

FLACCESS - test access (_#a_#r_#g_#1 is mode string).

FLREMOVE - remove file named by _#a_#r_#g_#1.

FLRENAME - rename file named by _#a_#r_#g_#1 to _#a_#r_#g_#2

FLCLEAR - clear the remote file datagram port

init(device)

When called with an _#r_#f device, initializes the pseudo-

device data structures at system startup.  Initializing

the master pseudo-device has no effect.

seek(device,offset)

Positions the file to _#o_#f_#f_#s_#e_#t bytes from the beginning.

Version 6b               Printed 1/12/87                        2