Notice

The information in this document is subject to change in order to improve the reliability, design, or function of this product without prior notice and does not represent a commitment on the part of this company.

In no event will the manufacturer be liable for direct, indirect, special, or consequential damages arising out of the use the product or documentation, even if advised of the possibility of such damages.

No part of this reference manual may be reproduced or transmitted in any form or by any means without the prior written permission of this company.

Trademark Acknowledgments

Microsoft, MS, and MS-DOS are registered trademarks, and Windows is a trademark of Microsoft Corporation. Rockwell is a trademark of Rockwell Semiconductor systems. IBM PC is a registered trademark and PC/XT, PC/AT are registered trademarks of International Business Machines Corporation. All other trademarks and registered trademarks are property of their respective holders and are hereby acknowledged.

Information in this manual is subject to change without notice.

Version 1.1

FCC Compliance Statement

Certified to comply with the limits for a Class B computing device according to Subpart J or Part 15 of  FCC rules. See instructions if interference to radio reception is suspected.

FCC WARNING

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation.

This device complies with Part 15 of FCC Rules. Operation is subject to the following two conditions: (1) This device may not cause harmful interference and, (2) this device must accept any interference received, including interference that may cause undesired operation.

However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful  interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

*	Reorient or relocate the receiving antenna.

*	Increase the separation between the equipment and receiver.

*	Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

*	Consult the dealer or an experienced radio/TV technician for help.

In order for an installation of this product to maintain compliance with the limits for a Class B device, shielded cables must be used for the connection of any devices external to this product.

FCC Part 68

This equipment complies with Part 68 of the FCC rules. On the base of this unit is a label that contains, among other information, the FCC registration number and Ringer Equivalence Number (REN) for this equipment. If requested, this information must be given to your telephone company.

The REN is used to determine the quantity of devices you may connect to your telephone line and still have all of those devices ring when your number is called. In most, but not all areas, the RENs of all devices should not exceed five(5.0). To be certain of the number of devices you may connect to your line, as determined by the total RENs, you should call your local telephone company to determine the maximum RENs for your calling area.

If the telephone company suspects a problem with your telephone line is related to an add-on electronic device, such as your modem, they have the right to temporarily suspend your service. If is your responsibility to remove from the telephone line any malfunctioning electronic communications equipment to avoid damage to the telephone system.

If your equipment causes harm to the telephone network, the telephone company may discontinue your service temporarily. If possible, they notify you in advance. But if advance notice is not practical, you will be notified as soon as possible. You will be informed of your right to file a complaint with the FCC, Your telephone company may make changes to its facilities, equipment, operations, or procedures that could affect the proper functioning of your equipment. If they do, you will be notified in advance to give you an opportunity to maintain uninterrupted telephone service.

The telephone company may ask that you disconnect this equipment from the network until the problem has been corrected or until you are sure that the equipment is not malfunctioning

This equipment may not be used on coin service provided by the telephone company. Connection to party lines is subjected to state tariffs.

If you experience trouble with this telephone equipment, please contact your place of purchase for information on obtaining service or repairs.

Hardware/Software Contents

1.

Introduction	1

1.1	Product Overview	1

1.1.1	External Front Panel	1

1.1.2	External Rear Panel	1

1.1.3	Internal Bracket	2

1.2	Product Features	2

2.	Installations	3

2.1	Installation of External Modem	3

2.2	Installation of Internal Modem	4

2.2.1	Jumper Setting	4

2.2.2	Installing Internal Modem Card Procedure	5

2.3	Installation of Windows 95	5

2.3.1	With Plug and Play Function User	5

2.3.2	Without Plug and Play Function User	5

2.4	Installation of Communication Software	6

3.	Commands	7

3.1	Note When Typing Commands	7

3.2	AT Commands	7

3.3	Voice Command Summary	15

3.4	Using AudioSpan/DSVD Function	16

3.4.1	AudioSpan	17

3.4.2	DSVD	18

3.5	Result Codes	20

3.6	S-Register Definitions	23

4.	Appendix	28

4.1	Troubleshooting	28

4.2	What is K56flex?	31

Web Excel Fax/Modem

1

# Introduction

The Web Excel Fax/Modem can connect your computer to INTERNET, and all kinds of BBS, and other popular Fax/Modems. This manual describes the features, procedures of installations, components and AT command set.. etc. of this modem.

## 1.1  Product Overview

There are eight indicators lights and five connectors on the front and back sides of this modem. Following is the function description of these lights and connectors:

### 1.1.1  External Front Panel

| PW | : Power on indicator. | AA | : Auto Answer indicator. |
| --- | --- | --- | --- |
| MR | : Modem Ready indicator. | CD | : Carrier Detect indicator. |
| DTR | : DTR signal indicator. | TD | : Transmitted Data indicator. |
| HS | : High baudrate Speed indicator. | RD | : Received Data indicator. |
| MIC | : Microphone Jack. | SPK | : Speaker Jack. |

### 1.1.2  External Rear Panel

| PHONE | : Phone jack. |
| --- | --- |
| LINE | : Telephone line jack. |
| RS232 | : RS232 cable socket. |
| AC-IN | : AC adapter input |
| ON/OFF | : Power switch. |

Web Excel Fax/Modem	Section 1- Introduction

### 1.1.3  Internal Bracket:

| LINE | : Telephone line jack. |
| --- | --- |
| PHONE | : Phone jack. |
| MIC | : Microphone jack. |
| SPK | : Speaker jack. |

## 1.2  Product Features

This Web Excel Fax/Modem supports the following communication standards:

### Data:

· ITU-T  V.34	    · ITU-T  V.32bis

· ITU-T  V.32	    · ITU-T  V.22bis

· ITU-T  V.22	    · Bell 211A

· Bell 103	    · V.42bis__Data compression

· V.42__Error correction	    · MNP 5__Data compression

· MNP2-4__Error correction

· K56flex__56K bps Down stream only

NOTE:  ITU-T was formerly known as CCITT.

### Fax:

· V.17(14400 bps FAX)	     · V.29(9600 bps FAX)

· V.27ter(4800 bps FAX)

### Voice:

· Voice/Audio mode	     · Full-Duplex speakerphone

· AudioSpan (SAVD)	     · DSVD(optional)

· Voice View 	     · Caller ID (optional)

Web Excel Fax/Modem

2

# Installations

This chapter will describe the detailed steps of how to install and power on your fax modem. Do not power on your modem before finishing the following installation.

## 2.1  Installation of External Modem

If your modem is external one, please refer to following procedure:

1.	Plug the male end of the RS-232 cable into the connector marked RS-232 on the back of the modem.

2.	Plug the other end of this cable into the serial port on the back of your   computer.

3.	Be sure your modem is Power off. Plug the power cable into the AC-IN connector on the back of the modem.

4.	Plug the power adapter into a wall outlet.

5.	Plug one end of the phone cable into the modem’s LINE jack. Plug the other end into the phone outlet.

6.	To use telephone and the modem on the same line, plug one end of the optional phone cable into the PHONE jack on the modem; plug the other end into the phone. Lift the telephone handset and listen for a DIALTONE to check the connection.

7.	Turn your modem on. The PW, MR, HS lights should light up.

8.	Turn your computer on.

Web Excel Fax/Modem	Section 2- Installations

## 2.2  Installation of Internal Modem

If your modem is internal fax modem card, please refer to the following procedure.

### 2.2.1  Jumper Setting

Before installing your internal modem card, please make sure your jumper setting is correct. The jumpers default setting of your modem is at COM 4 and IRQ3. To change the setting on your modem, lift the black plastic pieces and place them on the contacts to match the desired settings. Reboot your computer after reinstalling the modem with the changed jumpers.

To change the COM port and IRQ, the jumpers should be set as follows:

| PORT | IRQ3 | IRQ4 | IRQ5 | IRQ7 | IRQ9 | Base Address |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COM1 | O | S | O | O | O | 03F8 | IRQ4 |
| COM2 | S | O | O | O | O | 02F8 | IRQ3 |
| COM3 | O | S | O | O | O | 03E8 | IRQ4 |
| COM4 | S | O | O | O | O | 02E8 | IRQ3 |

S: Select      O: Not Select

PnP or COM port  select table :

| Jumper | PnP | COM PORT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | COM1 |  | COM2 |  | COM3 |  | COM4 |  |
|  |  | 2, 4 |  | 2, 6 |  | 1, 4 |  | 1, 6 |  |
| JP1 | OPEN | IRQ |  |  |  |  |  |  |  |
|  |  | IRQ 3 | IRQ 4 |  | IRQ 5 |  | IRQ 7 |  | IRQ 8 |
|  |  | 5 | 3 |  | 7 |  | 8 |  | 9 |
| JP2 | 1-2 | 2-3 |  |  |  |  |  |  |  |
| JP3 | 1-2 | 2-3 |  |  |  |  |  |  |  |
| JP4 | 1-2 | 2-3 |  |  |  |  |  |  |  |

Web Excel Fax/Modem	Section 2- Installations

### 2.2.2  Installing Internal Modem Card Procedure

1.	Turn off your computer. Carefully remove the cover form the computer and select a slot for your modem.

2.	Determine how many serial ports are built into your computer. The internal modem COM port setting defaults is in COM 4, so do not conflict with COM port existed in your computer.

3.	Carefully slide the internal modem into the slot.

4.	Replace the slot cover screw to secure the modem and replace the computer’s cover.

5.	Connect the phone cable from modem’s LINE connector to the phone outlet.

6.	You can also connect your telephone to the modem’s PHONE jack optionally.

7.	Turn on your computer.

## 2.3  Installation of Windows 95

### 2.3.1  With Plug and Play Function User

1.	Set jumpers for PnP mode. Turn on the PLUG AND PLAY feature on your system BIOS for initializing the PnP device.

2.	The “Rockwell V.34 Plug-Play Modem” message will be shown on your screen when you boot up Win95 with Rockwell PnP internal modem in the first time.

3.	Select the option. Driver from disk provided by hardware manufacturer.

4.	Put your driver CD title into CD-ROM X: and find the X:\MODEM\ ROCKWELL\R6749\WIN95\PNP\ROCK_15P.INF file then click “OK”¡CThe computer will as PnP ID setup.

### 2.3.2  Without Plug and Play Function User

1.	Start the Windows 95 and process the following steps:

2.	Click “My computer” Folder.

3.	Click “Control Panel” Folder.

4.	Select “Modems” icon. The “Install New Modem” utility will pop-up.

Click “Next”.

Web Excel Fax/Modem	Section 2- Installations

5.	The “Add New hardware Wizard” utility will pop-up.

Select “Next”. Computer will ask you to detect new hardware.

Select “Next” for auto detect.

After finish all the hardware detection select “Finish”.

6.	The “Install New Modem” utility will pop-up again.

Mark “Don’t detect my modem, I will select it from a list”.

Select “Next”.

Select “Have Disk” and Insert the distribution CD title into CD-ROM  X: and find the X:\MODEM\ROCKWELL\R6749\WIN95\AULMDM.INF file then select “OK”¡C

7.	The screen will show the listing as following items to be selected. Select the modem model as you have.

33600bps External Fax Modem

33600bps External Fax Voice Modem

33600bps Internal Fax Modem

33600bps Internal Fax Voice Modem

for Internal Fax Modem

Select “33600bps Internal Fax Voice Modem”, then,

Select “Communication Port(COM4)”.

Select “Next”.

Select “Finish”.

for External Fax Modem

Select “33600bps External Voice Modem”, then,

Select “Communication Port (COM2)”.

Select “Next”.

Select “Finish”.

8. The modem setup is completed now.

## 2.4  Installation of Communication Software

For CD Utility Drivers you can start software installation from X:\MODEM\ ROCKWELL\R6749\SUPERVOC\SV\PICSHELL.EXE.

(X: indicate your CD-ROM drive.)

Web Excel Fax/Modem

3

# Commands

Most people use the communication software programs to tell modem what they want the modem to do.Therefore, you may not use the commands in this chapter.

However, if you prefer to communicate with your modem more directly, you can type the following commands. Please note that when your typing appears on the screen, your modem is in a “terminal mode”. this chapter will be helpful if you like to work in a terminal mode.

## 3.1  Note When Typing Commands

	Use the BACKSPACE key to delete typing errors.

	Every command (except A/ and +++) must begin with the AT or at prefix and be entered by pressing the ENTER key. For example, to execute the V command, you would type ATV and press the ENTER key.

	When you see an n, replace the n with one of the letter or numeric options listed for that command. For example, for the En command, you might type ATE1.

	If a command has numeric options and you don’t include an number, zero is assumed. For example, if you type ATB to indicate a Bn command to be ATB0.

	All defaults are based on the &F Hardware Flow Control template load in NVRAM when the modem is shipped.

## 3.2  AT Commands

A/	Re-executes the last issued command. Used mainly to redial.

A	Go off-hook and attempt to answer a call.

Web Excel Fax/Modem	Section 3- Commands

AT?	Read Selected S-Register. This command reads and displays the           selected S-Register. An S-Register can be selected by using the ATSn           command.

Bn	Bell/ITU-T answer sequence

B0	Select V.22 connection at 1200bps.

B1	Select Bell 212A connection at 1200bps.

Cn	Carrier Control

C1	return OK message.

Dn	Dial

0-9	DTMF digits 0 to 9.

*	The ‘Star’ digit (tone dialing only)

#	The ‘gate’ digit (tone dialing only).

A-D	DTMF digits A,B,C, and D. Some countries may prohibit sending of  these digits during dialing.

L	Re-dial last number: the modem will re-dial the last valid telephone number. The L must be immediately after the D with all the following characters ignored.

P	Pulse (rotary) dial.

T	Tone dial.

R	This command will be accepted, but not acted on.

S=n	Dial the number stored in the directory (n=0 to 3). (See &Z.)

!	Flashes the switch hook.

W	Wait for second dial tone (X3 or higher); linked to S6 register.

@	Dials, waits for quiet answer, and continues (X3 or higher).

,	Dial pause: the modem will pause for a time specified by S8 before dialing the digits following “,”.

;	Return to command mode after dialing.

^	Toggles calling tone enable/disable.

En	Command Echo

E0	disables command echo.

E1	Enables command echo.

Web Excel Fax/Modem	Section 3- Commands

Hn	Disconnect (Hang-up)

H0	Hang up. (goes on-hook)

H1	Goes off-hook.

In	Identification

I0	Reports product code.

I1	Reports “OK”

I2	Reports “OK” or “ERROR”.

I3	Reports firmware revision.

I4	Reports OEM defined identifier string.

I6	Reports modem data pump model and internal code revision.

Ln	Speaker Volume

L0	Speaker off

L1	Set low speaker volume.

L2	Set medium volume.

L3	Set high volume.

Mn	Speaker Control

M0	Speaker is always off.

M1	Speaker ON until CONNECT.

M2	Speaker is always on.

M3	Speaker off during dialing and receiving carrier and turn speaker on during answering.

Nn	Automode Enable

N0	Turn off automode detection.

N1	Turn on automode detection.

On	Return to On-Line Data Mode

O0	Go on-line.

O1	Go on-line and retrain.

P	Set Pulse Dial(for phone lines that don’t support touch-tone dialing)

Qn	Quiet Results Codes Control.

Web Excel Fax/Modem	Section 3- Commands

Q0	Allow result codes to DTE .

Q1	Inhibit Result codes to DTE.

Sr=n	Sets register r to n

Sn?	Display contents of S-Register n

T 	Set Tone Dial

Vn	Result Code Form

V0	Numeric codes.

V1	Verbal codes.

W	Connect Message Control

W0	Report DTE speed in EC mode.

W1	Report line speed, EC protocol and DTE speed.

W2	Report DCE speed in EC mode.

Xn	Extended Result Codes

X0	Report basic call progress result code, i.e., OK, CONNECT, RING, NO CARRIER, NO ANSWER and ERROR.

X1	Report basic call progress result codes and connections speed OK,  CONNECT, RING, NO CARRIER, NO ANSWER, CONNECT XXXX  and ERROR.

X2	Report basic call progress result codes and connections speeds, i.e.,OK,       CONNECT, RING, NO CARRIER, NO ANSWER, CONNECT XXXX and ERROR.

X3	Report basic call progress result codes and connections rate, i.e.,  OK,    CONNECT, RING, NO CARRIER, NO ANSWER, CONNECT XXXX  BUSY and ERROR.

X4	Report all call progress result codes and connections rate, i.e.,  OK, CONNECT, RING, NO CARRIER, NO ANSWER, CONNECT XXXX BUSY, ERROR and NO DIAL TONE.

Yn	Long Space Disconnect

Y0	Disables long space disconnect before on-hook.

Y1	Enable long space disconnect before on-hook.

Web Excel Fax/Modem	Section 3- Commands

Zn	Soft Reset and Restore Profile

Z0	Restore stored profile 0 after warm reset.

Z1	Restore stored profile 1 after warm reset.

&Cn	Controls Carrier Detect (CD) signal

&C0	CD override.

&C1	Normal CD operations.

&Dn	DTR Option

&D0	Ignore an on-to-off transition of DTR.

&D1	Switch to on-line command mode without disconnection.

&D2	Normal  DTR operations.

&D3	Modem re-initialized. &Y determines which profile is loaded.

&Fn	Load Factory Configuration (Profile)

&F0	Restore factory configuration 0.

&F1	Restore factory configuration 1.

&Gn	Select Guard Tone

&G0	Disables guard tone. (for USA area)

&G1	Disables guard tone.

&G2	Selects 1800Hz guard tone.

&Kn	Flow Control

&K0	Disables flow control.

&K3	Enables RTS/CTS flow control. (Default for data modem modes)

&K4	Enables XON/XOFF flow control.

&K5	Enables transparent XON/XOFF flow control.

&K6	Enables both RTS/CTS and XON/XOFF flow control.(Default for fax modem modes.)

&Mn	Asynchronous/Synchronous Mode Selection

&M0	Selects direct asynchronous operation.

&M1	Selects synchronous connect mode with asynchronous off-line command mode.

Web Excel Fax/Modem	Section 3- Commands

&M2	Selects synchronous connect mode with asynchronous off-line command mode.

&M3	Selects synchronous connect mode.

&Qn	Sync/Async Mode

&Q0	Selects direct asynchronous operation.

&Q4	Select Hayes AutoSync mode. (optional)

&Q5	Modem negotiates an error-corrected link.

&Q6	Selects asynchronous operation in normal mode.

&Rn	RTS/CTS Option

&R0	CTS tracks RTS(async) or acts per V.25 (sync).

&R1	CTS is always active.

&Sn	DSR Override

&S0	DSR is always active.

&S1	DSR acts per V.25

&V	Display Current Configuration

&V1	Display Last Connection Statistics

&Wn	Store Current configuration

&W0	Store the current configuration as profile 0.

&W1	Store the current configuration as profile 1.

&Yn	Designate a Default Reset Profile

&Y0	The modem will use profile 0.

&Y1	The modem will use profile 1.

&Zn=x	Store telephone number

&Zn=x	n=0 to 3 and string.

%C	Enable /Disable Data Compression

%C0	Disables data compression.

%C1	Enables MNP 5 data compression.

%C2	Enables V.42 bis data compression.

%C3	Enables both V.42 bis and MNP 5 data compression.

Web Excel Fax/Modem	Section 3- Commands

%En	Enable/Disable Line Quality Monitor and Auto-Retrain or Fallback/Fall  Forward

%E0	Disable line quality monitor and auto-retrain.

%E1	Enable line quality monitor and auto-retrain.

%E2	Enable line quality monitor and fallback/fall forward.

%L	Line Signal Level

Returns a value which indicates the received signal level. The value returned is direct indication of the receive level. For example, 009=-9dBm, 043=-43dBm, and so on.

%Q	Line Signal Quality

Reports the line signal quality. Returns the higher order byte of the  EQM value. Based on the EQM value, retrain or fallback/fall forward may be initiated if enabled by %E1 or %E2.

Example:

AT%Q

015

\An	Selects Maximum MNP Block Size

\A0	64 characters.

\A1	128 characters.

\A2	192 characters.

\A3	256 characters.

\Bn	Transmit Break to Remote

\B1-\B9	Break length in 100 ms units. (Default=3.)

(Non-error corrected mode only.)

\Nn	Operating Mode

\N0	Selects normal speed buffered mode.

\N2	Selects reliable (error-correction) link mode.

\N3	Selects auto reliable mode.

\N4	Force LAPM error-correction mode.

\N5	Force MNP error-correction mode.

Web Excel Fax/Modem	Section 3- Commands

+MS	Select Modulation

This command selects the modulation, optionally enables or disables automode, and optionally specifies the lowest and highest connection rates using one to four subparameters. The format is:

AT+MS=<mode>,<automode>,<min_rate>, <max_rate>

| Mod | Modulation | Data Rates(bps) |
| --- | --- | --- |
| 0 | V.21 | 300 |
| 1 | V.22 | 1200 |
| 2 | V.22bis | 2400,1200 |
| 3 | V.23 | 1200 |
| 9 | V.32 | 9600,4800 |
| 10 | V.32bis | 14400,12000,9600,7200,4800 |
| 11 | V.34 | 33600,31200,28800,26400,24000,21600,19200,16800,14400,12000,9600,7200,4800,2400 |
| 56 | K56flex | 56000,54000,52000,50000,48000,46000,44000,42000,40000,38000,36000,34000,32000 |
| 64 | Bell 03 | 300 |
| 68 | Bell 212 | 1200 |

NOTE: K56flex for 56,000bps Voice/Fax Modem only

| <automode> | Option Selected |
| --- | --- |
| 0 | Automode disabled |
| 1 | Automode enable |

For example:   AT+MS=11,1,300,33600

where:	11	: select V.34

1	: automode enable

300	: min_rate data speed 300 bps

33,600	: max_rate data speed 33,600 bps

Web Excel Fax/Modem	Section 3- Commands

If you want to select V.34 28800 bps only, please ensure

AT+MS=11,1,28800,28800

-SSE=n	Enable/Disable DSVD (optional)

This command enables or disables DSVD (Digital Simultaneous                      Voice and Data) in modem supporting DSVD. The syntax is AT-SSE=n,where n is number from 0 to 1.

-SSE=0	Disable DSVD.

-SSE=1	Enable DSVD.

-SMS=x,y,z,t	Select AudioSpan/DSVD Mode (Voice modem only)

x :

| x | Mode |
| --- | --- |
| 0 | Data mode |
| 1 | DSVD mode (A modem not supporting DSVD will respond with ERROR) |
| 2 | AudioSpan mode (A modem not supporting AudioSpan will respond with ERROR) |
| 3 | Automatic mode select |

y :	Minimum data speed (4800) with AudioSpan mode.

z :	Maximum data speed (14400) with AudioSpan mode.

t :	Symbol rate (Default: 0. Auto selection)

## 3.3  Voice Command Summary

ATA	Answer in voice/audio mode.

ATD	Dial command in voice/audio mode.

ATH	Hang up in voice/audio mode.

ATZ	Reset from voice/audio mode.

#BDR	Select baud rate (turn off autobaud).

#CID	Enable caller ID detection and select reporting format.

#CLS	Select data, fax, or voice/audio.

Web Excel Fax/Modem	Section 3- Commands

#MDL	Identify model.

#MFR	Identify manufacturer.

#REV	Identify revision level.

#SPK	Speakerphone setting.

#TL	audio output transmit level.

#VBQ	Query buffer size.

#VBS	Bits per sample(ADPCM or PCM).

#VBT	Beep tone timer.

#VCI	Identify compression method(ADPCM).

#VLS	Voice line select(ADPCM or PCM).

#VRA	Ringback goes away timer (originate).

#VRN	Ringback never came timer (originate).

#VRX	Voice receive mode (ADPCM or PCM).

#VSD	Silence deletion tuner (voice receive, ADPCM)

#VSK	Buffer skid setting.

#VSP	Silence detection period(voice receive, ADPCM)

#VSR	Sampling rate selection (ADPCM or PCM).

#VSS	Silence deletion tuner (voice receive, ADPCM).

#VTD	DTMF tone reporting capability.

#VTM	Enable timing mark placement.

#VTS	Generate tone signals.

#VTX	Voice transmit mode(ADPCM or PCM).

## 3.4  Using AudioSpan/DSVD Function

AudioSpan and DSVD are two technologies that allow simultaneous voice and data operation using a Rockwell modem connected to a telephone line. AudioSpan and DSVD implementations differ in the manner that voice and modem data are combined. Voice and data channels are combined in the analog domain for AudioSpan and in the digital domain for DSVD.

Web Excel Fax/Modem	Section 3- Commands

A handset, headset, or microphone/speaker can be used for voice communication during AudioSpan or DSVD.

NOTE: The DTE rate must be equal to or greater than the anticipated DCE connection speed. In general, DTE speed of 38400 bps or higher should be used.

### 3.4.1  AudioSpan

Example 1:  Establish a AudioSpan data connection between two voice modems with AudioSpan function and use handset as audio interface.

(For U.S. and Asia area only)

| Originate Modem |  |  | Answer Modem |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DTE | DCE | DTE |  | DCE |  |  |
| AT&F |  | AT&F |  |  | Reset modem. |  |
|  | OK |  |  | OK |  |  |
| AT-SMS=2 |  | AT-SMS=2 |  |  | Enable AudioSpan with default settings. |  |
|  | OK |  |  | OK |  |  |
| ATDTxxxx |  |  |  |  | Originate modem dials remote modem |  |
|  |  |  |  | RING |  |  |
|  |  | ATA |  |  | Answer DTE responds to RING by answering. |  |
|  | CONNECT xxxx |  |  | CONNECT xxxx | AudioSpan is established and users can pick up handsets to converse. The connect message represents the DTE speed on each side. |  |
| <data> |  | <data> |  |  | Modems exchange data. |  |
|  |  |  |  |  | Users hang up handsets to terminate audio link. |  |
| +++ |  |  |  |  | Prepare for disconnect. |  |
|  | OK |  |  |  |  |  |
| ATH |  |  |  |  |  |  |
|  | NO Carrier |  |  | NO Carrier | Modem disconnect. |  |

Example 2: 	Establish a AudioSpan data connection between two voice modems with AudioSpan function and use MIC/SPK as audio interface.

Web Excel Fax/Modem	Section 3- Commands

| Originate Modem |  |  | Answer Modem |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DTE | DCE | DTE |  | DCE |  |  |
| AT&F-SMS=2 #VLS=6 |  | AT&F-SMS=2 #VLS=6 |  |  | Originate and answer modems are configured for AudioSpan Speaker-phone call. |  |
|  | OK |  |  | OK |  |  |
| ATDT xxxxxx |  |  |  | RING | Originate modem dials remote modem. |  |
|  |  | ATA |  |  | DTE responds to RING by answering |  |
|  | CONNECT XXXX |  |  | CONNECT XXXX | AudioSpan is established and MIC/SPK are used for conversation. |  |
| <data> |  | <data> |  |  | Modems exchange data. |  |
|  |  |  |  | +++ | Prepare for disconnect. |  |
| OK |  |  |  |  |  |  |
| ATH |  |  |  |  |  |  |
|  | NO CARRIER |  |  | NO CARRIER | Modem disconnect. |  |

### 3.4.2  DSVD

Example 1:	Establish a DSVD data connection between two voice modems with DSVD function and use handset as audio interface.

(For U.S. and Asia area only)

| Originate Modem |  | Answer Modem |  | Comments |
| --- | --- | --- | --- | --- |
| DTE | DCE | DTE | DCE |  |
| AT&F |  | AT&F |  | Reset modem. |
|  | OK |  | OK |  |
| AT-SSE=1 |  | AT-SSE=1 |  | Enable DSVD with default settings. |
|  | OK |  | OK |  |
| ATDTxxxx |  |  |  | Originate modem dials remote modem |
|  |  |  | RING |  |

Web Excel Fax/Modem	Section 3- Commands

|  |  | ATA |  | Answer DTE responds to RING by answering. |
| --- | --- | --- | --- | --- |
|  | CONNECT xxxx |  | CONNECT xxxx | DSVD is established and users can pick up handsets to converse. The connect message represents the DTE speed on each side. |
| <data> |  | <data> |  | Modems exchange data. |
|  |  |  |  | Users hang up handsets to terminate audio link. |
| +++ |  |  |  | Prepare for disconnect. |
|  | OK |  |  |  |
| ATH |  |  |  |  |
|  | NO CARRIER |  | NO CARRIER | Modem disconnect. |

Example 2: 	Establish a DSVD data connection between two voice modems with DSVD function and use MIC/SPK as audio interface.

| Originate Modem |  |  | Answer Modem |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DTE | DCE | DTE |  | DCE |  |  |
| AT&F-SSE=1 #VLS=6 |  | AT&F-SSE=1 #VLS=6 |  |  | Originate and answer modems are configured for DSVD Speaker-phone call. |  |
|  | OK |  |  | OK |  |  |
| ATDT xxxxxx |  |  |  | RING |  |  |
|  |  | ATA |  |  |  |  |
|  | CONNECT XXXX |  |  | CONNECT XXXX | DSVD is established and MIC/SPK are used for conversation. |  |
| <data> |  | <data> |  |  | Modems exchange data. |  |
| +++ |  |  |  |  | Prepare for disconnect. |  |
|  |  |  |  | OK |  |  |
| ATH |  |  |  |  |  |  |
|  | NO CARRIER |  |  | NO CARRIER | Modem disconnect. |  |

Web Excel Fax/Modem	Section 3- Commands

## 3.5  Result Codes

| Numeric | Verbose | Description |
| --- | --- | --- |
| 0 | OK | Modem successfully executed at AT command. |
| 1 | CONNECT | A connection is established. |
| 2 | RING | Modem detected an incoming call. |
| 3 | NO CARRIER | Modem lost or could not detect a remote carrier signal with the register S7 time. |
| 4 | ERROR | Modem detected an error in an AT command. |
| 5 | CONNECT 1200 | Connect made at 1200bps. |
| 6 | NO DIAL TONE | Modem did not detect a dial tone after off-hook. |
| 7 | BUSY | Modem detected a busy tone |
| 8 | NO ANSWER | Modem did not detect 5 seconds of silence when using the @ dial modifier in the dial command. |
| 9 | CONNECT 600 | Connection made at 600bps |
| 10 | CONNECT 2400 | Connection made at 2400bps |
| 11 | CONNECT 4800 | Connection made at 4800bps |
| 12 | CONNECT 9600 | Connection made at 9600bps |
| 13 | CONNECT 7200 | Connection made at 7200bps |
| 14 | CONNECT 12000 | Connection made at 12000bps |
| 15 | CONNECT 14400 | Connection made at 14400bps |
| 16 | CONNECT 19200 | Connection made at 19200bps |
| 17 | CONNECT 38400 | Connection made at 38400bps |
| 18 | CONNECT 57600 | Connection made at 57600bps |
| 19 | CONNECT 115200 | Connection made at 115200bps |

Web Excel Fax/Modem	Section 3- Commands

| 20 | CONNECT 230400 | Connection made at 230400bps |
| --- | --- | --- |
| 22 | CONNECT 1200TX/ | Connection made at transmit 1200/75/RX receive 75 bps. |
| 23 | CONNECT 75TX/ | Connection made at transmit 75/1200/RX receive 1200 bps. |
| 33 | FAX | Fax modem connection established |
| 35 | DATA | Data modem connection established. |
| 40 | CARRIER 300 | Carrier rate of 300 bps. |
| 44 | CARRIER 1200/75 | Carrier rate of transmit 1200 receive 75 bps. |
| 45 | CARRIER 75T/1200 | Carrier rate of transmit 75 receive 1200 bps. |
| 46 | CARRIER 1200 | Carrier rate of 1200 bps. |
| 47 | CARRIER 2400 | Carrier rate of 2400 bps. |
| 48 | CARRIER 4800 | Carrier rate of 4800 bps. |
| 49 | CARRIER 7200 | Carrier rate of 7200 bps. |
| 50 | CARRIER 9200 | Carrier rate of 9600 bps. |
| 51 | CARRIER 12000 | Carrier rate of 12000 bps. |
| 52 | CARRIER 14400 | Carrier rate of 14400 bps. |
| 53 | CARRIER 16800 | Carrier rate of 16800 bps. |
| 54 | CARRIER 19200 | Carrier rate of 19200 bps. |
| 55 | CARRIER 21600 | Carrier rate of 21600 bps. |
| 56 | CARRIER 24000 | Carrier rate of 24000 bps. |
| 57 | CARRIER 26400 | Carrier rate of 26400 bps. |
| 58 | CARRIER 28800 | Carrier rate of 28800 bps. |
| 59 | CONNECT 16800 | Connection made at 16800 bps. |
| 61 | CONNECT 21600 | Connection made at 21600 bps. |
| 62 | CONNECT 24000 | Connection made at 24000 bps. |
| 63 | CONNECT 26400 | Connection made at 26400 bps. |

Web Excel Fax/Modem	Section 3- Commands

| 64 | CONNECT 28800 | Connection made at 28800 bps. |
| --- | --- | --- |
| 66 | COMPRESSION: CLASS 5 | Data compression MNP CLASS 5 connection established. |
| 67 | COMPRESSION: V.42bis | Data compression V.42bis connection established. |
| 69 | COMPRESSION: NONE | Connection established without data compression. |
| 70 | PROTOCOL: NONE | Connection established without error correction. |
| 77 | PROTOCOL: LAPM | V.42/LAMP error correction established. |
| 78 | CARRIER 31200 | Carrier rate of 31200bps |
| 79 | CARRIER 33600 | Carrier rate of 33600bps |
| 80 | PROTOCOL: ALT | MNP 3-4 error correction connection established. |
| 84 | CONNECT 33600 | Connection made at 33600 bps. |
| 91 | CONNECT 31200 | Connection made at 31200 bps. |
| 150 | CARRIER 32000 | Carrier rate of 32000bps. |
| 151 | CARRIER 34000 | Carrier rate of 34000bps. |
| 152 | CARRIER 36000 | Carrier rate of 36000bps. |
| 153 | CARRIER 38000 | Carrier rate of 38000bps. |
| 154 | CARRIER 40000 | Carrier rate of 40000bps. |
| 155 | CARRIER 42000 | Carrier rate of 42000bps. |
| 156 | CARRIER 44000 | Carrier rate of 44000bps. |
| 157 | CARRIER 46000 | Carrier rate of 46000bps. |
| 158 | CARRIER 48000 | Carrier rate of 48000bps. |
| 159 | CARRIER 50000 | Carrier rate of 50000bps. |
| 160 | CARRIER 52000 | Carrier rate of 52000bps. |

Web Excel Fax/Modem	Section 3- Commands

| 161 | CARRIER 54000 | Carrier rate of 54000bps. |
| --- | --- | --- |
| 162 | CARRIER 56000 | Carrier rate of 56000bps. |
| 165 | CONNECT 32000 | Connection made at 32000 bps. |
| 166 | CONNECT 34000 | Connection made at 34000 bps. |
| 167 | CONNECT 36000 | Connection made at 36000 bps. |
| 168 | CONNECT 38000 | Connection made at 38000 bps. |
| 169 | CONNECT 40000 | Connection made at 40000 bps. |
| 170 | CONNECT 42000 | Connection made at 42000 bps. |
| 171 | CONNECT 44000 | Connection made at 44000 bps. |
| 172 | CONNECT 46000 | Connection made at 46000 bps. |
| 173 | CONNECT 48000 | Connection made at 48000 bps. |
| 174 | CONNECT 50000 | Connection made at 50000 bps. |
| 175 | CONNECT 52000 | Connection made at 52000 bps. |
| 176 | CONNECT 54000 | Connection made at 54000 bps. |
| 177 | CONNECT 56000 | Connection made at 56000 bps. |

## 3.6  S-Register Definitions

S0	Number of Rings to Auto-Answer

The number of ring the modem waits for before it auto answer.

Range:	0-255 (rings)

Default:	0

S1	Ring Counter

Count the number of rings before the modem answers.

Range:	0-255 (rings)

Default:	0

S2	Escape Character

Define the character used for the three-character escape code sequence.

Web Excel Fax/Modem	Section 3- Commands

Range:	0-255

Default:	43 (“+”)

S3	Carriage Return Character

Define the character for carriage return.

Range:	0-127

Default:	13 (Carriage Return)

S4	Line Feed Character

Define the character for line speed

Range:	0-127

Default:	10 (Line Feed)

S5	Backspace character

Define the character for backspace.

Range:	0-255

Default:	8 (Backspace)

S6	Wait Time Before Blind Dialing

The time to pause after off-hook before blind

Range:	0-255

Default:	2 (Country dependent)

S7	Waiting Time Before Carrier Detect

The time to wait for a carrier from the remote modem before hanging

up.

Range:	1-255

Default:	50 (seconds)

S8	Pause Time For Dial Delay

The time to pause for the pause dial modifier, “comma”

Range:	0-255

Default:	2 (seconds)

S9	Carrier Detect Response Time

The time a signal is detected as qualified as a carrier. This timing lets

Web Excel Fax/Modem	Section 3- Commands

your modem ignore spurious signals that are the same frequency as the                       carrier. Higher S9 values reduce the chance of a carrier being detected.

Range:	1-255

Default:	6 (0.6 second)

S10	Lost Carrier To Hang Up Delay

The time the modem waits before hanging up for carrier loss.

Range:	1-255

Default:	14 (1.4 seconds)

S11	DTMF Tone Duration

The time for DTMF tone dialing and the time between the tone spacing.

Range:	50-255

Default:	95 (0.95 second, Country dependent)

S12	Escape Prompt Delay (EPD)

The minimum “quiet” time between the last three-character escape

code and the first character of AT command, which is “a” or “A”.

Range:	0-255

Default:	50 (1 second)

S18	Test Timer

The testing time of the loopback testing.

Range:	0-255

Default:	0 (second)

S25	Delay To DTR

The time the modem ignore DTR before hanging up.

Range:	0-255

Default:	5 (0.05 second)

S30	Disconnect Inactivity Timer

The time allowed for inactivity before the connection is hang up.

Range:	0-255 (seconds)

Default:	0 (disabled)

Web Excel Fax/Modem	Section 3- Commands

S32	XON Character

Sets the value of the XON character.

Range:	0-255

Default:	17

S33	XOFF Character

Sets the value of the XOFF character.

Range:	0-255

Default:	19

S37	Desired Line Connection Speed

0= Attempt auto mode connection

3= Attempt to connect at 300 bps.

4= Attempt to connect at V.22 1200 bps.

6= Attempt to connect at V.22 bis 2400 bps.

7= Attempt to connect at V.32 bis/V.32  4800 bps.

8= Attempt to connect at V.32 bis 7200 bps.

9= Attempt to connect at V.32 bis/V.32  9600 bps.

10= Attempt to connect at V.32 bis 12000 bps.

11= Attempt to connect at V.32 bis 14400 bps.

12= Attempt to connect at V.34.

Default:    0

S38	Delay Before Forced Hang Up

The delay time to hang up after the disconnecting command is received.

Range:	0-255

Default:	20(seconds)

S86	Call Failure Reason Code

0	Normal disconnect, no error occurred.

4	Loss of carrier.

5	V.42 negotiation failed to detect an error-correction modem at the other end.

6	No response to complete negotiation.

Web Excel Fax/Modem	Section 3- Commands

9	The modem could not find a common protocol.

12	Normal disconnect initiated by the remote modem.

13	Remote modem does not respond after 10 re-transmissions of the same message.

14	Protocol violation.

20	Hang up by inactivity time out.

Range:	0,4,5,9,12,13 or 14

Default:	0

S91	PSTN Transmit Level

Sets the transmit level.

Range:	0 to 15

Default:	10 (-10dBm, country dependent)

S95	Extended Result Codes

The bits in this register can be set to override some of the Wn command

options. A bit set to a 1 in this register will enable the corresponding

result code regardless of the Wn setting.

Bit 0	CONNECT result code indicates DCE speed instead of DTE  speed.

Bit 1	Append/ARQ to CONNECT XXXX result code in error correction mode (XXXX=rate).

Bit 2	Enable CARRIER XXXX result code (XXXX=rate)

Bit 3	Enable PROTOCOL XXXX result code (XXXX=protocol identifier).

Bit 4	Reserved.

Bit 5	Enable COMPRESSION result code (XXXX=compression  type).

Web Excel Fax/Modem

4

# Appendix

## 4.1  Troubleshooting

If you experience a problem with your fax/modem, refer to this appendix for suggestions on resolving the problem.

1.	If your modem doesn’t respond to any AT commands.

	Make sure the modem is on. (externals only)

	Make sure you selected the correct COM port and IRQ in your communications software, and /or in your windows Control Panel.

	Make sure the computer is in Terminal mode of your communications software.

	Type in all upper (AT) or lower (at) case.

	There may be a COM port/IRQ conflict. Refer back to Internal Installation. You will need to remove your modem from the PC and change your COM port and IRQ setting to COM port 3 and IRQ 5 or 7. If you change your COM port and IRQ setting, also make the changes in your software and in Windows.

2.	Type a command line and receive an ERROR result code.

	You typed an unacceptable command.

	Your command line contains more than 40 characters.

3.	Do not receive a result code after your fax/modem executes a command line.

	All fax/modem result codes may have been disabled with the ATQ1

Web Excel Fax/Modem	Section 4- Appendix

command. Send the ATQ0 command to enable result codes.

4.	Your fax/modem responds to commands but your computer screen doesn‘t show the characters you type.

	Make sure the command echo is off (ATE1 in effect) to have your fax/modem echo commands.

	Make sure your communications program is configured to echo command characters.

5.	Each character you type appears twice on your computer screen.

	Your computer or communications software has its echo feature is also turned on. If the system you are calling echoes your typed characters, turn off local echo at your computer, terminal, or computer software. If the system you are calling doesn‘t echo type characters, turn off the fax/modem’s echo feature by typing ATE0 (which appears as AATT EE00 on your screen) and pressing the Enter key.

6.	Your fax/modem does not automatically answer calls.

	Make sure your communications software is set up to automatically answer calls.

	Change the value of Register S0 by typing AT S0=n and pressing the Enter key, where n is the number of rings that must occur before the fax/modem auto-answer calls.

7.	You can‘t hear through your fax/modem speaker.

	Use the ATMn command to make sure the speaker is turned on.

	Use the ATLn command to adjust the listening level.

## During data transfer

8.	If your screen display random or garbage characters.

	Set your software to the same word length, parity, and Stop bits as the remote modem.

Web Excel Fax/Modem	Section 4- Appendix

	Make sure that your software and modem are set to the same flow control setting and to either a fixed or variable serial port rate.

	Type the following command to load the template that enable hardware flow control as well as other optimal settings.

AT&F <Enter>

	Disable any Terminate and Stay Resident (TSR) programs running in the background.

9.	If your communications software is reporting many Cyclic Redundancy check (CRC) error and low characters per second (CPS).

	You might have experienced noise on the phone line. Place the call again. The phone company routes calls differently each time your call.

	Type the following command to load the template that enables hardware flow control as well as other optimal settings:

AT&F <enter>

	Lower the serial port rate in your communications software to 38400 bps or 19200 bps.

	Try a different file transfer protocol (do not use Xmodem if other protocols are variable).

	Disable any Terminate and Stay Resident (TSR) programs running in the background, such as screen savers.

10.	If the modem disconnect while on-line.

	Check for loose connections between the standalone modem and the computer.

	Check for loose connections between the modem and the telephone connections.

	Line noise or interference may be interfering with modem signals. Retry the connection by dialing the number again.

Web Excel Fax/Modem	Section 4- Appendix

## 4.2  What is K56flex?

K56flex is a new modem technology that enable Internet connect at blistering rates up to 56 Kbps over standard telephone lines. Bridging the gap between current analog transmission rates and fully digital communication -such as Integrated Services Digital Network (ISDN)- this new technology offers substantially faster speeds without requiring the added investment of installing digital ISDN lines.

This means that graphics-heavy Web pages, sound and video files-the information that usually makes you wait - now race to your computer at nearly twice the speed of current V.34 connects. Only if K56flex modem connected to an Internet Service Provider (ISP) or corporate site using K56flex technology over their digital connection to the network, you can realize the increase in speed.