USAGE

srec2bin <eprom_Kbytes_size> <eprom_start_address> <infile> <outfile>

DESCRIPTION

This program interprets a Motorola S-rec file and generates

a binary image file.  It currently supports type S1 and S9

records.  Checksums are verified.  Address locations are

validated, i.e.,

address  >=  eprom_start_address

and

address  <  eprom_start_address  +  eprom_Kytes_size * 1024

The values can be passed either on the command line or

interactively (when no values are given on the command line).

The values are described as follows:

<eprom_Kbytes_size>

The eprom size has to be specified in factors of 1024. For the

MC68xx family, the range is from 2 to 64 (depending on the

specific chip and the modes used).

<eprom_start_address>

The eprom start address is the chips selects lowest address.

The MC6801 in mode 7, for example, has its lowest ROM address

at $F800.  The value specified then for srec2bin would be f800h

or its decimal equivalent 63488.

<infile>

This is the file containing the Motorola S records to be converted.

<outfile>

Filename of the output containing the binary image.