A Parallel Processing Support Library Based On

**********************************************

Synchronized Aggregate Communication

*************************************

H. G. Dietz, T. M. Chung, and T. I. Mattox

School of Electrical Engineering

Purdue University

West Lafayette, IN 47907-1285

hankd@ecn.purdue.edu

April 1995

Abstract

=========

The concept of "data parallelism" is a pervasive force throughout parallel

processing. Although a certain level of processing-element autonomy

can help performance, the fact is that many parallel algorithms,

applications, and compiler analysis techniques focus on identifying a set

of data objects that can be processed using loosely synchronous

parallelism. Thus, it is not surprising that a large number of

communication libraries support at least a few synchronized aggregate

operations on data. In this paper, we present an overview of the parallel

processing support library for PAPERS clusters.

Unlike most other systems, which construct aggregate communications

by layering message-passing or shared-memory communication, the

PAPERS hardware and software directly implements a model of parallel

execution based on synchronized aggregate communications.

Asynchronous processor operation is fully supported, but asynchronous

communications are not directly supported (although they can be derived

using the PAPERS parallel interrupt mechanism). Thus, PAPERS

provides highly efficient aggregate communications for MIMD, SIMD,

and VLIW execution modes. We demonstrate the effectiveness of this

implementation by presenting detailed benchmarks for a variety of other

libraries supporting dedicated parallel machines and workstation clusters.

This work was supported in part by the Office of Naval Research (ONR)

under grant number N00014-91-J-4013 and by the National Science

Foundation (NSF) under Parallel Infrastructure Grant number

CDA-9015696.

1. Introduction

================

PAPERS, Purdue's Adapter for Parallel Execution and Rapid

Synchronization, is inexpensive hardware designed to provide very low

latency barrier synchronization and aggregate communication operations

to user-level unix processes running on a cluster of personal computers

or workstations. The PAPERS library is designed to provide both:

o Good support for hand-coding of parallel applications and

o An efficient target for optimizing/parallelizing compilers.

Of course, these goals can only be accomplished by avoiding the usual

software interface layering and the associated increases in latency.

Thus, the PAPERS library must solve two fundamental problems. The

first problem is how to avoid the usual ill effects of using a cluster, as we

discuss in Section 1.1. The second, discussed in Section 1.2, is how to

construct a parallel execution model that can provide the appropriate

functionality without sacrificing performance.

1.1. Clustering

++++++++++++++++

The use of clusters of workstations/PCs for parallel computation is an

attractive idea for many reasons. However, there are also many reasons

why such clusters do not make effective parallel computing platforms.

The most obvious problem is that of efficiently transmitting data

between the processing elements (PEs) of the cluster. Traditional

networks, ranging from Ethernet to FDDI and ATM, were all designed

to maximize bandwidth rather than to minimize latency. This is not just

a matter of hardware latency, but also of software latency. Standard

network communication protocols (e.g., BSD sockets) require at least

one system call and process context switch for each operation. This often

adds as much as 100 microseconds to the latency of each operation.

Libraries like PVM, which introduce even more software layers, can add

milliseconds of delay to each operation. Given such high latency, only

very coarse-grain parallelism is effective.

In contrast, everything about PAPERS is designed with the goal of

supporting fine-grain parallelism. Although a PAPERS cluster might

use the exact same personal computers and workstations that a

conventional cluster would employ, there is only one software layer

between the PAPERS hardware and the user code: the PAPERS library.

This library executes as part of the user process, directly accessing the

PAPERS hardware either by executing port I/O instructions or by

volatile references to memory-mapped device registers. There is no

system call required for a typical operation. The PAPERS hardware

provides such low latency that, if we had used a system call for each

PAPERS operation, the average latency would have been between 5 and

50 times longer. In fact, the simplest PAPERS operations are so fast that

they are implemented by in-line code to avoid the overhead of a

subroutine call.

Another interesting implication of the PAPERS mechanism and

software interface is that the execution time of each operation is very

consistent and predictable. This property makes it possible for compiler

analysis to more precisely estimate the performance of alternative

codings, making static scheduling far more effective than it would be for

a traditional cluster. In fact, PAPERS provides such a precise time

reference that it can even be used to support scheduling of hard

real-time operations; latency may be a few microseconds, but timing

accuracy (repeatability or clock synchronization) is typically within a

single microsecond.

1.2. Parallel Computation Model

++++++++++++++++++++++++++++++++

The PAPERS library is not compliant with emerging standards for

message-passing libraries (e.g., PVM [GeS93] or MPI [Mes94] ) because

the basic operations supported by the PAPERS hardware are not based

on a message-passing model. While interactions between PEs using

message-passing systems are often asynchronous and are always

initiated by individual PEs, PAPERS operations are aggregate operations

which are requested and performed in unison by all PEs within a

specified group. Thus, although a PAPERS cluster is not restricted to

SIMD execution, all PAPERS operations are based on a model of

aggregate interaction that closely resembles SIMD communication. This

is not a coincidence; years of research involving Purdue University's

PASM (PArtitionable Simd Mimd) prototype supercomputer have

experimentally demonstrated that SIMD-like interactions are more

efficient than traditional message-passing for a wide range of

applications [BeS91] .

The PAPERS execution model is based on the properties of the

generalized barrier synchronization mechanism that we have been

developing since [DiS88] . In barrier MIMD execution, each processor

executes independently until it wishes to synchronize with an arbitrary

group of (one or more) other processors. The barrier synchronization is

accomplished by each processor individually informing the barrier unit

that it has arrived, and then waiting to be notified that all participating

processors have arrived. Once the barrier synchronization has completed,

execution of each processor is again completely independent. Thus,

barrier MIMD execution is a purely MIMD execution model that can, at

any point desired, impose timing properties accurately emulating either a

SIMD or VLIW execution model.

Suppose that, instead of executing just a barrier wait, all participating

processors may additionally perform a communication operation:

communication is literally a side- effect of an arbitrary set of PEs

executing a barrier synchronization. This communication model is

somewhat like synchronous message-passing in concept, but is much

more powerful because communication need not be point-to-point. In

this model, the PAPERS unit collects an aggregate of the data items

output by each PE that participates in the current barrier. The value

obtained by each PE can thus be any of a wide range of functions of the

aggregated data -- and these functions can be computed in parallel

within the PAPERS unit. For example, reductions, scans (parallel prefix

operations), and various other functions can yield performance

comparable to or better than the cost of performing a simple permutation

communication using a more traditional network. Current PAPERS

hardware directly implements only a subset of all possible functions, but

the high performance achieved with remarkably simple hardware clearly

demonstrates the power of the model.

Despite the synchronous nature of aggregate communication, it is

important to remember that only the specified group of PEs must

participate in each communication, and every PE can be executing a

unique program. In fact, each PE participating in a particular aggregate

communication even can behave differently within that communication

(e.g., one processor broadcasts while others receive). However, unlike

asynchronous message-passing, the role of each processor in a

synchronization or communication operation must be determined by that

processor. Processors are free to dynamically alter how they interact, but

only by means that preserve the static qualities of the interactions. For

example, the only way for processor A to get data from processor B is for

B to know that some processor might want this data, and to act

accordingly in aggregate with A.

The PAPERS hardware also provides a parallel interrupt mechanism

that can be used by any processor to signal the selected group of

processors, and this device could be used to simulate asynchronous

message-passing. However, the performance characteristics of

asynchronous message-passing using PAPERS in this way are similar to

those obtained using PVM. Thus, we do not include any such messaging

operations in this paper, nor do we consider such operations to be

properly part of the PAPERS library communication model. PAPERS

parallel interrupts are normally used only by PEN, the Papers

ENvironment that performs high-level parallel OS functions such as

gang scheduling of parallel programs.

In summary, the PAPERS library model is neither message- passing nor

shared memory, but is an inherently different model based on the concept

of synchronously aggregating data from an arbitrary group of PEs. We

suggest that this barrier MIMD based model yields simple and efficient

hardware that better supports the needs of many parallel applications

executing in any combination of MIMD, SIMD, and VLIW execution

modes.

2. Overview

============

The PAPERS library discussed in this document is designed to provide

all the basic components needed to support a wide range of parallel

applications. The routines are intended to be called directly within C

code to be compiled by the Gnu C Compiler. Thus, this library can be

used:

o Directly by the application programmer using C or any language

that can be translated into C (e.g., Fortran via f2c, Pascal via p2c).

o By an optimizing/parallelizing compiler generating stylized C

code as a "portable assembly language" target. We have

experimentally confirmed that expressing tuples as C code written

using temporary variables declared with the register attribute

results in an efficient, and predictable, translation to native

assembly code. For example, the PAPERS MPL compiler

generates code in this style. This approach allows

instruction-level analysis and optimization to be easily

parameterized to account for differences between machines --

which is particularly important if the machines within a cluster

are heterogeneous.

In this paper, the basic PAPERS library functions are described and

benchmarked in detail. The PAPERS library also contains other

operations such as support for parallel file I/O; however, these routines

are beyond the scope of the current work.

Before describing the library routines themselves and their performance

relative to comparable routines in other libraries, it is useful to review

the basic data types managed by the routines. This is done in Section 2.1.

Likewise, the performance evaluation requires some background

discussion of the methods used to measure performance and scalability.

Section 2.2 reviews these issues.

2.1. Data Types

++++++++++++++++

Unlike most implementations of PVM [GeS93] or MPI [Mes94] , the

PAPERS hardware and interface software does not have a significant

set-up time; an operation on n units of data generally takes nearly the

same time as performing n single- unit data operations. This fact is

clearly demonstrated by the benchmark data given in the following

sections. Thus, there is no reason to pack/unpack blocks of data into a

memory-based message buffer. It is actually more efficient to perform

the PAPERS operations directly using processor registers, with one

PAPERS operation performed for each datum being acted upon, because

this saves the cost of having to read and write each datum to a local

memory buffer. In fact, the time to transmit a datum from a processor

register through a PAPERS unit could be comparable to the time taken

for two local memory buffer accesses.

Thus, unlike PVM or MPI, the PAPERS library routines focus on

low-latency communication operations that can act upon individual data

values within a computation. Functions that return data values have

multiple versions, one for each basic data type supported by the Gnu C

Compiler (i.e., each type that could be held in a processor register). Each

of these types is listed in Table 1, along with the equivalent portable type

definition used in the PAPERS library and the corresponding function

name suffix. For example, the function that sums a 32-bit unsigned int

value from each participating processor is called p_reduceAdd32u(),

where the suffix 32u identifies the type of both the operand and the

return value.

+--------------------------+-------------+---------------+

|        GCC Type          | PAPERS Type | PAPERS Suffix |

+--------------------------+-------------+---------------+

|boolean (unsigned int)    | uint1       | 1u            |

|8-bit char                | int8        | 8             |

|8-bit unsigned char       | uint8       | 8u            |

|16-bit short              | int16       | 16            |

|16-bit unsigned short     | uint16      | 16u           |

|32-bit int                | int32       | 32            |

|32-bit unsigned int       | uint32      | 32u           |

|64-bit long long          | int64       | 64            |

|64-bit unsigned long long | uint64      | 64u           |

|32-bit float              | f32         | f             |

|64-bit double             | f64         | d             |

+--------------------------+-------------+---------------+

Table 1: PAPERS Data Types

The portable definition of these types yields a minor complication: if the

machines within a cluster are not all based on the same processor, each

may use a slightly different representation for the same value. In PVM 2,

for example, this was handled using the notoriously slow UNIX socket

XDR (eXternal Data Representation) library routines to convert to a

standard exchange format as values are moved into or out of the message

buffer in local memory. However, the PAPERS unit is fed data in a way

that is inherently insensitive to memory byte order; each datum is

apparently sent directly from a processor register to a processor register.

Thus, all integer data are inherently portable, and byte-order variations

on floating point formats are also handled correctly. Currently, we

assume that the floating point formats used on different machines do not

differ by more than byte ordering -- an assumption that holds true for

the vast majority of workstations. Later versions of the PAPERS library

may handle more drastic floating point format variations by sending

floating point values as separate exponent and mantissa, because these

can be portably obtained and sent faster using PAPERS than the

standard XDR routine could be applied. Of course, none of these

problems arise in homogeneous clusters or dedicated parallel machines.

A similar portability constraint arises with respect to taking advantage of

the fact that PAPERS may be able to operate faster on lower-precision

values by transmitting only the specified number of bits. Although it

might be possible to, for example, send only a portion of the mantissa of

a floating point value, such operations are not portable. Thus, only

functions that operate on unsigned integer values can be augmented by

versions that have the ability to restrict the PAPERS operations to a

specific number of bits. These functions are named with the suffix Bits

followed by the suffix that specifies the unsigned integer type from which

the low bits are extracted. The benchmark data presented in this paper

ignores the Bits operations because none of the other libraries supports

any such operations.

We have also omitted benchmarks of the 1u routines, which are used to

manage 1-bit logical true/false values. Most of these routines take the

same time as p_any(). Further, none of the other libraries directly

supports such operations.

2.2. Performance Analysis

++++++++++++++++++++++++++

Although it is very difficult to argue against the basic concept of

reducing hardware and software overhead, the primary purpose of the

performance analysis in this paper is to show how effective the PAPERS

library is in comparison to other libraries and other hardware. As a

secondary issue, because these operations are portable at least across the

listed platforms, these performance numbers also can provide a useful

guide for estimating the performance of codes being ported from one

platform to another.

We were able to obtain at least some performance numbers for four

different libraries running on dedicated commercial supercomputers. The

libraries benchmarked were:

o Cray T3D [Cra93] . A 4 PE partition was benchmarked. This

library supports very few of the aggregate operations, but is very

effective in implementing these few operations.

o Cray T3D (PVM). A 4 PE partition was benchmarked. At the

time we conducted these benchmarks, this PVM-compatible

library supported only those aggregate operations supported in the

standard Cray library.

o MasPar MP-1 MPL support library. The MasPar that we

benchmarked is a 16,384 PE SIMD machine that was run with

only 4 PEs enabled. It is significant, however, that most

operations become only slightly slower when more PEs are

enabled.

o Paragon XP/S [Int94] . The Paragon that we benchmarked is a

140 PE MIMD machine, but the benchmarks reported were run

on a 4 PE partition. We found that execution time of most

operations increased nearly linearly as the number of PEs in the

partition is increased.

For our library running on a PAPERS cluster, we actually benchmarked

two different versions of the PAPERS hardware using the same four

personal computers as PEs. Each PE was an IBM ValuePoint 486DX33

system running Linux 1.1.75. The PAPERS units were:

o PAPERS1. A PAL-based version of PAPERS that connects to 4

PEs via their parallel ports. This unit supports the full dynamic

barrier mechanism [OKD90a] (implemented as described in

[Muh95] ) and provides hardware support for both putget and

NAND aggregate operations. Because the external data path for

each PE is just 4 bits wide, only operations that require each PE

to obtain more data as the design is scaled to larger systems incur

proportionally more delay.

o TTL_PAPERS [DiC95] . A TTL-based version of PAPERS that

connects to 4 PEs via their parallel ports. This design has been

greatly simplified to make it cheaper and easier to build, so

performance of TTL_PAPERS generally marks the lower bound

on PAPERS performance. This unit supports the full static

barrier mechanism [OKD90] and provides hardware support for

NAND aggregate operations. Because both the internal and

external data paths are just 4 bits wide, operations that require

more data to pass through the unit as the design is scaled to larger

systems incur proportionally more delay.

To provide a reference for the performance of a more traditional cluster

parallelism library, we also benchmarked performance using the same

PEs that we used to benchmark the PAPERS units listed above. These

four IBM ValuePoint 486DX33 systems running Linux 1.1.75 were

connected using a standard Ethernet (with insignificant other traffic on

the Ethernet). The library benchmarked was:

o PVM 3.3.7 [GeS93] . This library is the de-facto standard for

parallel computing on workstation clusters, and this version

generally yields significantly better benchmark times than older

versions of PVM. Because communication is using Ethernet, the

execution time of most operations increases nearly linearly as the

number of PEs is increased.

Because appropriately detailed performance measurements are very

rarely collected and published, we found it necessary to conduct nearly

all of the benchmarking ourselves. To minimize any bias, we have used

the standard library routine without modification for each supported

operation, and indicate times obtained for operations that were not

directly supported by following the time with the "*" character; a "?"

character is given instead of a time in cases where the routine was not

directly supported and we were unable to create an efficient

implementation. We also found that some operations could not be

supported without major restructuring, in which case we list a "-"

character rather than an execution time.

All the tables in the remainder of this document have the same form.

Each line corresponds to the system and library being measured. Each

column corresponds to the operation being performed. Operations within

the same table that differ only by the size and type of their

operand/return value are indicated by giving the size of the integer value

in bits or listing "Float for 32-bit floating-point and "Double" for 64-bit

floating-point. (The timing difference between signed and unsigned

integer operations is generally negligible.) All execution times are listed

in microseconds.

3. PAPERS Management Operations

================================

p_init()     Check-in with PAPERS hardware

p_exit(e)    Check-out with PAPERS hardware, exit code e

p_error(s)   Print error message s, check-out, and exit

Before any program can use the PAPERS hardware, it must:

o Obtain PAPERS access permission from the operating system.

o Confirm that the software and hardware environment is

consistent.

o Generate an initial barrier synchronization.

All of these functions are performed by simply calling p_init().

To indicate that a program has completed using PAPERS, it calls

p_exit() with an argument which is used much like the argument to the

ordinary UNIX exit() call. Notice, however, that calling p_exit() does not

necessarily terminate the calling process -- it just terminates the use of

PAPERS. The routine p_error() is provided to terminate the current

PAPERS parallel program with the given error message.

4. Low-Level PAPERS Operations

===============================

p_enqueue(m)   Enqueue the barrier mask m

p_wait()       Barrier synchronize with current mask

p_waitvec(f)   Return  bit  vector  assembled from f of each

active processor

p_any(f)       Did any enabled processor have f true?

p_all(f)       Did all enabled processors have f true?

At the lowest level of the PAPERS library, these routines form the

interface for the most direct use of the PAPERS hardware.

To set the current barrier group, p_enqueue() is called with a bit mask

that has a 1 bit in each position corresponding to a member of the barrier

group. Currently, by default, all available processors are in the initial

barrier group. To perform a barrier wait with the current barrier group,

simply call p_wait().

The remaining three functions all operate on integer flags as single-bit

truth values. The p_any() and p_all() operations are familiar enough to

any user of parallel processing.

The p_waitvec() operation is actually a one-bit multibroadcast that

allows all processors to obtain a mask vector constructed from the

one-bit votes of all processors. Processor k's vote is in bit k of the result.

Enabled processors vote 1 if f is true (non-zero), 0 otherwise. Disabled

processors always vote 0. This voting operation was originally devised so

that processors could vote on which partition they wanted to be in when

a barrier group is divided by a parallel conditional, trivially creating the

new barrier mask for each processor. However, it is also used to

determine things like which processors want to access a shared resource,

or which have extra work to do, etc. Thus, p_waitvec() is very commonly

used, for example, by code generated by compilers or translators to

schedule I/O operations; however, users are not expected to directly

access this type of function.

The times measured for these functions are listed in Table 2. Note that

the Cray T3D hardware barrier mechanism does not support arbitrary

partitioning, but only static partitioning on fixed boundaries; thus, the

concept of dynamically enqueuing a new barrier grouping does not exist.

Likewise, both the Paragon XP/S and PVM 3 treat groupings as

essentially static. However, as a SIMD machine with enable masking

hardware, the MasPar MP-1 supports runtime re- grouping of PEs very

efficiently. The MasPar's completely synchronous nature explains the

remarkable Wait time, and its global OR network makes the other times

also very respectable.

+---------+----------+---------+-----+-----+

| Enqueue |   Wait   | Waitvec | Any | All |

+---------------+---------+----------+---------+-----+-----+

|Cray T3D       |    -    |      1.7 |    ?    |  ?  |  ?  |

|Cray T3D (PVM) |    -    |     21   |    ?    |  ?  |  ?  |

|MasPar MP-1    |   4.7   |      0.1 |   9.4   | 2.7 | 5.1 |

|Paragon XP/S   |    -    |    530   |    ?    |  ?  |  ?  |

+---------------+---------+----------+---------+-----+-----+

|PAPERS1        |   3.5   |      3.1 |   3.2   | 3.2 | 3.3 |

|TTL_PAPERS     |   0.1   |      2.5 |   6.3   | 6.3 | 6.3 |

+---------------+---------+----------+---------+-----+-----+

|PVM 3          |    -    | 49,000   |    ?    |  ?  |  ?  |

+---------------+---------+----------+---------+-----+-----+

Table 2: Timing for Low-Level Operations (microseconds)

5. Putget Operations

=====================

p_putgettype(d, s)   Put d, get (return) d from PE s

The PAPERS library putget operation is the most fundamental

mechanism for supporting arbitrary communication between processors.

The model is simply that each processor in the current barrier group

outputs a datum, specifies which processor it wants the datum from, and

returns the selected value. Logically, each processor writes to its own bus

buffer within the PAPERS unit and simply snoops the value written on

the bus it selects.

Although some versions of PAPERS directly implement this mechanism

(e.g., PAPERS1), TTL_PAPERS does not. Instead, it must map the

putget into a series of simple broadcasts (see Section 9). There are two

basic approaches to this mapping. One method is to simply force every

processor in the barrier group to perform a broadcast; this is simple and

consistent, but is inefficient for patterns in which the values sent by some

processors are not used (i.e., this technique is best for permutations). The

alternative is to first collect a vote from the processors as to which PEs

would be sending useful values, and then to send only those values.

Surprisingly, this voting operation is trivially accomplished for n

processors using n/4 4-bit NAND operations to accumulate a vote bit

vector in which a 1 bit corresponds to a processor that at least one PE

wants the value from, and a 0 bit indicates that the corresponding PE

need not send any data. In fact, this voting operation is just p_waitvec().

The timing of putget operations is particularly important because this

operation is used to implement arbitrary communication patterns. For

example, a single putget can implement any permutation or any

multi-cast. There is no concept of "neighbors" or network contention for

putget -- every processor is just one communication away.

Table 3 gives the times for putget operations. Clearly, the MasPar's

router network supports the notion of putget surprisingly well. What is

not evident in this table is that even the times for the Paragon and PVM

are essentially independent of the communication pattern implied by the

putget.

+----------+---------+----------+----------+---------+----------+

|      8   |     16  |      32  |      64  |  Float  |  Double  |

+-------------+----------+---------+----------+----------+---------+----------+

|MasPar MP-1  |     33   |     36  |      44  |      67  |     44  |      67  |

|Paragon XP/S |    710*  |    700* |     700* |     700* |    700* |     700* |

+-------------+----------+---------+----------+----------+---------+----------+

|PAPERS1      |      8.5 |     15  |      27  |      62  |     29  |      63  |

|TTL_PAPERS   |     58   |    111  |     216  |     434  |    219  |     467  |

+-------------+----------+---------+----------+----------+---------+----------+

|PVM 3        | 97,000*  | 96,000* | 100,000* | 100,000* | 96,000* | 101,000* |

+-------------+----------+---------+----------+----------+---------+----------+

Table 3: Timing for Putget Operations (microseconds)

6. Gather Operations

=====================

p_gathertype(p, d)   Gather an array p[i] = d from PE i

The PAPERS library gather operations collect a datum from each

processor in the current barrier group, and fill-in the corresponding

entries in each processor's local memory array whose address is passed as

the first argument to the function. Array entries corresponding to

processors that are not in the current barrier group are unaffected by the

gather. Thus, a gather operation is essentially implemented by

performing either n broadcast operations (as in TTL_PAPERS) or n-1

putget operations (as in PAPERS1).

There is no scheduling overhead introduced for TTL_PAPERS to

schedule the broadcasts because each processor in the group has a local

copy of the barrier bit mask. Thus, no communication is needed to agree

on which processor will broadcast when. The order is from the

lowest-numbered enabled PE to the highest, determined independently

by each processor shifting through its local copy of the barrier bit mask.

Likewise, PAPERS1 putgets are scheduled without additional overhead.

The performance of these functions is summarized in Table 4.

+---------+---------+---------+---------+---------+----------+

|      8  |     16  |     32  |     64  |  Float  |  Double  |

+-------------+---------+---------+---------+---------+---------+----------+

|MasPar MP-1  |    125* |    144* |    191* |    290* |    191* |     290* |

|Paragon XP/S |    710* |    700* |    700* |    700* |    700* |     700* |

+-------------+---------+---------+---------+---------+---------+----------+

|PAPERS1      |     33  |     53  |     89  |    194  |     95  |     199  |

|TTL_PAPERS   |     57  |    110  |    216  |    439  |    248  |     444  |

+-------------+---------+---------+---------+---------+---------+----------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* | 98,000* | 100,000* |

+-------------+---------+---------+---------+---------+---------+----------+

Table 4: Timing for Gather Operations (microseconds)

7. Arithmetic Reduction Operations

===================================

p_reduceAddtype(d)   Return the sum of d from each PE

p_reduceMultype(d)   Return the product of d from each PE

p_reduceMintype(d)   Return the minimum of d from each PE

p_reduceMaxtype(d)   Return the maximum of d from each PE

By definition, a reduction operation is an associative operation; i.e., the

order in which the component operations are performed does not

significantly alter the resulting value. One way to perform these

operations is to have all processors collect all the data and then serially

reduce the local copy. This yields a very simple implementation suitable

for clusters with up to about 8 processors, and this technique is currently

used for the 4 PE TTL_PAPERS library. For example, all the basic

variations on p_reduceAdd can be implemented by:

#define REDUCEADD(name, type, suffix) \

type \

name(register type datum) \

{ \

type buf[NPROC]; \

buf[0] = (buf[1] = (buf[2] = (buf[3] = 0))); \

p_gather##suffix(&(buf[0]), datum); \

return(buf[0] + buf[1] + buf[2] + buf[3]); \

}

REDUCEADD(p_reduceAdd8, int8, 8)

REDUCEADD(p_reduceAdd8u, uint8, 8u)

REDUCEADD(p_reduceAdd16,int16, 16)

REDUCEADD(p_reduceAdd16u, uint16, 16u)

REDUCEADD(p_reduceAdd32,int32, 32)

REDUCEADD(p_reduceAdd32u, uint32, 32u)

REDUCEADD(p_reduceAdd64,int64, 64)

REDUCEADD(p_reduceAdd64u, uint64, 64u)

REDUCEADD(p_reduceAddf, f32, f)

REDUCEADD(p_reduceAddd, f64, d)

The problem with using this code on larger clusters is that it executes in

O(n), rather than O(log2n), time for n processors. This is remedied by

using the usual tree reduction with communication implemented by a

putget operation for each of the log2n communication steps. However,

the tree reduction is somewhat complicated by the fact that some

processors might not be members of the current barrier group, in which

case they do not participate in the reduction. Thus, if a tree reduction is

to be used, the participating nodes must effectively be "renumbered" to

determine their positions within the reduction tree. Fortunately, the

renumbering can be computed without any communication operations

-- each node is numbered by p_enumerate() and the tree contains

p_population() leaves (see Section 11). This is the method used by

PAPERS1.

Benchmark times for sum reduction, product reduction, minimum

reduction, and maximum reduction are presented in Tables 5 through 8.

+----------+----------+---------+--------+---------+---------+

|       8  |      16  |      32 |     64 |  Float  | Double  |

+-------------+----------+----------+---------+--------+---------+---------+

|MasPar MP-1  |     124  |     139  |     180 |    266 |     302 |     428 |

|Paragon XP/S |     700* |     700* |     700 |    700 |     710 |     680 |

+-------------+----------+----------+---------+--------+---------+---------+

|PAPERS1      |      20  |      33  |      57 |    128 |      63 |     131 |

|TTL_PAPERS   |      58  |     112  |     217 |    438 |     219 |     444 |

+-------------+----------+----------+---------+--------+---------+---------+

|PVM 3        | 101,000* | 101,000  | 100,000 | 99,000 | 100,000 | 116,000 |

+-------------+----------+----------+---------+--------+---------+---------+

Table 5: Timing for ReduceAdd Operations (microseconds)

+----------+----------+---------+---------+---------+---------+

|       8  |      16  |      32 |      64 |  Float  | Double  |

+-------------+----------+----------+---------+---------+---------+---------+

|MasPar MP-1  |     158  |     235  |     425 |   1,030 |     413 |     849 |

|Paragon XP/S |     700* |     700* |     700 |     700 |     710 |     690 |

+-------------+----------+----------+---------+---------+---------+---------+

|PAPERS1      |      21  |      34  |      58 |     132 |      63 |     130 |

|TTL_PAPERS   |      60  |     113  |     218 |     447 |     222 |     442 |

+-------------+----------+----------+---------+---------+---------+---------+

|PVM 3        | 102,000* | 102,000  | 101,000 | 105,000 | 103,000 | 138,000 |

+-------------+----------+----------+---------+---------+---------+---------+

Table 6: Timing for ReduceMul Operations (microseconds)

+----------+----------+--------+---------+---------+---------+

|       8  |      16  |     32 |      64 |  Float  | Double  |

+-------------+----------+----------+--------+---------+---------+---------+

|MasPar MP-1  |      32  |      49  |     81 |     120 |      87 |     172 |

|Paragon XP/S |     690* |     690* |    690 |     690 |     710 |     690 |

+-------------+----------+----------+--------+---------+---------+---------+

|PAPERS1      |      20  |      33  |     57 |     127 |      63 |     131 |

|TTL_PAPERS   |      59  |     112  |    218 |     442 |     244 |     475 |

+-------------+----------+----------+--------+---------+---------+---------+

|PVM 3        | 104,000  | 101,000  | 99,000 | 102,000 | 102,000 | 137,000 |

+-------------+----------+----------+--------+---------+---------+---------+

Table 7: Timing for ReduceMin Operations (microseconds)

+----------+----------+--------+--------+--------+---------+

|       8  |      16  |     32 |     64 | Float  | Double  |

+-------------+----------+----------+--------+--------+--------+---------+

|MasPar MP-1  |      28  |      40  |     64 |    131 |     72 |     140 |

|Paragon XP/S |     700* |     700* |    700 |    690 |    710 |     690 |

+-------------+----------+----------+--------+--------+--------+---------+

|PAPERS1      |      20  |      33  |     57 |    127 |     63 |     131 |

|TTL_PAPERS   |      59  |     113  |    218 |    439 |    297 |     458 |

+-------------+----------+----------+--------+--------+--------+---------+

|PVM 3        | 104,000  | 101,000  | 98,000 | 98,000 | 99,000 | 140,000 |

+-------------+----------+----------+--------+--------+--------+---------+

Table 8: Timing for ReduceMax Operations (microseconds)

8. Bitwise Reduction Operations

================================

p_reduceAnditype(d)   Return the bitwise AND of d from each PE

p_reduceOritype(d)    Return the bitwise OR of d from each PE

The TTL_PAPERS hardware communication mechanism is actually a

4-bit NAND of the data from all processors in the barrier group, which

is accessed within the library by the in-line function p_nand(). Thus, a

4-bit bitwise OR can be implemented simply by complementing the

inputs to the NAND. Likewise, a 4-bit bitwise AND is generated by

complementing the result of a NAND. Similar logic is also used within

PAPERS1.

Because bitwise operations are defined for each integer data type, there

are nine different versions of each bitwise reduction operation.

Fortunately, a 4*k-bit bitwise operation can be implemented by

performing k 4-bit operations and using shifts to disassemble and

reassemble the values. The use of shifts is important because it ensures

that the resulting value is computed correctly even though some of the

machines in the cluster may differ in the byte order that they use to store

data in local memory.

For example, the following C code defines the eight different multi-bit

versions of p_reduceOr:

#define REDUCEOR(name, type, bitcnt) \

type \

name(register type datum) \

{ \

if (last_mask & ~OUR_MASK) { \

register int bits = (bitcnt - 4); \

\

/* Four bits at a time, or = nand not */ \

do { \

datum |= (p_nand((~(datum >> bits)) & 0xf) \

<< bits); \

} while ((bits -= 4) >= 0); \

} \

\

return(datum); \

}

REDUCEOR(p_reduceOr8, int8, 8)

REDUCEOR(p_reduceOr8u, uint8, 8)

REDUCEOR(p_reduceOr16, int16, 16)

REDUCEOR(p_reduceOr16u, uint16, 16)

REDUCEOR(p_reduceOr32, int32, 32)

REDUCEOR(p_reduceOr32u, uint32, 32)

REDUCEOR(p_reduceOr64, int64, 64)

REDUCEOR(p_reduceOr64u, uint64, 64)

Notice that the check involving OUR_MASK bypasses the entire

communication if only this processor is in the current barrier group.

Timing for AND and OR reductions are given in Table 9 and Table 10.

+----------+----------+--------+---------+

|       8  |      16  |     32 |      64 |

+-------------+----------+----------+--------+---------+

|MasPar MP-1  |     118  |     139  |    180 |     266 |

|Paragon XP/S |     700* |     700* |    700 |     700 |

+-------------+----------+----------+--------+---------+

|PAPERS1      |       9  |      16  |     29 |      81 |

|TTL_PAPERS   |      16  |      31  |     59 |     137 |

+-------------+----------+----------+--------+---------+

|PVM 3        | 102,000  | 100,000  | 99,000 | 100,000 |

+-------------+----------+----------+--------+---------+

Table 9: Timing for ReduceAnd Operations (microseconds)

+-----------+---------+---------+---------+

|       8   |     16  |      32 |      64 |

+-------------+-----------+---------+---------+---------+

|MasPar MP-1  |       9.4 |     12  |      17 |      31 |

|Paragon XP/S |     710*  |    710* |     710 |     710 |

+-------------+-----------+---------+---------+---------+

|PAPERS1      |       9   |     16  |      30 |      81 |

|TTL_PAPERS   |      16   |     30  |      59 |     137 |

+-------------+-----------+---------+---------+---------+

|PVM 3        | 104,000   | 99,000  | 100,000 | 103,000 |

+-------------+-----------+---------+---------+---------+

Table 10: Timing for ReduceOr Operations (microseconds)

9. Broadcast Operations

========================

p_bcastPutitype(d)   Broadcast put d from this PE

p_bcastGetitype()    Broadcast get value from sending PE

Although both PAPERS1 and TTL_PAPERS directly implement

broadcasting, these functions are actually implemented as macros of

other PAPERS library routines. For example, both the TTL_PAPERS

and PAPERS1 definitions of p_bcastPut32() and p_bcastGet32() are:

#define p_bcastPut32(d) ((void) p_reduceOr32(d))

#define p_bcastGet32() p_reduceOr32(0)

Notice that a PE which is simply getting the broadcast value is really

contributing 0 to a global OR.

Table 11 summarizes the benchmark results for paired broadcast put and

get operations. The MasPar times are very low because the global OR

network is used (in the guise of the MPL proc[] construct) to obtain the

value from the PE that is broadcasting and the SIMD broadcast

hardware is then used to transmit that value to all PEs.

+----------+--------+--------+--------+--------+--------+

|      8   |     16 |     32 |     64 | Float  | Double |

+---------------+----------+--------+--------+--------+--------+--------+

|Cray T3D (PVM) |    ?     |   ?    |   ?    |     82 |   ?    |     82 |

|MasPar MP-1    |      9.5 |     12 |     18 |     31 |     18 |     31 |

|Paragon XP/S   |    210   |    220 |    210 |    210 |    210 |    210 |

+---------------+----------+--------+--------+--------+--------+--------+

|PAPERS1        |      9   |     16 |     30 |     81 |     30 |     81 |

|TTL_PAPERS     |     16   |     30 |     59 |    137 |     59 |    137 |

+---------------+----------+--------+--------+--------+--------+--------+

|PVM 3          | 38,000   | 38,000 | 38,000 | 40,000 | 39,000 | 41,000 |

+---------------+----------+--------+--------+--------+--------+--------+

Table 11: Timing for Broadcast Put/Get Operations (microseconds)

10. Scan Operations

====================

p_scanAddtype(d)    Return the sum of d up to this PE

p_scanMultype(d)    Return the product of d up to this PE

p_scanAnditype(d)   Return the bitwise AND of d up to this PE

p_scanOritype(d)    Return the bitwise OR of d up to this PE

p_scanMintype(d)    Return the minimum of d up to this PE

p_scanMaxtype(d)    Return the maximum of d up to this PE

p_ranktype(d)       Return  the  rank of d for each PE, such that

the smallest value has rank 0

A scan, or parallel prefix, computation is much like a reduction, but

returns partial results to each of the processors. Like reductions, these

operations can be performed in parallel or serially. For a small number of

processors, using a gather operation to collect the data values in each

processor and then serially computing the result for each processor in

local memory works well, but there are many alternative

implementations.

Aside from using a tree structured implementation (with the same

barrier group numbering problem described for reductions in Section 7),

many of the scan operations that use integer data can be implemented

efficiently by methods based on p_waitvec(). In some cases, the

p_waitvec() would be used for processors to vote high or low relative to

the value output by one processor (e.g., for a rank operation). For the

Bits variants of the scans, a series of p_waitvec() operations can be used

to transmit only the desired data bits from all processors.

Both TTL_PAPERS and PAPERS1 currently implement these

operations using a gather and serial local evaluation. The timing of sum,

product, AND, OR, minimum, and maximum parallel prefix operations

are given in Tables 12 through 17. Although not strictly a scan

operation, the ranking operation (used for sorting) behaves very much

like a scan, and is thus benchmarked in Table 18.

+---------+---------+---------+---------+---------+----------+

|      8  |     16  |     32  |     64  |  Float  |  Double  |

+-------------+---------+---------+---------+---------+---------+----------+

|MasPar MP-1  |    381  |    561  |    920  |  1,640  |  1,041  |   1,799  |

|Paragon XP/S |    710* |    700* |    700* |    700* |    700* |     700* |

+-------------+---------+---------+---------+---------+---------+----------+

|PAPERS1      |     35  |     55  |     91  |    197  |     97  |     204  |

|TTL_PAPERS   |     63  |    124  |    227  |    463  |    245  |     456  |

+-------------+---------+---------+---------+---------+---------+----------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* | 98,000* | 100,000* |

+-------------+---------+---------+---------+---------+---------+----------+

Table 12: Timing for ScanAdd Operations (microseconds)

+---------+---------+---------+---------+---------+----------+

|      8  |     16  |     32  |     64  |  Float  |  Double  |

+-------------+---------+---------+---------+---------+---------+----------+

|MasPar MP-1  |    421  |    651  |  1,165  |  2,045  |  1,150  |   2,220  |

|Paragon XP/S |    710* |    700* |    700* |    700* |    700* |     700* |

+-------------+---------+---------+---------+---------+---------+----------+

|PAPERS1      |     37  |     57  |     92  |    205  |     97  |     200  |

|TTL_PAPERS   |     65  |    116  |    223  |    454  |    235  |     466  |

+-------------+---------+---------+---------+---------+---------+----------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* | 98,000* | 100,000* |

+-------------+---------+---------+---------+---------+---------+----------+

Table 13: Timing for ScanMul Operations (microseconds)

+---------+---------+---------+---------+

|      8  |     16  |     32  |     64  |

+-------------+---------+---------+---------+---------+

|MasPar MP-1  |    381  |    561  |    920  |  1,645  |

|Paragon XP/S |    710* |    700* |    700* |    700* |

+-------------+---------+---------+---------+---------+

|PAPERS1      |     31  |     61  |    118  |    245  |

|TTL_PAPERS   |     58  |    114  |    222  |    481  |

+-------------+---------+---------+---------+---------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* |

+-------------+---------+---------+---------+---------+

Table 14: Timing for ScanAnd Operations (microseconds)

+---------+---------+---------+---------+

|      8  |     16  |     32  |     64  |

+-------------+---------+---------+---------+---------+

|MasPar MP-1  |    381  |    561  |    920  |  1,640  |

|Paragon XP/S |    710* |    700* |    700* |    700* |

+-------------+---------+---------+---------+---------+

|PAPERS1      |     34  |     68  |    131  |    272  |

|TTL_PAPERS   |     67  |    120  |    244  |    501  |

+-------------+---------+---------+---------+---------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* |

+-------------+---------+---------+---------+---------+

Table 15: Timing for ScanOr Operations (microseconds)

+---------+---------+---------+---------+---------+----------+

|      8  |     16  |     32  |     64  |  Float  |  Double  |

+-------------+---------+---------+---------+---------+---------+----------+

|MasPar MP-1  |    390  |    573  |    934  |  1,662  |    940  |   1,740  |

|Paragon XP/S |    710* |    700* |    700* |    700* |    700* |     700* |

+-------------+---------+---------+---------+---------+---------+----------+

|PAPERS1      |     36  |     56  |     92  |    198  |    104  |     206  |

|TTL_PAPERS   |     61  |    114  |    220  |    442  |    226  |     465  |

+-------------+---------+---------+---------+---------+---------+----------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* | 98,000* | 100,000* |

+-------------+---------+---------+---------+---------+---------+----------+

Table 16: Timing for ScanMin Operations (microseconds)

+---------+---------+---------+---------+---------+----------+

|      8  |     16  |     32  |     64  |  Float  |  Double  |

+-------------+---------+---------+---------+---------+---------+----------+

|MasPar MP-1  |    390  |    571  |    934  |  1,663  |    941  |   1,741  |

|Paragon XP/S |    710* |    700* |    700* |    700* |    700* |     700* |

+-------------+---------+---------+---------+---------+---------+----------+

|PAPERS1      |     36  |     56  |     92  |    198  |    102  |     206  |

|TTL_PAPERS   |     61  |    114  |    220  |    443  |    225  |     451  |

+-------------+---------+---------+---------+---------+---------+----------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* | 98,000* | 100,000* |

+-------------+---------+---------+---------+---------+---------+----------+

Table 17: Timing for ScanMax Operations (microseconds)

+---------+---------+---------+---------+---------+----------+

|      8  |     16  |     32  |     64  |  Float  |  Double  |

+-------------+---------+---------+---------+---------+---------+----------+

|MasPar MP-1  |  5,465  |  6,754  |  9,318  | 14,451  |  9,884  |  14,975  |

|Paragon XP/S |    710* |    700* |    700* |    700* |    700* |     700* |

+-------------+---------+---------+---------+---------+---------+----------+

|PAPERS1      |     37  |     57  |     93  |    201  |    103  |     205  |

|TTL_PAPERS   |     63  |    116  |    222  |    510  |    228  |     522  |

+-------------+---------+---------+---------+---------+---------+----------+

|PVM 3        | 96,000* | 96,000* | 98,000* | 99,000* | 98,000* | 100,000* |

+-------------+---------+---------+---------+---------+---------+----------+

Table 18: Timing for Rank Operations (microseconds)

11. Group Property Operations

==============================

p_population()    Return the total number of active PEs

p_enumerate()     Return a consecutive number for each active PE

p_selectFirst()   Return the lowest active PE number

p_selectOne()     Return any active PE's number

It is sometimes necessary to determine basic properties of the set of

processors in the current barrier group. All of these functions compute

properties directly from the local copy of the current barrier bit mask,

and hence need no communication.

The value of p_population() is literally the population count of (number

of 1 bits within) the barrier bit mask. Some machines make this

operation a single instruction. For other machines, it can be implemented

using a table to count bits within each chunk of the mask (e.g., an 8-bit

mask section can be counted using a 256-element table). For very large

clusters, the population count can be computed using a log2n time for n

bits "SIMD bit summation" within each processor (e.g., for 4 PEs,

t=(mask&5)+((mask>>1)&5); population=(t>>2)+t;).

The value of p_enumerate() can be determined in essentially the same

way, but before computing the bit population, the barrier mask should be

bitwise ANDed with the current processor's bit mask minus 1. Thus,

each processor counts the number of participating processors in the mask

that have lower processor numbers than itself.

The remaining two operations are actually implemented by just one

function in the current PAPERS library, which simply scans the current

barrier bit mask for the lowest 1 bit and returns that position. Again,

some machines have single instructions that can perform this type of

operation very efficiently, and table lookup also can be used.

The benchmark results are given in Table 19. Both PAPERS1 and

TTL_PAPERS implement all of these functions as table lookups based

on the barrier bit mask. The MasPar yields a more costly implementation

because the bit mask is spread across the PEs (i.e., each PE knows only

its own enable status), and communication is required. Because the other

libraries do not support partitioning into arbitrary barrier groups, they

have no barrier bit mask to examine, and these operations are not

supported.

+------------+-----------+-------------+-----------+

| Population | Enumerate | SelectFirst | SelectOne |

+------------+------------+-----------+-------------+-----------+

|MasPar MP-1 |   147*     |    572    |    35       |   12      |

+------------+------------+-----------+-------------+-----------+

|PAPERS1     |     0.5    |      1    |     0.7     |    0.8    |

|TTL_PAPERS  |     0.5    |      1    |     0.7     |    0.8    |

+------------+------------+-----------+-------------+-----------+

Table 19: Timing for Group Property Operations (microseconds)

12. Voting Operations

======================

p_voteCount(v)        Return number of PEs that voted for us

p_vote(v)             Return vote bit mask

p_matchCounttype(v)   Return number of PEs that voted as we did

p_matchtype(v)        Return match bit mask

One of the most powerful uses of the PAPERS library involves the use

of PAPERS to statically schedule operations at runtime based on precise

knowledge of the global state of the computation. We have found that

such scheduling often is based on determining which PEs, or how many

PEs, are contending for the same resource.

The most obvious resource upon which PEs might contend is the data

bus that connects each PE to the PAPERS unit. For example, suppose

that we need to have the sender, rather than the receiver, determine

which PE will get a datum. Unlike putget, sender-targeted

communication can have conflicts if multiple PEs want to send to the

same PE. The receiver can determine that this contention exists by

examining the return value obtained when each PE uses p_voteCount()

to register the number of the PE to which it will send a datum. If the

return value is 0, no PE targets this PE. If the return value is 1, the

communication can be accomplished in one operation. A higher number

indicates contention.

The use of p_vote() is similar to that of p_voteCount(), but the return

value is a new barrier mask containing only processors that voted for this

PE. Although this barrier mask might, or might not, be used to create a

new barrier group, it can be used very effectively to statically schedule

the contending accesses.

The concept of the match operations is very similar to that of the vote

operations, but each PE gets either the count or the barrier mask

representing the set of PEs that put forth the same data value as itself.

Although future versions of PAPERS will directly implement some of

these functions, both PAPERS1 and TTL_PAPERS currently implement

these operations by performing a sequence of log2n p_waitvec()

operations to transmit an n- bit vote or match value from all PEs. The

algorithm can be summarized as follows.

For each bit in the vote or match value, use p_waitvec() to transmit the

bit. The returned bit vector is inverted if this PE's corresponding PE

number/match value bit was a 0. By anding all the resulting bit vectors

together, we create a bit mask with a 1 in each position corresponding to

a PE that is contending for/with us. The counts are obtained by simply

performing a population count of bits in this mask.

The resulting execution times are listed in Table 20. Notice that none of

the other libraries directly provides this type of scheduling support

operation, although the MasPar's router network incorporates similar

logic for detecting contention within its network.

+-----------+------+-------------+--------+

| VoteCount | Vote | MatchCount8 | Match8 |

+-----------+-----------+------+-------------+--------+

|PAPERS1    |    6.9    |  6.4 |          26 |     26 |

|TTL_PAPERS |   13      | 13   |          51 |     50 |

+-----------+-----------+------+-------------+--------+

Table 20: Timing for Voting Operations (microseconds)

13. Conclusion

===============

In this paper, we have presented a brief overview of the new

communication model implemented by the PAPERS hardware and

library. This library is suitable for use either as a vehicle for

hand-coding applications or as a target for an optimizing compiler.

We have also documented that many of the functions provided by this

library are familiar in the sense that similar functions appear in a variety

of other libraries. However, as the detailed benchmarks reported in this

paper show, the PAPERS library routines are consistently among the

most efficient. We suggest that this efficiency is not due to our

cleverness, but due to the use of a computation model which treats

parallel communication as a tightly-coupled parallel operation rather

than as a bunch of seemingly independent, potentially conflicting,

block-oriented, high- latency, one-to-one transmissions.

The benefits of synchronous aggregate communication have long been

known in the SIMD world, and have been cited as a primary advantage

of SIMD/MIMD mixed-mode computation. However, it is a

misconception that such operations require SIMD execution. Making

aggregate operations efficient simply requires the ability to efficiently

achieve synchronization across a group of PEs -- which is to say it

requires fast hardware barrier synchronization. One also has to be

careful not to bury the communication hardware under too many

latency-laden layers of interface software.

All aspects of the PAPERS system design and support software are fully

public domain, and a wide variety of projects involving PAPERS are

underway. For more information, see WWW URL:

http://garage.ecn.purdue.edu/~papers/

References

===========

[BeS91]

T.B. Berg and H.J. Siegel, "Instruction Execution Trade-Offs for

SIMD vs. MIMD vs. Mixed Mode Parallelism," 5th International

Parallel Processing Symposium, April 1991, pp. 301-308.

[Cra93]

Cray Research Incorporated, Cray T3D System Architecture

Overview, Cray Research Incorporated, 1993.

[DiC95]

H. G. Dietz, T. M. Chung, T. Mattox, and T. Muhammad,

"Purdue's Adapter for Parallel Execution and Rapid

Synchronization: The TTL_PAPERS Design," submitted to

ACM- IEEE-CS Supercomputing '95, December 1995.

[DiS88]

H. G. Dietz and T. Schwederski, Extending Static Synchronization

Beyond SIMD and VLIW, Purdue University School of Electrical

Engineering, Technical Report TR-EE 88-25, June 1988.

[GeS93]

G. Geist and V. Sunderam, Evolution of the PVM concurrent

computing system, 38th Annual IEEE Computer Society

International Computer Conference, February 1993.

[Int94]

Intel Corporation, Paragon User's Guide, Intel Supercomputer

Systems Division, 1994.

[Mas91]

MasPar Computer Corporation, MasPar Programming Language

(ANSI C compatible MPL) Reference Manual, Software Version 2.2,

Document Number 9302-0001, Sunnyvale, California, November

1991.

[Mes94]

Message Passing Interface Forum, MPI: A Message-Passing

Interface Standard, Rice University, Technical Report CRPC-

TR94439, April 1994.

[Muh95]

T. Muhammad, Hardware Barrier Synchronization For A Cluster

Of Personal Computers, Purdue University School of Electrical

Engineering, MS Thesis, May 1995.

[OKD90]

O'Keefe, M.T., and Dietz, H.G., Hardware barrier

synchronization: static barrier MIMD (SBM). Proc. of 1990 Int'l

Conf. on Parallel Processing, St. Charles, IL, August 1990, pp. I

35-42.

[OKD90a]

O'Keefe, M.T., and Dietz, H.G., Hardware barrier

synchronization: dynamic barrier MIMD (DBM). Proc. of 1990

Int'l Conf. on Parallel Processing, St. Charles, IL, August 1990, pp.

I 43-46.