Notes for Application Implementors on

An Experimental ST-II Socket API

Contents

1.      Overview   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  1

1.1.        What ST is and What it Provides   .  .  .  .  .  .  .  1

2.      Two-party Overview  .  .  .  .  .  .  .  .  .  .  .  .  .  3

2.1.        Unidirectional Communication   .  .  .  .  .  .  .  .  3

2.2.        Bidirectional Communication .  .  .  .  .  .  .  .  .  5

2.2.1.          Two (Half-duplex) Streams  .  .  .  .  .  .  .  .  5

2.2.2.          Single (Full-duplex) Stream   .  .  .  .  .  .  .  5

3.      Multiple-party Overview   .  .  .  .  .  .  .  .  .  .  .  7

3.1.        Full-Duplex Variation .  .  .  .  .  .  .  .  .  .  .  7

4.      Example .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  8

4.1.        socket .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  8

4.2.        bind   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  9

4.3.        listen .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  9

4.4.        connect   .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 10

4.5.        accept .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 11

4.6.        recvmsg to obtain new connection parameters   .  .  . 11

4.7.        sendmsg to refuse a connection .  .  .  .  .  .  .  . 11

4.8.        sendmsg to accept a connection .  .  .  .  .  .  .  . 12

4.9.        recvmsg at origin to receive control messages .  .  . 12

4.10.       sending data .  .  .  .  .  .  .  .  .  .  .  .  .  . 13

4.11.       recvmsg at targets to receive data or control messages

.  .  .  . 14

4.12.       sendmsg at origin to disconnect targets .  .  .  .  . 15

4.13.       close  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 15

4.14.       select .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 15

*-------------------------------------------------------------------*

|       Copyright (c) 1991 by BBN Systems and Technologies,         |

|            A Division of Bolt Beranek and Newman Inc.             |

|                                                                   |

|  Permission to use, copy, modify, distribute, and sell this       |

|  software and its documentation for any purpose is hereby         |

|  granted without fee, provided that the above copyright notice    |

|  and this permission appear in all copies and in supporting       |

|  documentation, and that the name of Bolt Beranek and Newman      |

|  Inc.  not be used in advertising or publicity pertaining to      |

|  distribution of the software without specific, written prior     |

|  permission. BBN makes no representations about the suitability   |

|  of this software for any purposes.  It is provided "AS IS"       |

|  without express or implied warranties.                           |

*-------------------------------------------------------------------*

- 1 -

1.  Overview

This document is intended to provide application programmers

with an overview of the services provided by the ST-II protocol

and how an application may access those services in a BSD socket

environment.  A working knowledge of BSD sockets is assumed.  A

detailed knowledge of the ST-II protocol defined in RFC 1190 is

not assumed, but references are made to that document so that the

detailed descriptions of parameters and their elements that it

contains do not have to be repeated here.

The main goals of the experimental API described here are to

function with a SunOS 4.1 or later kernel without modifications

to the standard socket interface.  This requirement means not only

that only standard system calls are used, but also that they are

used in a manner consistent with their normal semantics and that

they are used in a way that does not provoke any bugs in their

implementation.

1.1.  What ST is and What it Provides

ST is a protocol at the same layer as IP and has been

developed to support efficient delivery of streams of packets

to either single or multiple destinations in applications

requiring guaranteed data rates and controlled delay

characteristics.  Consequently, ST has quantitative

Type-of-Service (TOS) parameters and requires that the

intermediate ("gateway") agents reserve resources for each

stream of data flowing through them, both within the agents

and the interconnecting networks.  Resources are negotiated

and reserved during a setup phase before data is transferred;

there is also a teardown phase during which resources are

released.

... for more details, see RFC1190 Sections:

2 Introduction (page 7),

2.2 Concepts and Terminology (page 9),

2.3 Relationship Between Applications and ST (page 11), and

2.5 Flow Specifications (page 14).

ST requires several complex parameters to be communicated

between an application program and the ST layer; see Sections

4.2.2.* in RFC 1190.  In general, these parameters are passed as a

list in the cmsghdr control structure of sendmsg()/recvmsg()

("accrights" argument) or getsockopt()/setsockopt() ("optval"

argument) functions, or in a sockaddr_st2 structure when

appropriate.  To meet the goals stated in Section 1, the size of

the sockaddr_st2 structure is limited to MAX_ST_NAM bytes (the

data area in a simple mbuf) and the size of the cmsghdr structure

is limited to MAX_ST_CTL bytes.  If the list of parameters to be

passed between the application program and ST exceeds these limits,

- 2 -

the parameters must be partitioned into two or more sublists and all

but the last sublist passed to the ST layer using the setsockopt()

function calls before issuing the desired system call, e.g.,

connect(), with the last sublist.  Note that control information

is never placed into a "data" buffer, as it is expected that the

data buffers may be directly connected to hardware which might be

confused by control data.

ST permits and sometimes requires more control interaction

with an application program than many other protocols.  In general,

this interaction is asynchronous with the data stream.  Consequently,

the application's state machine may be more complex and the

application must be prepared to receive and process the control

information throughout the lifetime of a socket.  When an

application does not require complex interactions with the ST

layer, the simpler interactions used by other network applications

should suffice.

ST does not address the issue of how application instances

become aware of each other's identity.  Within ST, the IP address

of an application's host (IPAdr), the number identifying the

protocol layer above ST (NextPcol), and the Service Access Point

(SAP, aka "port") are used to identify an application.

2.  Two-party Overview

In general, ST assumes multiple-party (more than two)

communications.  However, if an application knows a priori that a

stream will never have more than two participants and so informs

ST by setting the Point-to-Point ("STOptPBit") flag in the

st_options field of the sockaddr_st2 structure, ST can then make

simplifying assumptions during stream setup and management.

2.1.  Unidirectional Communication

Each ST stream passes data from a single origin to the

stream's target (only one for two-party communication).  The

target does:

T1)  socket()  to create a raw ST-II socket,

T2)  bind()    to specify the protocol, SAP length, and SAP,

T3)  listen()  to prepare for receipt of a STReqConnect, and

T4)  accept()  to create a new socket for each STReqConnect

received.

The accept() will block until a STReqConnect request is

received.

- 3 -

The origin does:

O1)  socket()  to create a raw ST-II socket,

O2)  bind()    to specify the protocol, SAP length, and SAP,

O3)  connect() to specify the required resources (TOS), identify

the target, and send a STReqConnect to it, and

O4)  recvmsg() to see if the connect() was successful.

The (O4) recvmsg() will block until either an error or an

accept or refuse reply is received.  Currently, the (O2)

bind() is not optional as the protocol must be specified.

[The (T1 and O1) socket() call(s) cannot be used to specify a

protocol as none are currently provided above ST-II in the

kernel.]  The bind() is also required if a SAP length other

than two bytes (i.e., sixteen bits) is desired or the local

IPAdr is to be specified.

When the SCMP CONNECT message created by the (O3)

connect() arrives at the target, the application is notified

of pending input: the application's (T4) accept() will

complete and return a new socket for exclusive use by the

stream.  The application then does a

T5)  recvmsg()

on the new socket to receive the STReqConnect request and the

associated flowspec (TOS) parameters.

The application then examines the parameters to decide

whether to accept or refuse the connection.  The decision is

passed to ST via an STReqAccept or STReqRefuse request using

T6)  sendmsg().

Assuming the target chooses to accept the connection, the

target then does a

T7)  recvmsg() to receive data (or control, e.g.,

STReqDisconnect).

The (T7) recvmsg() will block until the data or some control

message is received.  Otherwise, the socket for the refused

connection should be closed using the

T9)  close() function.

ST sends the STReqAccept or STReqRefuse decision back to

the application at the origin.  Assuming the connection was

accepted, the origin's (O4) recvmsg() will unblock and return

the actual flowspec (TOS) parameters that describe the

resources actually reserved in the networks for the stream.

- 4 -

The origin may begin to send data to the target using any of

the output functions, e.g.,

O5a) write(),

O5b) send(),

O5c) sendto(), or

O5d) sendmsg().

At any time, either the origin or target may issue a

STReqDisconnect request using (O5d or T8) sendmsg() to

gracefully tear down the stream and then (O6 or T9) close()

the socket.

2.2.  Bidirectional Communication

In the case of bidirectional data flow, two methods are

permitted: two (half-duplex) streams (each as described in

Section 2.1), or a single (full-duplex) stream.  When a single

full-duplex stream is used, both data streams will pass

through the same set of intervening ST Agents and networks.

When two half-duplex streams are used, the data flowing in one

stream may traverse a different set of intervening ST Agents

or networks than does data flowing in the reverse stream.

2.2.1.  Two (Half-duplex) Streams

This method of providing bidirectional communication

permits the data flowing in opposite directions to traverse

different ST Agents.  For example, different routes would be

selected if the resource requirements for the two directions

are significantly different.  This method also permits the

SAPs used by the two streams to be distinct.  Each participant

performs the roles of origin and target on different sockets,

as described under unidirectional communication in Section

2.1.  Note that if a single SAP is to be used for each

direction, then each socket must have the SO_REUSEADDR option

set (using the setsockopt() function (for "level" SOL_SOCKET).

2.2.2.  Single (Full-duplex) Stream

Since two-party bidirectional communication is such a

common occurrence, ST provides a way to create two streams

at once.  The sequence of system calls described in

Section 2.1 is used; the only difference is that the

parameters for the connect() function contain flowspec

(TOS) parameters for both directions.  In this case, the

"server" application performs the role of a target, as

described above:

S1)  socket()  to create a raw ST-II socket,

- 5 -

S2)  bind()    to specify the protocol, SAP length, and

SAP,

S3)  listen()  to prepare for receipt of a STReqConnect,

and

S4)  accept()  to create a new socket for each STReqConnect

received.

The accept() will block until a STReqConnect is received.

The "client" application does a

C1)  socket()  to create a raw ST-II socket,

C2)  bind()    to specify the protocol, SAP length, and

SAP,

C3)  connect() to specify the required resources (TOS)

for each direction (an STpFlowSpec and

STpRFlowSpec parameter), identify the

target, and send an SCMP CONNECT to it,

and

C4)  recvmsg() to see if the connect() was successful.

When the SCMP CONNECT message created by the (C3)

connect() arrives at the server, the application is

notified of pending input: the application's (S4) accept()

will complete and return a new socket for exclusive use by

the stream.  The server application then does a

S5)  recvmsg()

on the new socket to receive the STReqConnect request

which contains flowspecs for both the client-to-server and

server-to-client directions.  The server then accepts (or

refuses) the connection using

S6)  sendmsg()

with an STReqAccept, which is returned to the client.

Each participant then has a single socket which may be

used for bidirectional communication.

C5)  sendmsg()		S5)  recvmsg()

S6)  sendmsg()		C4)  recvmsg()

Either the client or server may issue a

STReqDisconnect by:

C5)  sendmsg()	or	S6)  sendmsg().

- 6 -

The other end will receive notification via the (C4 or S5)

recvmsg().  Both ends should then close() the socket:

C6)  close()	and	S7)  close().

3.  Multiple-party Overview

ST permits multiple-party (more than two) communications,

wherein each stream has a single origin that sources all data and

one or more targets which receive the data.  Recall however that

ST is not a reliable protocol; if reliability is required by an

application, it must be implemented by a protocol layer above ST.

Some implementations may support a multiple-party full-duplex

(star) variation which can be useful in certain applications.

In the unidirectional case, each of the targets follows the

procedures described above.  The origin simply includes

information about each target in the STpTargetList parameter in

the connect() call.  To support full intercommunication between N

participants, N streams are created.  Each participant performs

the roles of both a target (which will result in N-1 new sockets

being accept()ed), and that of an origin specifying the other N-1

participants as targets.

Each participant then does N-1 accept() calls to obtain a

socket per participant.  For each new socket, a recvmsg() is used

to receive the parameters associated with a STReqConnect and a

STReqAccept is sent to accept the connection using sendmsg().

When responses from the other N-1 participants have been received

on the socket used for the connect(), a participant may safely

begin sending data to the others.

3.1.  Full-Duplex Variation

It is envisioned that a full-duplex multiple-party star

form of communication might be useful.  One such scenario is

televised instruction, where audio and video from a central

classroom is sent to each remote classroom, and separate

return audio and video data is returned back to the central

site for audio and video mixing or switching.  However,

support for such a variation is implementation dependent.

- 7 -

4.  Example

All ST-II definitions required by an application program are

defined in the netinet/st2_api.h include file.  In particular, a set

of macros is defined which allow ST data structures and parameter

lists to be customized by an application.  Each macro has a name

of the form "Instxxx(args)" where xxx is the name of the structure

or parameter of interest.  The first argument is the symbolic

variable name of the struct being defined.  Other optional

arguments are used to specify the desired size of variable length

elements or embedded structures.  Other macros, e.g., Initxxxx,

may be used to initialize the Instxxx structures.  The macros are

only appropriate, however, for passing fixed-format information to

ST.  Variable format parameters must be constructed dynamically.

Information passed by ST to the application must be parsed by the

application since the application cannot predict which parameters

will be present or their relative order.

4.1.  socket

The socket() function is used to obtain an ST socket by

specifying protocol family PF_COIP.  Currently, only protocol type

SOCK_RAW is supported; other protocols usually implemented in the

kernel, e.g., UDP or TCP, have not yet been modified for use over

ST.  Consequently, the "prot" argument should be specified as 0.

#include <sys/types.h>

#include <sys/socket.h>

#include <netinet/st2_api.h>

int s;

s = socket( PF_COIP, SOCK_RAW, /*prot*/0 );

if ( s == -1 )

{ error processing; }

- 8 -

4.2.  bind

The bind() function may be used to specify the local host's IP

address "IPAdr" (if the host has more than one), the number

"NextPcol" (an STPROTO_xxx) identifying the application-layer

implementation of the protocol above ST (when "prot" was zero in

the socket call), the length of the SAP, and/or the SAP for the

local socket.  The sockaddr_st2 structure in the bind() call

should specify AF_COIP and contain an aApplEntity structure.

[Support for a sockaddr_in with AF_INET is not yet available.]

Use INADDR_ANY to leave unspecified the local host address, i.e.,

any of the host's network addresses.  Zero (0) to leave

unspecified the protocol above ST, i.e., if it was specified in

the socket call.  An application which desires to receive all SAPs

of a given length should specify the SAP length "SAPBytes" and set

all bytes in the SAP to binary zeros.  If an application is to

receive all SAPs of any length, the SAPBytes value should be zero.

The SAP matching criteria first checks for an exact match of

length and value, then checks for a match on length with a zero

value, and finally for a zero length.  The st_request field may be

either STReqBind or, since the context is clear, STReqUnspec.  The

opt_xxx field is used to specify the "OR" of the desired options:

STOptPBit for point-to-point (two-party) streams, STOptSBit for

the no recovery option, etc.

#define	SAPLEN 2		/* Length of our SAPs */

#define Bytesof(v) Bytes2 (v)	/* !@#$ cpp lacks an "eval" */

#define opt_xxx STOptSBit|STOptPBit	/* Desired options */

#define STPROTO_xxx 63

#define sap_xxx IPPORT_DISCARD

int error;

Instsockaddr_st2 (local,InstaApplEntity (here,SAPLEN,);) = {

Initsockaddr_st2 (STReqBind,opt_xxx),

InitaApplEntity (STLclAppEnt,local.here,

INADDR_ANY,STPROTO_xxx,SAPLEN,Bytesof (sap_xxx),)

};

error = bind( s, (struct sockaddr *) &local, sizeof (local) );

if ( error == -1 )

{ error processing; }

4.2.  listen

Targets should then do a listen() and an accept() which will

block until a STReqConnect request arrives.

error = listen( s, /*backlog*/ 5 );

if ( error == -1 )

{ error processing; }

- 9 -

4.4. connect

The origin should use the connect() function to create a

stream.  All parameters required for an SCMP CONNECT must be

specified before or at the time of the connect(), e.g., protocol

(via an STLclAppEnt in a bind()), FlowSpec (STpFlowSpec/struct

aFlowSpec3) and TargetList (STpTargetList/struct aTargetList), and

any of the optional parameters required by the application.

Parameters may be specified prior to the connect() using

setsockopt() function call(s).  In particular, use of the

setsockopt() function call(s) is required when the size of the

sockaddr_st2 structure containing all the necessary parameters

exceeds the maximum permitted (MAX_ST_NAM) for the name argument

of the connect().

Instsockaddr_st2 (remote,

InstaFlowSpec3 (tos);

InstaTargetList (targlst,

InstaTarget (targ1,SAPLEN,);

InstaTarget (targ2,SAPLEN,

InstaSrcRut (theway,1);

);

);

) = {

Initsockaddr_st2 (STReqConnect,opt_xxx),

InitaFlowSpec3 (STpFlowSpec,

/*min*/  128/*bytes*/,  2/*pps*/,128*2/*bandwidth*/,

/*des*/ 1024/*bytes*/, 10/*pps*/),

InitaTargetList (remote.targlst,2/*targets*/,

InitaTarget (remote.targlst.targ1,

INADDR_ANY,SAPLEN,Bytesof (sap_xxx),)

InitaTarget (remote.targlst.targ2,

INADDR_ANY,SAPLEN,Bytesof (sap_xxx),

InitaSrcRut (STpSTLSrcRut,

remote.targlst.targ2.theway,INADDR_ANY)

)

)

};

SetIPAdr (&remote.targlst.targ1,,targ1_ip_address);

SetIPAdr (&remote.targlst.targ2,,targ2_ip_address);

SetIPAdr (&remote.targlst.targ2.theway,[0],targ2_gateway_address);

error = connect( s, (struct sockaddr *) &remote, sizeof (remote) );

if ( error == -1 ) { error processing; }

Addition of targets to a stream after the initial connect are

effected using a sendmsg whose sockaddr_st2 st_request field

contains STReqConnect and whose st_parms list contains the

STpTargetList/struct aTargetList specifying the targets to be

added.  Note that routing optimizations which are possible when

the targets are specified prior to the (first) connect will not be

performed if the targets are specified one at a time using

multiple connects (sendmsg[setsockopt}/STReqConnect).

- 10 -

4.5.  accept

When a target receives an SCMP CONNECT message, a new socket

for the connection is obtained using the accept() function.

char namebuf[ MAX_ST_NAM ];

int namelen;

int s2;

struct sockaddr_st2 *namep;

namelen = sizeof (namebuf);

s2 = accept( s, (struct sockaddr *) &namebuf, &namelen );

if ( s2 == -1 ) { error processing; }

namep = (struct sockaddr_st2 *) namebuf;

if ( namep->st_request != STReqConnect ) { error processing; }

4.6.  recvmsg to obtain new connection parameters

After accept has returned a new socket for a connection, the

recvmsg() function is used to read the parameters for the

connection.

char ctrlbuf[ MAX_ST_CTL ];

struct msghdr msg;

struct cmsghdr *ctrlp;

InitMsg (&msg,namebuf,sizeof (namebuf),0,0,ctrlbuf,sizeof (ctrlbuf));

error = recvmsg( s2, &msg, 0/*flags*/ );

if ( error == -1 ) { error processing; }

ctrlp = (struct cmsghdr *) ctrlbuf;

if (   (ctrlp->cmsg_level != SOL_STII)

|| (ctrlp->cmsg_type != STReqConnect) )

{ error processing; }

4.7.  sendmsg to refuse a connection

The connection can be refused by calling sendmsg() with a

STReqRefuse request and an STReasonCode in the cmsghdr structure.

Instcmsghdr (refuse,InstaReasonCode (why);) = {

Initcmsghdr (refuse,STReqRefuse),

InitaReasonCode (ApplAbort)

};

namep->st_request = STReqRefuse;

msg.msg_accrights = &refuse; /* replace with refuse struct */

msg.msg_accrightslen = sizeof (refuse);

error = sendmsg( s2, &msg, 0/*flags*/ );

close( s2 );

- 11 -

4.8.  sendmsg to accept a connection

The connection can be accepted by a target using the sendmsg()

STReqAccept request.  A STpFlowSpec parameter is only necessary

when the target wishes to modify, e.g., reduce, the amount of

resources which were obtained along the path from the origin.  Or

when the application wnats to send a UserData Parameter.  For

example, if reverse charging (collect calls) is not to be

accepted, the target can clear the STFSRevChrg flag in the

received flowspec before accepting the connection.

struct aParameter *parmp;

struct aFlowSpec3 *fsp;

parmp = FindpCode(STpFlowSpec, ctrlp->cmsg_len,

(struct aParameter *) ctrlp->cmsg_data);

if ( parmp == (struct aParameter *) NULL )

{ inconsistency processing; }

fsp = (struct aFlowSpec3 *) parmp;

if ( fsp->FlowVer != STFSVer3 )

{ no support processing; }

fsp->Tradeoffs &= ~ STFSRevChrg; /* no reverse charging */

namep->st_request = STReqAccept; /* accept the connection */

/* just return all parameters EXCEPT UserData, let ST extract

the necessary parameters for the ACCEPT */

error = sendmsg( s2, &msg, 0/*flags*/ );

if ( error == -1 ) { error processing; }

4.9.  recvmsg at origin to receive control messages

The origin is notified of each target's acceptance or refusal

through a cmsghdr structure in a recvmsg().  The acceptance

(STReqAccept) includes the FlowSpec (STpFlowSpec) that the target

accepted; a refusal (STReqRefuse) includes the ReasonCode

(STReasonCode).  Either may contain a UserData Parameter.

InitMsg (&msg,namebuf,sizeof (namebuf),0,0,ctrlbuf,sizeof (ctrlbuf));

error = recvmsg( s, &msg, 0/*flags*/ );

if ( error == -1 ) { error processing; }

ctrlp = (struct cmsghdr *) ctrlbuf;

if ( ctrlp->cmsg_level != SOL_STII )

{ error processing; }

switch ( ctrlp->cmsg_type ) {

default:

case STReqRefuse:

close( s );

exit( 1 );

break;

case STReqAccept:

break;

}

- 12 -

4.10.  sending data

The origin may send data to the targets using any of the

"write" functions: write(), send(), sendto(), or sendmsg().

char data[] = "I'm alive!";

error = write( s, data, strlen( data ) );

if ( error != strlen( data ) )

{ error processing; }

error = send( s, data, strlen( data ), 0/*flags*/ );

if ( error != strlen( data ) )

{ error processing; }

namep->st_request = STReqUnSpec;	/* not control */

error = sendto( s, data, strlen( data ), 0/*flags*/,

(struct sockaddr *) namep, namelen );

if ( error != strlen( data ) )

{ error processing; }

#include <sys/uio.h>

struct iovec datavec;

datavec.iov_base = (caddr_t) data;

datavec.iov_len = strlen( data );	/* namep/namelen */

InitMsg (&msg,namep,namelen,&datavec,1,0,0);  /* optional */

namep->st_request = STReqUnSpec;	/* not control */

error = sendmsg( s, &msg, 0/*flags*/ );

if ( error != strlen( data ) )

{ error processing; }

The origin should also be prepared to receive notifications,

such as a disconnect request or error condition, while it is

sending data, see 4.9.  Use of select() is suggested.

- 13 -

4.11.  recvmsg at targets to receive data or control messages

The target enters a loop to receive data and control messages

using recvmsg().  The recvmsg() function must be used since the

cmsghdr structure in the "accrights" argument is used to pass

control information to the application;  read()/recv()/recvfrom()

must not be used.  Consequently, a return from recvmsg() with no

data may not imply EOF or that the connection has been closed; the

application must check the sockaddr_st2 st_request field to

determine what action is required.

char databuf[ MAX_DATAGRAM ];

int disconnected = FALSE;

struct iovec datavec;

while ( ! disconnected ) {

datavec.iov_base = (caddr_t) databuf;

datavec.iov_len = sizeof (databuf);

InitMsg (&msg, namebuf,sizeof (namebuf), &datavec,1,

ctlbuf,sizeof (ctlbuf));

error = recvmsg( s2, &msg, 0/*flags*/ );

if ( error == -1 )

{ error processing; break; }

namep = (struct sockaddr_st2 *) namebuf;

namelen = msg.msg_namelen;

if ( msg.msg_accrightslen > 0 ) {

ctlp = (struct cmsghdr *) ctlbuf;

if ( ctlp->cmsg_level != SOL_STII )

{ non ST-II protocol processing; }

parmp = (struct aParameter *) ctlp->cmsg_data;

/* ctlp->cmsg_type is identical to namep->st_request */

switch ( namep->st_request )

{   case STReqXxx:

process control;

break;

case STReqDisconnect:

process control;

disconnected = TRUE;

}

}

else if ( datavec.iov_len > 0 ) {

process data;

}

} /* end of while not disconnected */

- 14 -

4.12.  sendmsg at origin to disconnect targets

The origin can disconnect one or all targets from a connection

gracefully.  Individual targets are disconencted by supplying a

TargetList parameter containing the target(s) to be disconnected.

The sendmsg() function is used.

Instcmsghdr (disconn,InstaReasonCode (why);) = {

Initcmsghdr (disconn,STReqDisconnect),

InitaReasonCode (ApplDisconnect)

/* No target list means ALL targets */

};

namep->st_request = STReqDisconnect;

InitMsg (&msg,namebuf,namelen,0,0,&disconn,sizeof (disconn));

error = sendmsg( s /* s2 at target */, &msg, 0/*flags*/ );

if ( error == -1 )

{ error processing; }

4.13.  close

The close() function is used to abort (ApplAbort) an

application/ST interaction.

close( s2 );

close( s );

4.14.  select

The select() function may be used in the usual manner to

be signaled of the receipt of control or data messages instead

of blocking in accept() or recvmsg().  Non-blocking IO may

also be used when polling is desired.

- 15 -